M E M O R A N D U M

DATE:	May 2, 2013

TO:	Richard D. Garrity, Ph.D.

FROM:	Lora Webb		THRU:  Diana M. Lee, P.E.

Sterlin K. Woodard, P.E.

SUBJECT:	Draft Air Construction Permit - Quad/Graphics Marketing, LLC

Permit No. 0570254-026-AC

Attached is Draft Permit No. 0570254-026-AC to construct a sixth printing press line at Quad/Graphics Marketing, LLC (Quad) located at 4646 S. Grady Avenue, Tampa, FL.  Quad just recently purchased this facility from Vertis, Inc.  Quad is a lithographic printing facility primarily engaged in the printing of advertisements.

This permit authorizes the construction of one 4-unit Goss C700, 66-inch heatset web offset lithographic printing press designated as Press Line I.  The press is equipped with an 8.8 MMBtu/hr natural gas fired dryer.  The press can print on both sides of a continuous paper roll simultaneously in multiple colors up to 3,000 fpm and 90,000 impressions per hour.  Press Line I will use both manual and automatic blanket wash.

Permit No. 0570254-023-AC authorized the facility to install one 4-unit Harris N954, 54-inch heatset web offset lithographic printing press with an 8.0 MMBtu/hr natural gas fired dryer.  In addition, Permit No. 0570254-024-AC authorized the facility to install one 4-unit Goss C500, 46-inch heatset web offset lithographic printing press with a TEC dryer.  However, the facility decided not to install either press.  Quad will install the 4-unit Goss C700 press described above instead of the Harris and Goss C500 presses that were previously permitted.

In order to provide operational flexibility, the permit does not include a limit on the material usages.  Each type of material used at the facility has a different VOC content.  The facility may be able to use a larger amount of one material and still maintain compliance with the VOC Potential to Emit (PTE).  Therefore, in order to provide the facility maximum flexibility, this permit includes a limit on the overall facility VOC PTE, but does not include a limit on the material usage.  The facility VOC PTE is limited to 95.0 TPY in order to ensure that the facility does not exceed the VOC TV threshold.

Quad’s current operation consists of five existing printing press lines with a total of six natural gas fired dryers.  The VOC emissions from the five existing press lines and the new press line are controlled by a Langbein Engelbracht America, Model No. TR3090C, Regenerative Thermal Oxidizer (RTO).  The RTO has a minimum design destruction efficiency of 95%, which is the minimum destruction efficiency requirement being specified in this permit.  The facility is subject to the requirements of Rule 62-296.320, General Pollutant Emission Limiting Standards, F.A.C.

Based on our review, we recommend issuance of the permit as drafted.

LAW:  0570254-026-AC

TECHNICAL EVALUATION

AND

PRELIMINARY DETERMINATION

FOR

Quad/Graphics Marketing, LLC

Hillsborough County

Construction Permit

Application Number

0570254-026-AC

Environmental Protection Commission of

Hillsborough County

Tampa, FL

May 2, 2013

I.  Project Description

A.  Applicant:

Ron Garrison

Quad/Graphics Marketing, LLC

4646 S. Grady Ave.

Tampa, Fl 33611

B.  Engineer:

Jeffrey H. Twaddle, P.E.

P.E. No.:  51752

Environmental Resources Management

7106 Crossroads Blvd., Suite 228

Brentwood, TN 37024

- Project and Location:

This permit authorizes construction of a new printing line, Press Line I.  Quad/Graphics Marketing, LLC (Quad) will install a 4-unit Goss C700, 66-inch heatset web offset lithographic printing press with an 8.8 MMBtu/hr natural gas fired dryer.  The press can print on both sides of a continuous paper roll simultaneously in multiple colors up to 3,000 fpm and 90,000 impressions per hour.  Press Line I will use both manual and automatic blanket wash.

This project has been assigned NEDS Source Classification Codes 4-05-004-01 for Lithographic printing and 4-90-999-98 for miscellaneous VOC evaporation.  The Standard Industrial Code for this project is 27 for Printing, Publishing and Allied Industries.  The operation is located at 4646 S. Grady Avenue, Tampa, FL 33611. UTM Coordinates are 17-350.3 East and 3086.4 North.

- Process and Controls:

Quad/Graphics Marketing, LLC is a lithographic printing facility primarily engaged in the printing of advertisements.  Quad just recently purchased this facility from Vertis, Inc.  The operation consists of five existing printing lines and one new printing line described as follows.

Existing Press Lines A, D, E, G, and H:

Line A:		One 8-unit Harris N900, 46-inch heatset web offset printing press with two (2) MEG dryers.  The press can print both sides of a continuous paper roll simultaneously, in multiple colors up to 1,800 fpm.  The maximum heat input per dryer is 3.96 MMBTU/hr.

Line D:	One 4 unit Harris N954, 54-inch heatset web offset printing press with a C-2400

dryer having a maximum heat input of 6.160 MMBTU/hr. The press can print

both sides of a continuous paper roll simultaneously in multiple colors up to

1,800 fpm.

Line E:	One 4 unit Harris N956, 56-inch heatset web offset printing press with a TEC

dryer having a maximum heat input of 7.7 MMBTU/hr.  The press can print

both sides of a continuous paper roll simultaneously in multiple colors up to

2,000 fpm.

Line G:	One 4-unit Goss C700, 66-inch heatset web offset printing press with a MEGTEC dryer having a maximum heat input of 8.0 MMBtu/hr.  The press can print both sides of a continuous paper roll simultaneously in multiple colors up to 3,000 fpm.  An auto blanket wash system is installed on Line G.

Line H:		One 4-unit Goss C700, 66-inch heatset web offset printing press with a dryer having a maximum heat input of 8.0 MMBTU/hr.  The press can print both sides of a continuous paper roll simultaneously in multiple colors up to 3,000 fpm.   An auto blanket wash system is installed on Line H.

New Press Line I:

Line I:		One 4-unit Goss C700, 66-inch heatset web offset lithographic printing press with a dryer having a maximum heat input of 8.8 MMBtu/hr.  The press can print on both sides of a continuous paper roll simultaneously in multiple colors up to 3,000 fpm and 90,000 impressions per hour.  Press Line I will use both manual and automatic blanket wash.

Permit No. 0570254-023-AC authorized the facility to install one 4-unit Harris N954, 54-inch heatset web offset lithographic printing press with an 8.0 MMBtu/hr natural gas fired dryer.  In addition, Permit No. 0570254-024-AC authorized the facility to install one 4-unit Goss C500, 46-inch heatset web offset lithographic printing press with a TEC dryer.  However, the facility decided not to install either press.  Quad will install the 4-unit Goss C700 press described above instead of the Harris and Goss C500 presses that were previously permitted.

The volatile organic compound (VOC) emissions from the six press lines with a total of seven dryers including the auto blanket wash systems are controlled by a Langbein Engelbracht America, Model No. TR3090C, Regenerative Thermal Oxidizer (RTO).  The VOC laden air from the dryers is vented to the RTO through the entire length of the oxidizer system.  The dryers operate under negative pressure in order to capture the VOC emissions from the printing operations.  The RTO operates at a minimum temperature of 1,400 oF under normal operating conditions and is fired on natural gas at a maximum heat input rate of 5 MMBtu/hr. The RTO afterburner has a minimum design destruction efficiency of 95 percent at a maximum inflow rate of 30,000 scfm and 500 pounds of VOC per hour.   The five existing press lines and the new press line have flow rates as follows:

Line A	 3,824 scfm

Line D	 4,000 scfm

Line E	 4,200 scfm

Line G	 4,500 scfm

Line H	 4,500 scfm

Line I   4,500 scfm

25,524 scfm < 30,000 scfm

The operation also emits fugitive VOC emissions from the manual blanket wash and miscellaneous coatings/additives.  These VOC emissions dissipate uncontrolled, indoors and have the potential to escape outdoors through building openings and vents.  Some of the additives may be introduced into the ink or fountain solution, which results in the capture of additive VOC emissions in the dryers.

Because of the high boiling point and low vapor pressure of solvents in the heatset inks and based on EPA guidance and tests performed on similar units, VOC emissions from the heatset inks are considered to be 100% captured in the respective dryers.  Possible solvent retained in the substrate (printed material after passing through the dryers) from the heatset web is not expected to release significant VOC levels because the printed paper quickly cools down as it leaves the dryer.   Based on the stack test conducted on November 3, 2005, it showed that the percentage of ink retained by the paper is 0.1% and this value was used in the calculation of the VOC PTE.

In order to provide operational flexibility, the permit does not include a limit on the material usages.  Each type of material used at the facility has a different VOC content.  The facility may be able to use a larger amount of one material and still maintain compliance with the VOC PTE.  Therefore, in order to provide the facility maximum flexibility, this permit includes a limit on the VOC PTE but does not include a limit on the material usage.  In addition, in order to provide reasonable assurance that the PTE will not be exceeded, the facility is required to maintain a monthly tracking system, which shall include records detailing the material usages, the VOC content of the materials, and the emission calculations.  However, if the actual VOC emissions reach 80.0 tons or greater in any twelve consecutive month period, the facility is required to maintain daily records until the rolling twelve month totals are less than 80.0 tons.  The facility VOC PTE is limited to 95.0 TPY in order to ensure that the facility does not exceed the VOC TV threshold.  This permit includes a requirement to perform a Destruction Efficiency test and a Capture Efficiency test upon completion of the installation of the new press.

- Application Information:

Received on:  April 12, 2013

Information Requested:  NA

Application Complete:  April 12, 2013
II.  Rule Applicability

This project is subject to the preconstruction review requirements of Chapter 403, Florida Statutes, Chapters, 62-204, 62-210, 62-212, 62-296, and 62-297, Florida Administrative Code (F.A.C.) and Chapter 1-3 of the Rules of the Environmental Protection Commission of Hillsborough County.

This project is subject to the requirements of Rule 62-212.300, General Preconstruction Review Requirements, F.A.C., since the project is not exempt from the permit requirements in Rule 62-210.300, F.A.C.

This project is not subject to the requirements of Rule 62-212.400, Prevention of Significant Deterioration, F.A.C. or Rule 62-212.500, New Source Review for Nonattainment Areas, F.A.C. since the facility is a Synthetic Non-TV facility by state definition.

This project is not subject to the requirements of Rule 62-213, Operation Permits for Major Sources of Air Pollution, F.A.C., since the facility is a Synthetic Non-Title V facility by state definition.

This project is subject to the requirements of Rule 62-296.320, General Pollutant Emission Limiting Standards, F.A.C., since the facility is a source of volatile organic compounds and a potential source of odors.

This project is not subject to the requirements of Rule 62-296.401, Incinerators, F.A.C because the RTO does not meet the definition of an Incinerator as defined in Rule 62-210.200(165) – Incinerator.

This project is not subject to the requirements of Rule 62-296.500, Volatile Organic Compounds and Nitrogen Oxides Reasonably Available Control Technology, F.A.C., since there is no applicable source specific category in this rule.

This project is not subject to the requirements of Rule 62-296.600, Reasonably Available Control Technology - Lead, F.A.C., since there is no applicable source specific category in this rule.

This project is not subject to the requirements of Rule 62-296.700, Reasonably Available Control Technology - Particulate Matter, F.A.C., since there is not an applicable source category.

This project is not subject to the requirements of Rule 62-204.800, Federal Regulations Adopted by Reference, F.A.C., since there is not an applicable source specific category in this rule.

This project is subject to the requirements of Chapter 84-446, Laws of Florida and Chapter 1-3, Rules of the Environmental Protection Commission of Hillsborough County.

III. Summary of Emissions

| EU No. | Actual VOC (TPY) | VOC PTE (TPY) | Increase (TPY) |
| --- | --- | --- | --- |
| EU 002 – Solvent | 6.3 | 11.8 | 5.5 |
| EU 006 – Press Lines with RTO | 23.2 | 62.9 | 39.7 |
| EU 008 – Misc. Coatings/Additives | 1.2 | 20.3 | 19.1 |
| Facility-wide Total | 30.7 | 95.0 | 64.3 |

- Actual emissions for EU Nos. 002 and 006 are based on the average of 2010 and 2011 AOR data.

- Actual emissions for EU No. 008 are based on 2011 AOR data because this emission unit began operation in 2011.

- Potential VOC emissions from EU No. 002 are based on the VOC content of the material and a 50% retention factor in the shop towels.  The remaining 50% is emitted uncontrolled to the atmosphere.  This is based on EPA-453/R-06-002 - Control Techniques Guidelines for Offset Lithographic Printing and Letterpress Printing.

- Potential VOC emissions from EU No. 006 are based on the VOC content of the material, the following retention or capture factor, and an RTO destruction efficiency of 95%.  The emissions also include VOC emissions from the combustion of natural gas in the dryers and the RTO.

  - Based on the stack test conducted on November 3, 2005, the percentage of ink retained by the paper when using heatset inks is 0.1%.

  - Based on EPA-453/R-06-002 - Control Techniques Guidelines for Offset Lithographic Printing and Letterpress Printing, 70% of the fountain solution is captured and directed to the RTO.  The remaining 30% is emitted uncontrolled.

  - Based on EPA-453/R-06-002 - Control Techniques Guidelines for Offset Lithographic Printing and Letterpress Printing, 40% of the automatic blanket wash is captured and directed to the RTO.  The remaining 60% is emitted uncontrolled.

- Potential VOC emissions from EU No. 008 are based on the VOC content of the material.  Emissions from EU No. 008 are vented uncontrolled to the atmosphere.

- The VOC emissions from each emission unit may vary provided that the facility wide VOC emissions do not exceed 95.0 tons/year.

Inventory of Title III pollutants is estimated to be less than 10 TPY individually and less than 25 TPY collectively.

IV. Conclusions:

The emission limits proposed by the applicant will meet all of the requirements of Chapters 62-204, 62-209, 62-210, 62-212, 62-272, 62-275, 62-296, and 62-297, F.A.C., and Chapter 1-3, Rules of the Commission.

The General and Specific Conditions listed in the proposed permit (attached) will assure compliance with all the applicable requirements of Chapters 62-204, 62-209, 62-210, 62-212, 62-272, 62-275, 62-296, and 62-297, F.A.C.

V.   Proposed Agency Action:

Pursuant to Section 403.087, Florida Statutes and Rule 62-4.070, Florida Administrative Code the Environmental Protection Commission of Hillsborough County hereby gives notice of its intent to issue a permit to construct the aforementioned air pollution source in accordance with the draft permit and its conditions as stipulated (see attached).

CERTIFIED MAIL

In the Matter of an                          				File No.:   0570254-026-AC

Application for Permit by:                   				County:    Hillsborough

Ron Garrison

Quad/Graphics Marketing, LLC

4646 S. Grady Ave.

Tampa, FL 33611

__________________________________/

INTENT TO ISSUE

The Environmental Protection Commission of Hillsborough County (EPC), as delegated by the Florida Department of Environmental Protection (DEP) gives notice of its intent to issue a permit (copy attached) for the proposed project as detailed in the application specified above, for the reasons stated below.

The applicant, Quad/Graphics Marketing, LLC (Quad), a lithographic printing facility, applied on April 12, 2013 to the EPC for a construction permit to construct a new press line, Press Line I.  The emissions from the six press lines are controlled by a Regenerative Thermal Oxidizer (RTO).  Quad, located at 4646 S. Grady Avenue, Tampa, FL 33611, will remain a synthetic minor source of volatile organic compound (VOC) emissions and hazardous air pollutants (HAPs).

The EPC has permitting jurisdiction under Chapter 403 Florida Statutes (F.S.) and Florida Administrative Code (F.A.C.) Chapters 62-4 and 62-210.  The project is not exempt from permitting procedures.  The EPC has determined that an air pollution construction permit is required for the proposed work.

The EPC intends to issue this permit based on the belief that reasonable assurances have been provided to indicate the proposed project will not adversely impact air quality and the proposed project will comply with the appropriate provisions of Florida Administrative Code (F.A.C.) Rules 62-204, 62-210, 62-212, 62-296, 62-297, and 62-4.

Pursuant to Section 403.815 and 403.0872, F.S. and Rules 62-103.150 and 62-210.350(3), F.A.C., you (the applicant) are required to publish at your own expense the enclosed Notice of Intent to Issue Permit.  The notice shall be published one time as soon as possible, in the legal advertisement section of a newspaper of general circulation in the area affected.  For the purpose of this rule,

"publication in a newspaper of general circulation in the area affected" means publication in a newspaper meeting the requirements of Sections 50.011 and 50.031, F.S., in the county where the activity is to take place.  Where there is more than one newspaper of general circulation in the county, the newspaper used must be one with significant circulation in the area that may be affected by the permit.  If you are uncertain that a newspaper meets these requirements, please contact the EPC at the address or telephone number listed below.  The applicant shall provide proof of publication to the EPC, Air Permitting Section, at 3629 Queen Palm Drive, Tampa, Florida 33619 (Phone 813-627-2600,  FAX 813-627-2660) within 7 (seven) days of publication.  Failure to publish the notice and provide proof of publication within the allotted time may result in the denial of the permit pursuant to Rule 62-103.150(6), F.A.C.

The EPC will issue the final permit with the attached conditions unless a timely petition for an administrative hearing is filed pursuant to Section 120.569 and 120.57 F.S. before the deadline for filing a petition.  The procedures for petitioning for a hearing are set forth below.

A person whose substantial interests are affected by the proposed permitting decision may petition for an administrative proceeding (hearing) under Sections 120.569 and 120.57, F.S.  The petition must contain the information set forth below and must be filed (received) in the Legal Department of the EPC at 3629 Queen Palm Drive, Tampa, Florida 33619, Phone 813-627-2600, Fax 813-627-2602.  Petitions filed by the permit applicant or any of the parties listed below must be filed within 14 (fourteen) days of receipt of this notice of intent.  Petitions filed by any persons other than those entitled to written notice under Section 120.60(3), F.S. must be filed within 14 (fourteen) days of publication of the public notice or within 14 (fourteen) days of receipt of this notice of intent, whichever occurs first.  Under Section 120.60(3), however, any person who asked the EPC for notice of agency action may file a petition within 14 (fourteen) days of receipt of that notice, regardless of the date of publication.  A petitioner shall mail a copy of the petition to the applicant at the address indicated above, at the time of filing.  The failure of any person to file a petition within the appropriate time period shall constitute a waiver of that person's right to request an administrative determination (hearing) under Sections 120.569 and 120.57, F.S.; or to intervene in this proceeding and participate as a party to it.  Any subsequent intervention will be only at the approval of the presiding officer upon the filing of a motion in compliance with Rule 28-106.205, F.A.C.

A petition that disputes the material facts on which the EPC’s action is based must contain the following information:

- The name and address of each agency affected and each agency’s file or identification number if known;

- The name, address, and telephone number of the petitioner and the name, address, and telephone number of each petitioner’s representative, if any, which shall be the address for service purposes during the course of the proceedings; and an explanation of how the petitioner’s substantial interests will be affected by the EPC’s determination;

- A statement of how and when the petitioner received notice of the EPC action;

- A statement of all disputed issues of material fact. If there are none, the petition must so indicate;

- A concise statement of the ultimate facts alleged, including the specific facts the petitioner contends warrant reversal or modification of the EPC’s proposed action;

- A statement of specific rules or statutes the petitioner contends requires reversal or modification of the EPC’s proposed action; and

- A statement of the relief sought by the petitioner, stating precisely the action petitioner wishes the EPC to take with respect to the EPC’s proposed action.

A petition that does not dispute the material facts upon which the EPC’s action is based shall state that no such facts are in dispute and otherwise shall contain the same information as set forth above as required by Rule 28-106.301, F.A.C.

Because the administrative hearing process is designed to formulate final agency action, the filing of a petition means that the EPC's final action may be different from the position taken by it in this notice of intent. Persons whose substantial interests will be affected by any such final decision of the EPC on the application have the right to petition to become a party to the proceeding, in accordance with the requirements set forth above.

Mediation under section 120.573, F.S. is not available in this proceeding.

This action is final and effective on the date filed with the Clerk of the EPC unless a petition is filed in accordance with above.  Upon the timely filing of a petition, this order will not be effective until further order of the EPC.

In addition to the above, a person subject to regulation has a right to apply to the Department of Environmental Protection for a variance from or waiver of the requirements of particular rules, on certain conditions, under Section 120.542, F.S.  The relief provided by this state statute applies only to state rules, not statutes, and not to any federal regulatory requirements.  Applying for a variance or waiver does not substitute or extend the time for filing a petition for an administrative hearing or exercising any other right that a person may have in relation to the action proposed in this notice of intent.

The application for a variance or waiver is made by filing a petition with the Office of General Counsel of the Department of Environmental Protection, 3900 Commonwealth Boulevard, Mail Station #35, Tallahassee, FL  32399-3000.  The petition must specify the following information:

- The name, address, and telephone number of the petitioner,

- The name, address, and telephone number of the attorney or qualified representative of the petitioner, if any,

- Each rule or portion of a rule from which a variance or waiver is requested,

- The citation to the statute underlying (implemented by) the rule identified in (c) above,

- The type of action requested,

- The specific facts that would justify a variance or waiver for the petitioner,

- The reason why the variance or waiver would serve the purposes of the underlying statute (implemented by the rule), and

- A statement whether the variance or waiver is permanent or temporary and, if temporary, a statement of the dates showing the duration of the variance or waiver requested.

The Department will grant a variance or waiver when the petition demonstrates both that the application of the rule would create a substantial hardship or violate principles of fairness, as each of the those terms is defined in Section 120.542(2), F.S., and that the purpose of the underlying statute will be or has been achieved by other means by the petitioner.

Persons subject to regulation pursuant to any federally delegated or approved air program should be aware that Florida is specifically not authorized to issue variances or waivers from any requirements of any such federally delegated or approved program.  The requirements of the program remain fully enforceable by the Administrator of United States Environmental Protection Agency and by any person under the Clean Air Act unless and until the Administrator separately approves any variance or waiver in accordance with the procedures of the federal program.

Any person listed below may request to obtain additional information, a copy of the application (except for information entitled to confidential treatment pursuant to Section 403.111, F.S.), all relevant supporting materials, and all other materials available to the EPC that are relevant to the permit decision.  Interested persons may contact Diana M. Lee, P.E., at the above address or call (813) 627-2600, for additional information.

Any party to this order has the right to seek judicial review of it under Section 120.68 of the Florida Statues, by filing a notice of appeal under rule 9.110 of the Florida rules of Appellate Procedure with the EPC’s Legal Office at 3629 Queen Palm Drive, Tampa, Florida 33619 and with the clerk of the Department of Environmental Protection in the Office of General Counsel, Mail Station 35, 3900 Commonwealth Boulevard, Tallahassee, Florida 32399-3000, and by filing a copy of the notice of appeal accompanied by the applicable filing fees with the appropriate district court of appeal.  The notice must be filed within thirty days after this order is filed with the clerk of the Department.

Executed in Tampa, Florida

ENVIRONMENTAL PROTECTION COMMISSION

OF HILLSBOROUGH COUNTY

__________________________________

Richard D. Garrity, Ph.D.

Executive Director

cc:	 Jeffrey H. Twaddle, P.E. – ERM (via email)

Glenn Shaffer – Vertis, Inc. (via email)

Florida Department of Environmental Protection, Southwest District (via e-mail)

CERTIFICATE OF SERVICE

The undersigned duly designated clerk hereby certifies that this INTENT TO ISSUE and all copies were mailed before the close of business on _________________ to the listed persons.

FILING AND ACKNOWLEDGEMENT

FILED, on this date, pursuant to Section 120.52(7), Florida Statutes, with the designated clerk, receipt of which is hereby acknowledged.

_____________________   _____________

Clerk                                      Date

ENVIRONMENTAL PROTECTION COMMISSION

OF HILLSBOROUGH COUNTY

NOTICE OF INTENT TO ISSUE PERMIT

The Environmental Protection Commission of Hillsborough County (EPC), as delegated by the Florida Department of Environmental Protection (DEP) gives notice of its intent to issue air pollution Permit No. 0570254-026-AC to Quad/Graphics Marketing, LLC, a lithographic printing facility, for a construction permit to construct a new press line, a 4-unit Goss C700, 66-inch heatset web offset lithographic printing press with a dryer.  The emissions from all the press lines are controlled by a Regenerative Thermal Oxidizer.  The facility, located at 4646 S. Grady Avenue, Tampa, FL 33611, will remain a synthetic minor source of volatile organic compound emissions and hazardous air pollutants.

A Best Available Control Technology (BACT) determination was not required.

The EPC will issue the Final permit with the conditions of the DRAFT permit unless a timely petition for an administrative hearing is filed pursuant to Sections 120.569 and 120.57, F.S. before the deadline for filing a petition.  The procedures for petitioning for hearing are set forth below.

A person whose substantial interests are affected by the proposed permitting decision may petition for an administrative proceeding (hearing) under Sections 120.569 and 120.57, F.S.  The petition must contain the information set forth below and must be filed (received) in the Legal Department of the EPC at 3629 Queen Palm Drive, Tampa, Florida 33619, Phone 813-627-2600, Fax 813-627-2602.  Petitions filed by the permit applicant or any of the parties listed below must be filed within 14 (fourteen) days of receipt of this notice of intent. Petitions filed by any persons other than those entitled to written notice under Section 120.60(3), F.S. must be filed within 14 (fourteen) days of publication of the public notice or within 14 (fourteen) days of receipt of this notice of intent, whichever occurs first.  Under Section 120.60(3), however, any person who asked the EPC for notice of agency action may file a petition within 14 (fourteen) days of receipt of that notice, regardless of the date of publication.  A petitioner shall mail a copy of the petition to the applicant at the address indicated above, at the time of filing.  The failure of any person to file a petition within the appropriate time period shall constitute a waiver of that person's right to request an administrative determination (hearing) under Sections 120.569 and 120.57, F.S., or to intervene in this proceeding and participate as a party to it.  Any subsequent intervention will be only at the approval of the presiding officer upon the filing of a motion in compliance with Rule 28-106.205 of the F.A.C.

A petition that disputes the material facts on which the EPC’s action is based must contain the following information:

- The name and address of each agency affected and each agency’s file or identification number if known;

- The name, address, and telephone number of the petitioner, and the name, address, and telephone number of each petitioner’s representative, if any, which shall be the address for service purposes during the course of the proceedings; and an explanation of how the petitioner’s substantial interests will be affected by the agency determination;

- A statement of how and when petitioner received notice of the EPC action;

- A statement of all disputed issues of material fact.  If there are none, the petition must so indicate;

- A concise statement of the ultimate facts alleged, including the specific facts the petitioner contends warrant reversal or modification of the EPC proposed action;

- A statement of specific rules or statutes the petitioner contends requires reversal or modification of the EPC’s proposed action; and

- A statement of the relief sought by the petitioner, stating precisely the action petitioner wishes the EPC to take with respect to the EPC’s proposed action.

A petition that does not dispute the material facts upon which the EPC’s action is based shall state that no such facts are in dispute and otherwise shall contain the same information as set forth above as required by Rule 28-106.301.

Because the administrative hearing process is designed to formulate final agency action, the filing of a petition means that the EPC's final action may be different from the position taken by it in this notice of intent.  Persons whose substantial interests will be affected by any such final decision of the EPC on the application have the right to petition to become a party to the proceeding, in accordance with the requirements set forth above.

Mediation under section 120.573, F.S. is not available in this proceeding.

This action is final and effective on the date filed with the Clerk of the EPC unless a petition is filed in accordance with above.  Upon the timely filing of a petition this order will not be effective until further order of the EPC.

Any party to this order has the right to seek judicial review of it under Section 120.68 of the Florida Statues, by filing a notice of appeal under rule 9.110 of the Florida rules of Appellate Procedure with the EPC’s Legal Office at 3629 Queen Palm Drive, Tampa, Florida 33619 and with the clerk of the Department of Environmental Protection in the Office of General Counsel, Mail Station 35, 3900 Commonwealth Boulevard, Tallahassee, Florida 32399-3000, and by filing a copy of the notice of appeal accompanied by the applicable filing fees with the appropriate district court of appeal.  The notice must be filed within thirty days after this order is filed with the clerk of the Department.

The complete project file is available for public inspection during normal business hours, 8:00 a.m. to 5:00 p.m., Monday through Friday, except legal holidays, at the Environmental Protection Commission of Hillsborough County, 3629 Queen Palm Drive, Tampa, Florida 33619.  The complete project file includes the proposed Permit, the application, and the information submitted by the responsible official, exclusive of confidential records under Section 403.111, F.S.  Interested persons may contact Diana M. Lee, P.E., at the above address, or call 813-627-2600, for additional information.  Any written comments filed shall be available for public inspection.  If written comments received result in a significant change in the proposed agency action, the EPC shall revise the proposed permit and require, if applicable, another Public Notice.

ENVIRONMENTAL PROTECTION COMMISSION OF

HILLSBOROUGH COUNTY, as Delegated by

STATE OF FLORIDA

DEPARTMENT OF ENVIRONMENTAL PROTECTION

NOTICE OF PERMIT

Ron Garrison

Quad/Graphics Marketing, LLC

4646 S. Grady Ave.

Tampa, FL 33611

Dear Mr. Garrison:

Enclosed is Permit Number 0570254-026-AC for a construction permit to construct a 4-unit Goss C700, 66-inch heatset web offset lithographic printing press with a dryer.  This permit is issued pursuant to Section 403.087, Florida Statutes.

Any party to this order (permit) has the right to seek judicial review of the permit pursuant to Section 120.68, Florida Statutes, by the filing of a Notice of Appeal pursuant to Rule 9.110, Florida Rules of Appellate Procedure, with the Clerk of the EPC in the Legal Department at 3629 Queen Palm Drive, Tampa, FL 33619; and by filing a copy of the Notice of Appeal accompanied by the applicable filing fees with the appropriate District Court of Appeal.  The Notice of Appeal must be filed within 30 days from the date this Notice is filed with the clerk of the EPC.

Executed in Tampa, Florida.

Sincerely,

Richard D. Garrity, Ph.D.

Executive Director

RDG/LAW/law

Quad/Graphics Marketing, LLC									Page 2

Tampa, FL

cc:	 Jeffrey H. Twaddle, P.E. – ERM (via email)

Glenn Shaffer – Vertis, Inc. (via email)

Florida Department of Environmental Protection, Southwest District (via e-mail)

CERTIFICATE OF SERVICE

This is to certify that this NOTICE OF PERMIT and all copies were mailed before the close of business on _________________________ to the listed persons.

Clerk Stamp

FILED, on this date, pursuant to Section 120.52(7), Florida Statutes, with the designated clerk, receipt of which is hereby acknowledged.

__________________________   _____________

clerk                     	 	      Date

PERMITTEE:						PERMIT/CERTIFICATION

Quad/Graphics Marketing, LLC			Permit No.:  0570254-026-AC

4646 S. Grady Avenue	County:  Hillsborough

Tampa, FL 33611	Expiration Date:  August 30, 2014

Project:  Addition of Press Line I

This permit is issued under the provisions of Chapter 403, Florida Statutes, and Florida Administrative Code Rules 62-204, 62-210, 62-212, 62-296, 62-297, and 62-4.  The above named permittee is hereby authorized to perform the work or operate the facility shown on the application and approved drawing(s), plans, and other documents, attached hereto or on file with the EPC and made a part hereof and specifically described as follows:

This permit authorizes the construction of a new press line, Press Line I, at a lithographic printing facility.  The operation consists of five existing printing lines and one new press line described as follows:

Existing Press Lines A, D, E, F, G, and H:

Line A:		One 8-unit Harris N900, 46-inch heatset web offset printing press with two (2) MEG dryers.  The press can print both sides of a continuous paper roll simultaneously, in multiple colors up to 1,800 fpm.  The maximum heat input per dryer is 3.96 MMBTU/hr.

Line D:	One 4 unit Harris N954, 54-inch heatset web offset printing press with a C-2400

dryer having a maximum heat input of 6.160 MMBTU/hr. The press can print

both sides of a continuous paper roll simultaneously in multiple colors up to

1,800 fpm.

Line E:	One 4 unit Harris N956, 56-inch heatset web offset printing press with a TEC

dryer having a maximum heat input of 7.7 MMBTU/hr.  The press can print

both sides of a continuous paper roll simultaneously in multiple colors up to

2,000 fpm.

Line G:	One 4-unit Goss C700, 66-inch heatset web offset printing press with a MEGTEC dryer having a maximum heat input of 8.0 MMBtu/hr.  The press can print both sides of a continuous paper roll simultaneously in multiple colors up

to 3,000 fpm.  An auto blanket wash system is installed on Line G.

Line H:		One 4-unit Goss C700, 66-inch heatset web offset printing press with a dryer having a maximum heat input of 8.0 MMBTU/hr.  The press can print both sides of a continuous paper roll simultaneously in multiple colors up to 3,000 fpm.   An auto blanket wash system is installed on Line H.

New Press Line I:

Line I:		One 4-unit Goss C700, 66-inch heatset web offset lithographic printing press with a dryer having a maximum heat input of 8.8 MMBtu/hr.  The press can print on both sides of a continuous paper roll simultaneously in multiple colors up to 3,000 fpm and 90,000 impressions per hour.  Press Line I will use both manual and automatic blanket wash.

The volatile organic compound (VOC) emissions from the six press lines with a total of seven dryers including the auto blanket wash systems are controlled by a Langbein Engelbracht America, Model No. TR3090C, Regenerative Thermal Oxidizer (RTO), which has a maximum air inflow rate of 30,000 scfm.  The VOC laden air from the dryers is vented to the RTO through the entire length of the oxidizer system.  The dryers operate under negative pressure in order to capture the VOC emissions from the printing operations.  The RTO operates at a minimum temperature of 1,400 oF under normal operating conditions and is fired on natural gas at a maximum heat input rate of 5 MMBtu/hr.  The RTO afterburner has a minimum design destruction efficiency of 95 percent at a maximum inflow rate of 30,000 scfm and 500 pounds of VOC per hour.  The five existing press lines and the new press line have flow rates as follows:

Line A	 3,824 scfm

Line D	 4,000 scfm

Line E	 4,200 scfm

Line G	 4,500 scfm

Line H	 4,500 scfm

Line I   4,500 scfm

25,524 scfm < 30,000 scfm

Because of the high boiling point and low vapor pressure of solvents in the heatset inks and based on EPA guidance and tests performed on similar units, VOC emissions from the heatset inks are considered to be 100% captured in the respective dryers.  Possible solvent retained in the substrate (printed material after passing through the dryers) from the heatset web is not expected to release significant VOC levels because the printed paper quickly cools down as it leaves the dryer.

The printing operation also emits fugitive VOC emissions from the manual blanket wash and miscellaneous coatings/additives usage.  These VOC emissions dissipate uncontrolled, indoors and have the potential to escape outdoors through building openings and vents.  Some of the additives may be introduced into the ink or fountain solution, which results in the capture of additive VOC emissions in the dryers.

Location:  4646 S. Grady Avenue, Tampa, FL

UTM:  17-350.3 E 3086.4 N

NEDS NO:  0570254

Emission unit Nos.:  002 – Solvent (manual blanket wash usage)

006 – Heatset for Printing Press Lines A, D, E, G, H, and I with RTO

008 – Miscellaneous Coatings/Additives

References Permit No.:  NA

1.	A part of this permit is the attached General Conditions.  [Rule 62-4.160, F.A.C.]

2.	All applicable rules of the Environmental Protection Commission of Hillsborough County including design discharge limitations specified in the application shall be adhered to.  The permit holder may also need to comply with county, municipal, federal, or other state regulations prior to construction. [Rule 62-4.070(7), F.A.C.]

3.	Issuance of this permit does not relieve the permittee from complying with applicable emission limiting standards or other requirements of Chapters 62-204, 62-210, 62-212, 62-296, and 62-297, F.A.C., or any other requirements under federal, state, or local law.  [Rule 62-210.300, F.A.C.]

4.	As requested by the permittee, in order to limit the potential to emit and establish the facility as a synthetic minor source for both criteria and hazardous air pollutants (HAPs), the following emission limitations shall apply for any 12 consecutive month period: [Rules 62-210.200(247), 62-212.300, and 62-4.070(3), F.A.C.]

- The maximum facility wide VOC emissions shall not exceed 95.0 tons.

- The HAP, as defined in Rule 62-210.200, F.A.C., emissions shall be less than 10 tons for any individual HAP, and less than 25 tons for any combination of HAPs.

5. 	Visible emissions from any emission unit or activity, including the exhaust from the oxidizer controlling the press lines, shall not be equal to or greater than 20% opacity.  [Rule 62-296.320(4)(b)1., F.A.C.]

6.	The permittee shall not cause, suffer, allow or permit the discharge of air pollutants which cause or contribute to an objectionable odor.  [Rule 62-296.320(2), F.A.C.]

7.	In order to demonstrate compliance with the emission limitation in Specific Condition No. 4, the following restrictions and limitations shall apply: [Rule 62-4.070(3), F.A.C.]

- The RTO shall maintain a minimum destruction efficiency of 95%.

- The RTO afterburner temperature shall be maintained at a minimum of 1,400 oF under normal operating conditions.

- The maximum VOC inlet loading to the RTO shall not exceed 500 pounds per hour.

- All VOC emissions from the heatset inks shall be 100% captured in their respective dryers and vented to the RTO afterburner.

- If any thinning of ink(s) is necessary, the VOC and HAP emissions generated from such thinning shall be counted towards the emission limits specified in Specific Condition No. 4.

- All spent rags used in the manual blanket wash operations shall be stored in closed containers.

8.	When calculating emissions to demonstrate compliance with the emission limitation specified in Specific Condition No. 4, the following shall apply:  [Rule 62-4.070(3), F.A.C.]

- When calculating emissions from the manual blanket wash usage, EU No. 002, it shall be assumed that 50% of the VOC emissions are retained in the substrate and 50% of the VOC emissions are vented uncontrolled to the atmosphere.

- When calculating emissions from the heatset printing operation, EU No. 006, it shall be assumed that 0.1% of the VOC emissions are retained in the paper substrate and all remaining VOC emissions are captured and vented to the RTO.

- When accounting for the RTO destruction efficiency, the RTO destruction efficiency achieved during the most recent successful stack test shall be used.

- When calculating emissions from the fountain solution usage, EU No. 006, it shall be assumed that 70% of the VOC emissions are vented to the RTO and 30% of the VOC emissions are vented uncontrolled to the atmosphere.

- When calculating emissions from the auto blanket wash usage, EU No. 006, it shall be assumed that 40% of the VOC emissions are vented to the RTO and 60% of the VOC emissions are vented uncontrolled to the atmosphere.

- The VOC emissions from the combustion of natural gas and the ink oil usage shall be included in the emission calculations for EU 006.

- When calculating emissions from the miscellaneous coating/additive usage, EU No. 008, it shall be assumed that 100% of the VOC emissions are vented uncontrolled to the atmosphere.

9.	Test the RTO for visible emissions (VE), destruction efficiency (DE), and capture efficiency (CE) within 60 days of initial startup of the new Printing Press Line I.  In lieu of conducting a capture efficiency test, the applicant may elect to perform EPA Method 204.  Testing shall be performed utilizing the test methods listed in Specific Condition Nos. 10 and 11.  Testing of emissions shall be conducted with the source operating at capacity.  Capacity is defined as all presses operating under normal operating conditions.  Failure to submit the input rates and actual operating conditions may invalidate the test.  [Rules 62-4.070(3) and 62-297.310(2)(b), F.A.C.]

10.	Compliance with the applicable requirements of Specific Condition Nos. 5, 7.A), and 9 shall be determined using EPA Method 9 for visible emissions and EPA Methods 1, 2, 3, 4, and 25A for VOC emissions.  The EPA Method 25A test shall be conducted simultaneously on the inlet and outlet of the RTO afterburner in order to determine the VOC destruction efficiency.  The EPA Method 9 test shall be conducted by a certified observer and shall be done concurrently with the destruction efficiency test. The Method 9 test shall be a minimum of thirty (30) minutes in duration.  The minimum requirements for stack sampling facilities, source sampling and reporting, shall be in accordance with Rule 62-297, F.A.C.  [Rules 62-4.070(3) and 62-297, F.A.C.]

11.	In order to demonstrate compliance with the 100% capture efficiency requirement specified in Specific Condition No. 7.D), a capture efficiency test (CE) shall be conducted in accordance with EPA VOC Capture Efficiency Test Procedures Rule 62-297.440(7), F.A.C or EPA Method 204.  The sampling time for each capture efficiency test run shall be as follows as approved by the Department pursuant to Rule 62-297.620, F.A.C.:  [Rules 62-296.515 and 62-297.450(3), F.A.C.; and ASP No. 99-C-01]

- Capture efficiency tests which use a total temporary enclosure or building enclosure  with one of the liquid/gas or gas/gas methods identified in Rules 62-297.450(2)(a) through (d), F.A.C., shall consist of at least three sampling runs.  Each run shall cover at least one complete production cycle, but shall be at least 3 hours long.  The sampling time for each run need not exceed 8 hours, even if the production cycle has not been completed.

- Capture efficiency tests which use the traditional liquid/gas method identified in Rule 297.450(2)(e),F.A.C., shall consist of the total number of runs needed to comply with either the data quality objective criteria or lower confidence limit criteria of Section 3.0 of EPA Emission Measurement Technical Information Center Guideline Document GD-035, “Guidelines for Determining Capture Efficiency,” January 9, 1995, adopted by reference at Rule 62-297.440, F.A.C.  However, each traditional liquid/gas capture efficiency test shall consist of at least 3 sampling runs.  The sampling time for each run shall be neither less than 20 minutes nor more than 24 hours.

12.	During the compliance tests, the quantity of the ink consumed from the ink totes shall be recorded and the VOC content shall be determined.  In addition, the temperature in the RTO afterburner shall be reported.  When testing the RTO, all Press Lines A, D, E, G, H, and I must be operating with their dryers venting to the RTO afterburner.  Failure to submit actual operating conditions may invalidate the test.   [Rule 62-297, F.A.C.]

13.	The permittee shall notify the Air Compliance Section of the Environmental Protection Commission of Hillsborough County at least 15 days prior to the date on which each formal compliance test is to begin of the date, time, and place of each such test, and the contact person who will be responsible for coordinating and having such test conducted.  [Rules 62-297.310(7)(a)9., F.A.C]

14.	The permittee shall maintain and operate a continuous temperature measuring device to monitor and record the temperature in the combustion chamber of the RTO afterburner within 10 percent accuracy.  [Rule 62-4.070(3), F.A.C.]

15.	In order to demonstrate compliance with the VOC limit specified in Specific Condition No. 4, the permittee shall maintain monthly records for the most recent three-year period.  The records shall be made available to the Environmental Protection Commission of Hillsborough County, state and federal officials upon request and shall include, but not limited to, the following.  [Rules 62-4.070(3) and 62-4.160(14), F.A.C.]

- Month, Year

- The amount of heatset inks, fountain solution, auto blanket wash, manual blanket wash, and miscellaneous coatings/additives used.

- VOC and HAP content as applied for each type of material.

- MSDS for each ink, fountain solution, auto blanket wash, manual blanket wash, and miscellaneous coating/additive.

- Monthly and 12 consecutive month rolling total summary of the facility-wide natural gas usage (MMCF).

- Monthly and 12 consecutive month rolling total summary of the amount of heatset inks, fountain solution, auto blanket wash, manual blanket wash, and miscellaneous coatings/additives used.

- Monthly and 12 consecutive month rolling total summary of VOC emissions, and individual and total HAP emissions from the inks, fountain solution auto blanket wash, manual blanket wash, miscellaneous coatings/additives, and natural gas usage.  When calculating VOC emissions the assumptions listed in Specific Condition No. 8 shall be used.

- If any twelve consecutive month period shows the actual VOC emissions to be 80.0 tons or greater, the VOC and HAP emissions shall be calculated on a daily basis until the rolling twelve month VOC totals are less than 80.0 tons.

16.	The permittee shall not store, handle, process, or use in any process, volatile organic compounds or organic solvents without applying known and existing vapor emission control devices or systems as follows and as deemed necessary and ordered by the Environmental Protection Commission of Hillsborough County: [Rule 62-296.320, F.A.C.]

- Maintain tightly fitting covers, lids, etc. on all containers when they are not being handled, tapped, etc.

- Where possible and practical, procure/fabricate a tightly fitting cover for any open trough, basin, etc. of VOC so that it can be covered when not in use.

- Immediately attend to all spills/waste as appropriate.

17.	All reasonable precautions shall be taken to prevent and control generation of unconfined emissions of particulate matter.  These provisions are applicable to any source, including, but not limited to, vehicular movement, transportation of materials, construction, alterations, demolition or wrecking, or industrial related activities such as loading, unloading, storing and handling.  [Rule 62-296.320, F.A.C.]

18.	When the Environmental Protection Commission of Hillsborough County (EPC) after investigation, has good reason (such as complaints, increased visible emissions or questionable maintenance of control equipment) to believe that any applicable emission standard contained in Rules 62-204, 62-210, 62-212, 62-296, or 62-297, F.A.C., or in a permit issued pursuant to those rules is being violated, it may require the owner or operator of the source to conduct compliance tests which identify the nature and quantity of pollutant emissions from the source and to provide a report on the results of said tests to the EPC.  [Rule 62-297.310(7)(b), F.A.C.]

19.	Submit to the Environmental Protection Commission of Hillsborough County each calendar year on or before April 1st, completed DEP Form 62-210.900(5), “Annual Operating Report for Air Pollutant Emitting Facility”, for the preceding calendar year.  [Rule 62-210.370(3), F.A.C.]

20.	The permittee shall provide timely notification to the Environmental Protection Commission of Hillsborough County prior to implementing any changes that may result in a modification to this permit pursuant to Rule 62-210.200, F.A.C., Modification.  The changes do not include normal maintenance, but may include, and are not limited to, the following, and may also require prior authorization before implementation:  [Rules 62-210.300 and 62-4.070(3), F.A.C.]

- Alteration or replacement of any equipment or major component of such equipment listed on page 1 through 3 of this permit.

- Installation or addition of any equipment which is a source of air pollution.

21.	A minimum of two copies of an air operating permit application shall be submitted to the Environmental Protection Commission of Hillsborough County within 90 days of completion of the compliance tests for VE, CE, and DE or at least 90 days prior to the expiration date of this permit, whichever occurs first.  [Rules 62-4.050(2) and 62-4.090, F.A.C.]

ENVIRONMENTAL PROTECTION COMMISSION

OF HILLSBOROUGH COUNTY

_________________________________

Richard D. Garrity, Ph.D.

Executive Director