TECHNICAL EVALUATION

PRELIMINARY DETERMINATION

Blacklidge Emulsions, Inc., Plant #3

Air Construction Permit

Application Number

0570461-021-AC

Environmental Protection Commission of

Hillsborough County

Tampa, Florida

May 14, 2014

# Project Description

## Applicant:

Andy Esposito

Plant Manager

Blacklidge Emulsions, Inc., Plant #3

2701 East 2nd Avenue

Tampa, Florida 33605

## Engineer:

Dean H. Myers, P.E.

General Environmental Engineering, Inc.

205 W. Palmetto Street

Lakeland, FL 33815

Registration No. 47666

## Project and Location:

On April 11, 2014, Blacklidge Emulsions submitted a permit application to convert the two (2) existing asphalt emulsion tanks to hot asphalt tanks.  The emissions from the storage of the hot asphalt product will be controlled by the existing control device, Air-Clear TM Fiber-Bed Mist Collection system and Carbon Adsorber Unit.  A new ducting system will be installed, which will vent the emissions from the two storage tanks to the control device. Also, this permit re-incorporates the natural gas fired heaters as regulated emission units under EU No. 004, since they are an integral component of the hot storage tanks.

The project has been assigned the following NEDS Source Classification Codes (SCCs): 3-05-002-99 (Asphalt Concrete, Comment: Asphalt Storage Tank); 3-05-002-06 (Asphalt Concrete, Asphalt Heater: Natural Gas).  The primary business at this facility is identified by the Standard Industrial Classification (SIC) for Petroleum and Coal Products – 2951, Asphalt Paving Mixtures and Blocks (NAICS 324121).  The project is located at 2701 East 2nd Avenue, Tampa, Hillsborough County, Florida; at longitude 82° 26' 47" W, latitude 27° 57' 24" N.

## Process and Controls:

Blacklidge Emulsions, Inc., Plant #3 is an asphalt blending and mixing facility that produces paving asphalt products.  The facility receives liquid asphalt cement and “off-spec” or unusable paving asphalt by tank truck and stores the asphaltic material in holding tanks, mixing tanks, or “cook-off” tanks.

Permit No. 0570461-018-AC originally authorized the installation of these two storage tanks (T10 and T11), to store hot asphalt product.  The two tanks were installed, however, they were not put into service due to the slow economy.  As a result, these two tanks were not incorporated into the operating permit and have sat empty until last year.  On May 29, 2013, the facility requested to use these two tanks (now E7 and E8) to store emulsified asphalt, which were determined to be generically exempt from air permitting, pursuant to Rule 62-210.300(3)(b)1., F.A.C.

Also, under the issuance of the renewal permit (Permit No. 0570461-020-AF, issued on March 6, 2013) EU 004 (Natural Gas Burners) was determined to be categorically exempt from air permitting pursuant to Rule 62-210.300(3)(a)34., F.A.C., since the burners collectively burn less than 375 MMSCF of natural gas per year.

However, during the review of this project, EPC staff re-evaluated the previous determination, and determined that since the NG burners are an integral component of the hot storage tanks, which are regulated emission units at this facility, they cannot be exempted from permitting. Therefore, the emissions from the NG burners are included in the facility-wide PTE and EU 004 (Natural Gas Heaters) is being re-activated in this AC permit.

The facility is not requesting any changes in the material throughput limits.  The total heat input for all natural gas heaters is 24 MMBtu/hr.  The PM and VOC potential-to-emit for this facility are 3.8 tpy and 16.6 tpy, respectively.  This facility is subject to Rule 62-296.320, F.A.C., since it is a source of PM, VOC, and a potential source of odors.  The facility is also subject to Chapter 1-3, Rules of the EPC, which limits visible emissions to a 5% opacity standard.

## Application Information:

Received on:  April 11, 2014

Request for Additional Information:  N/A

Application Complete:  April 11, 2014

# Rule Applicability

This project is subject to the preconstruction review requirements of Chapter 403, Florida Statutes, Chapters, 62-204, 62-210, 62-212, 62-296, and 62-297, Florida Administrative Code (F.A.C.) and Chapter 1-3 of the Rules of the Environmental Protection Commission of Hillsborough County.

This project is subject to the requirements of Rule 62-212.300, General Preconstruction Review Requirements, F.A.C., since the project is not exempt from the permit requirements in Rule 62-210.300, F.A.C.

This project is not subject to the requirements of Rule 62-212.400, Prevention of Significant Deterioration, F.A.C., since the facility is a minor source for PSD.

This project is not subject to the requirements Rule 62-212.500, Preconstruction Review for Nonattainment Areas, F.A.C., since the facility is a minor source and is not located in a designated nonattainment area.

This project is not subject to the requirements Rule 62-213, Operation Permits for Major Sources of Air Pollution, F.A.C., since the facility is a Synthetic Minor Non-Title V source.

This project is subject to the requirements of Rule 62-296.320, General Pollutant Emission Limiting Standards, F.A.C., since the project is a source of particulate matter, volatile organic compounds, and a potential source of odors.

This project is not subject to the requirements of Rule 62-296.401 through 62-296.480, Specific Emission Limiting and Performance Standards, F.A.C., since there is not an applicable source specific category in this rule.

This project is not subject to the requirements of Rule 62-296.500, Reasonably Available Control Technology (RACT) - Requirements for Volatile Organic Compounds (VOC) and Nitrogen Oxides (NOx) Emitting Facilities, F.A.C., since there is not an applicable source specific category in this rule.

This project is not subject to the requirements of Rule 62-296.600, Reasonably Available Control Technology - Lead, F.A.C., since there is not an applicable source specific category in this rule.

This project is not subject to the requirements of Rule 62-296.700, Reasonably Available Control Technology (RACT) – Particulate Matter, since there is no applicable source specific category in this rule.

This project is not subject to the requirements of Rule 62-204.800, Federal Regulations Adopted by Reference, F.A.C., since there is not an applicable source specific category in this rule.

This project is subject to the requirements of Chapter 84-446, Laws of Florida and Chapter 1-3, Rules of the Environmental Protection Commission of Hillsborough County.

# Summary of Emissions

| Emission Unit (EU) | Regulated Parameter | Actual Emissions (tons/yr) a | Potential Emissions (tons/yr) b | Allowable Limits |
| --- | --- | --- | --- | --- |
| 002 –Tanks & Truck Loading | PM | 0.1 | 0.4 |  |
|  | VOC | 4.5 | 16.0 |  |
|  | VE |  |  | 5% Opacity c |
| 003 – Asphalt Emulsion Tanks | PM | 0.8 | 2.6 |  |
|  | VOC | 3.4 | 12.0 |  |
|  | VE |  |  | 5% Opacity c |
| 004 – Natural Gas Heaters | PM | 0.04 | 0.8 |  |
|  | VOC | 0.03 | 0.6 |  |
|  | NOx | 0.49 | 10.3 |  |
|  | SOx | 0.003 | 0.1 |  |
|  | CO | 0.42 | 8.6 |  |
|  | VE |  |  | 20% Opacity |
| TOTAL d | VOC | 4.6 | 16.6 |  |
|  | PM | 0.9 | 3.8 |  |

# a.	The actual emissions are based on 2012 AOR production of 27,816 tons of paving asphalt and a total natural gas consumed estimated at 101,280 therms (10.128 MMCF).

# b.	The PTE are based on annual production of 98,000 tons of paving asphalt and a total natural gas heat input of 24 MMBtu/hr.

# c.	EUs 002 and 003 are subject to Rule 1-3.52, Rules of the EPC.

# d.	Total emissions are the summation of EU 004 and the worst case emissions for either EU002 or EU 003 based on the maximum of 98,000 tons of asphalt processed and 4 transfers.

# Conclusions

The revisions proposed by the applicant will meet all of the requirements of Chapters 62-204, 62-210, 62-212, 62-296, and 62-297, F.A.C., and Chapter 1-3, Rules of the Environmental Protection Commission.

The General and Specific Conditions listed in the proposed permit (attached) will assure compliance with all the applicable requirements of Chapters 62-204, 62-210, 62-212, 62-296, and 62-297, F.A.C.

# Proposed Agency Action

Pursuant to Section 403.087, Florida Statutes and Rule 62-4.070, Florida Administrative Code the Environmental Protection Commission of Hillsborough County hereby gives notice of its intent to issue an Air Construction Permit for the construction of four (4) new asphalt tanks, four (4) additional natural gas-fired burners, alteration of the Air-ClearTM fiber-bed mist control system to control 15 asphaltic material tanks, and the removal of the hot oil heater.

CERTIFIED MAIL

In the Matter of an	File No.:	0570461-021-AC

Application for Permit by:	County:	Hillsborough

Andy Esposito

Plant Manager

Blacklidge Emulsions, Inc., Plant #3

2701 East 2nd Avenue

Tampa, FL 33605

______________		____________________/

INTENT TO ISSUE

The Environmental Protection Commission of Hillsborough County (EPC), as delegated by the Florida Department of Environmental Protection (DEP) gives notice of its intent to issue a permit (copy attached) for the proposed project as detailed in the application specified above, for the reasons stated below.

The applicant, Blacklidge Emulsions, Inc., applied on April 11, 2014 to convert the two (2) existing asphalt emulsion tanks to hot asphalt tanks.  The emissions from the two tanks will be controlled by the existing control device, Air-Clear TM Fiber-Bed Mist Collection system and Carbon Adsorber Unit.  The facility is located at 2701 East 2nd Avenue, Tampa, Florida.

The EPC has permitting jurisdiction under Chapter 403 Florida Statutes (F.S.) and Florida Administrative Code (F.A.C.) Chapters 62-4 and 62-210.  The project is not exempt from permitting procedures.  The EPC has determined that an air pollution construction permit is required to commence or continue operations at the described facility.

The EPC intends to issue this permit based on the belief that reasonable assurances have been provided to indicate that operation of the source will comply with the appropriate provisions of Florida Administrative Code (F.A.C.) Chapters 62-204 through 62-297 and 62-4.

Pursuant to Section 403.815 and 403.0872, F.S. and Rules 62-103.150 and 62-210.350(3), F.A.C., you (the applicant) are required to publish at your own expense the enclosed Notice of Intent to Issue Permit.  The notice shall be published one time as soon as possible, in the legal advertisement section of a newspaper of general circulation in the area affected.  For the purpose of this rule, "publication in a newspaper of general circulation in the area affected" means publication in a newspaper meeting the requirements of Sections 50.011 and 50.031, F.S., in the county where the activity is to take place.  Where there is more than one newspaper of general circulation in the county, the newspaper used must be one with significant circulation in the area that may be affected by the permit.  If you are uncertain that a newspaper meets these requirements, please contact the EPC at the address or telephone number listed below.  The applicant shall provide proof of publication to the EPC, Air Permitting Section, at 3629 Queen Palm Drive, Tampa, Florida  33619 (Phone 813-627-2600 - FAX 813-627-2660) within 7 (seven) days of publication.  Failure to publish the notice and provide proof of publication within the allotted time may result in the denial of the permit pursuant to Rule 62-103.150(6), F.A.C.

The EPC will issue the final permit with the attached conditions unless a timely petition for an administrative hearing is filed pursuant to Section 120.569 and 120.57 F.S. before the deadline for filing a petition.  The procedures for petitioning for a hearing are set forth below.

A person whose substantial interests are affected by the proposed permitting decision may petition for an administrative proceeding (hearing) under Sections 120.569 and 120.57, F.S.  The petition must contain the information set forth below and must be filed (received) in the Legal Department of the EPC at 3629 Queen Palm Drive, Tampa, Florida 33619, Phone 813-627-2600, Fax 813-627-2602.  Petitions filed by the permit applicant or any of the parties listed below must be filed within 14 (fourteen) days of receipt of this notice of intent.  Petitions filed by any persons other than those entitled to written notice under Section 120.60(3), F.S. must be filed within 14 (fourteen) days of publication of the public notice or within 14 (fourteen) days of receipt of this notice of intent, whichever occurs first.  Under Section 120.60(3), however, any person who asked the EPC for notice of agency action may file a petition within 14 (fourteen) days of receipt of that notice, regardless of the date of publication.  A petitioner shall mail a copy of the petition to the applicant at the address indicated above, at the time of filing.  The failure of any person to file a petition within the appropriate time period shall constitute a waiver of that person's right to request an administrative determination (hearing) under Sections 120.569 and 120.57, F.S.; or to intervene in this proceeding and participate as a party to it.  Any subsequent intervention will be only at the approval of the presiding officer upon the filing of a motion in compliance with Rule 28-106.205, F.A.C.

A petition that disputes the material facts on which the EPC’s action is based must contain the following information:

- The name and address of each agency affected and each agency’s file or identification number if known;

- The name, address, and telephone number of the petitioner and the name, address, and telephone number of each petitioner’s representative, if any, which shall be the address for service purposes during the course of the proceedings; and an explanation of how the petitioner’s substantial interests will be affected by the EPC’s determination;

- A statement of how and when the petitioner received notice of the EPC action;

- A statement of all disputed issues of material fact. If there are none, the petition must so indicate;

- A concise statement of the ultimate facts alleged, including the specific facts the petitioner contends warrant reversal or modification of the EPC’s proposed action;

- A statement of specific rules or statutes the petitioner contends requires reversal or modification of the EPC’s proposed action; and

- A statement of the relief sought by the petitioner, stating precisely the action petitioner wishes the EPC to take with respect to the EPC’s proposed action.

A petition that does not dispute the material facts upon which the EPC’s action is based shall state that no such facts are in dispute and otherwise shall contain the same information as set forth above as required by Rule 28-106.301, F.A.C.

Because the administrative hearing process is designed to formulate final agency action, the filing of a petition means that the EPC's final action may be different from the position taken by it in this notice of intent. Persons whose substantial interests will be affected by any such final decision of the EPC on the application have the right to petition to become a party to the proceeding, in accordance with the requirements set forth above.

Mediation under section 120.573, F.S. is not available in this proceeding.

This action is final and effective on the date filed with the Clerk of the EPC unless a petition is filed in accordance with above.  Upon the timely filing of a petition, this order will not be effective until further order of the EPC.

In addition to the above, a person subject to regulation has a right to apply to the Department of Environmental Protection for a variance from or waiver of the requirements of particular rules, on certain conditions, under Section 120.542, F.S.  The relief provided by this state statute applies only to state rules, not statutes, and not to any federal regulatory requirements.  Applying for a variance or waiver does not substitute or extend the time for filing a petition for an administrative hearing or exercising any other right that a person may have in relation to the action proposed in this notice of intent.

The application for a variance or waiver is made by filing a petition with the Office of General Counsel of the Department of Environmental Protection, 3900 Commonwealth Boulevard, Mail Station #35, Tallahassee, FL 32399-3000.  The petition must specify the following information:

- The name, address, and telephone number of the petitioner,

- The name, address, and telephone number of the attorney or qualified representative of the petitioner, if any,

- Each rule or portion of a rule from which a variance or waiver is requested,

- The citation to the statute underlying (implemented by) the rule identified in (c) above,

- The type of action requested,

- The specific facts that would justify a variance or waiver for the petitioner,

- The reason by the variance or waiver would serve the purposes of the underlying statute (implemented by the rule), and

- A statement whether the variance or waiver is permanent or temporary and, if temporary, a statement of the dates showing the duration of the variance or waiver requested.

The Department will grant a variance or waiver when the petition demonstrates both that the application of the rule would create a substantial hardship or violate principles of fairness, as each of the those terms is defined in Section 120.542(2), F.S., and that the purpose of the underlying statute will be or has been achieved by other means by the petitioner.

Persons subject to regulation pursuant to any federally delegated or approved air program should be aware that Florida is specifically not authorized to issue variances or waivers from any requirements of any such federally delegated or approved program.  The requirements of the program remain fully enforceable by the Administrator of United States Environmental Protection Agency and by any person under the Clean Air Act unless and until the Administrator separately approves any variance or waiver in accordance with the procedures of the federal program.

Any person listed below may request to obtain additional information, a copy of the application (except for information entitled to confidential treatment pursuant to Section 403.111, F.S.), all relevant supporting materials, and all other materials available to the EPC that are relevant to the permit decision.  Interested persons may contact Diana M. Lee, P.E., at the above address or call (813) 627-2600, for additional information.

Any party to this order has the right to seek judicial review of it under Section 120.68 of the Florida Statues, by filing a notice of appeal under rule 9.110 of the Florida rules of Appellate Procedure with the EPC’s Legal Office at 3629 Queen Palm Drive, Tampa, Florida 33619 and with the clerk of the Department of Environmental Protection in the Office of General Counsel, Mail Station 35, 3900 Commonwealth Boulevard, Tallahassee, Florida 32399-3000, and by filing a copy of the notice of appeal accompanied by the applicable filing fees with the appropriate district court of appeal.  The notice must be filed within thirty days after this order is filed with the clerk of the Department.

Executed in Tampa, Florida

ENVIRONMENTAL PROTECTION COMMISSION

OF HILLSBOROUGH COUNTY

__________________________________

Richard D. Garrity, Ph.D.

Executive Director

cc:  Dean H. Meyers, P.E. - General Environmental Engineering, Inc.

## CERTIFICATE OF SERVICE

The undersigned duly designated clerk hereby certifies that this INTENT TO ISSUE and all copies were mailed by certified mail before the close of business on _________________ to the listed persons.

FILING AND ACKNOWLEDGEMENT

FILED, on this date, pursuant to Section 120.52(7), Florida Statutes, with the designated clerk, receipt of which is hereby acknowledged.

_____________________   _____________

Clerk                                       Date

ENVIRONMENTAL PROTECTION COMMISSION

# OF HILLSBOROUGH COUNTY

NOTICE OF INTENT TO ISSUE PERMIT

The Environmental Protection Commission of Hillsborough County (EPC), as delegated by the Florida Department of Environmental Protection (DEP) gives notice of its intent to issue an air pollution Permit No. 0570461-021-AC, to Blacklidge Emulsions, Inc., an asphalt blending and mixing facility that produces paving asphalt products.  The air construction permit authorizes the facility to convert the two (2) existing asphalt emulsion tanks to hot asphalt tanks.  The emissions from the two tanks will be controlled by the existing control device, Air-Clear TM Fiber-Bed Mist Collection system and Carbon Adsorber Unit. The facility is a Synthetic Non-Title V source located at 2701 East 2nd Avenue, Tampa, Florida.

A Best Available Control Technology (BACT) determination was not required.

The EPC will issue the Final permit with the conditions of the DRAFT permit unless a timely petition for an administrative hearing is filed pursuant to Sections 120.569 and 120.57, F.S. before the deadline for filing a petition.  The procedures for petitioning for hearing are set forth below.

A person whose substantial interests are affected by the proposed permitting decision may petition for an administrative proceeding (hearing) under Sections 120.569 and 120.57, F.S.  The petition must contain the information set forth below and must be filed (received) in the Legal Department of the EPC at 3629 Queen Palm Drive, Tampa, Florida 33619, Phone 813-627-2600, Fax 813-627-2602.  Petitions filed by the permit applicant or any of the parties listed below must be filed within 14 (fourteen) days of receipt of this notice of intent.  Petitions filed by any persons other than those entitled to written notice under Section 120.60(3), F.S. must be filed within 14 (fourteen) days of publication of the public notice or within 14 (fourteen) days of receipt of this notice of intent, whichever occurs first.  Under Section 120.60(3), however, any person who asked the EPC for notice of agency action may file a petition within 14 (fourteen) days of receipt of that notice, regardless of the date of publication.  A petitioner shall mail a copy of the petition to the applicant at the address indicated above, at the time of filing.  The failure of any person to file a petition within the appropriate time period shall constitute a waiver of that person's right to request an administrative determination (hearing) under Sections 120.569 and 120.57, F.S., or to intervene in this proceeding and participate as a party to it.  Any subsequent intervention will be only at the approval of the presiding officer upon the filing of a motion in compliance with Rule 28-106.205 of the F.A.C.

A petition that disputes the material facts on which the EPC’s action is based must contain the following information:

- The name and address of each agency affected and each agency’s file or identification number if known;

- The name, address, and telephone number of the petitioner, and the name, address, and telephone number of each petitioner’s representative, if any, which shall be the address for service purposes during the course of the proceedings; and an explanation of how the petitioner’s substantial interests will be affected by the agency determination;

- A statement of how and when petitioner received notice of the EPC action;

- A statement of all disputed issues of material fact.  If there are none, the petition must so indicate;

- A concise statement of the ultimate facts alleged, including the specific facts the petitioner contends warrant reversal or modification of the EPC proposed action;

- A statement of specific rules or statutes the petitioner contends requires reversal or modification of the EPC’s proposed action; and

- A statement of the relief sought by the petitioner, stating precisely the action petitioner wishes the EPC to take with respect to the EPC’s proposed action.

A petition that does not dispute the material facts upon which the EPC’s action is based shall state that no such facts are in dispute and otherwise shall contain the same information as set forth above as required by Rule 28-106.301.

Because the administrative hearing process is designed to formulate final agency action, the filing of a petition means that the EPC's final action may be different from the position taken by it in this notice of intent.  Persons whose substantial interests will be affected by any such final decision of the EPC on the application have the right to petition to become a party to the proceeding, in accordance with the requirements set forth above.

Mediation under section 120.573, F.S. is not available in this proceeding.

This action is final and effective on the date filed with the Clerk of the EPC unless a petition is filed in accordance with above.  Upon the timely filing of a petition this order will not be effective until further order of the EPC.

Any party to this order has the right to seek judicial review of it under Section 120.68 of the Florida Statues, by filing a notice of appeal under rule 9.110 of the Florida rules of Appellate Procedure with the EPC’s Legal Office at 3629 Queen Palm Drive, Tampa, Florida 33619 and with the clerk of the Department of Environmental Protection in the Office of General Counsel, Mail Station 35, 3900 Commonwealth Boulevard, Tallahassee, Florida 32399-3000, and by filing a copy of the notice of appeal accompanied by the applicable filing fees with the appropriate district court of appeal.  The notice must be filed within thirty days after this order is filed with the clerk of the Department.

The complete project file is available for public inspection during normal business hours, 8:00 a.m. to 5:00 p.m., Monday through Friday, except legal holidays, at the Environmental Protection Commission of Hillsborough County, 3629 Queen Palm Drive, Tampa, Florida 33619.  The complete project file includes the proposed Permit, the application, and the information submitted by the responsible official, exclusive of confidential records under Section 403.111, F.S.  Interested persons may contact Diana M. Lee, P.E., at the above address, or call 813-627-2600, for additional information.  Any written comments filed shall be available for public inspection.  If written comments received result in a significant change in the proposed agency action, the EPC shall revise the proposed permit and require, if applicable, another Public Notice.

ENVIRONMENTAL PROTECTION COMMISSION OF

HILLSBOROUGH COUNTY, as Delegated by

STATE OF FLORIDA

DEPARTMENT OF ENVIRONMENTAL PROTECTION

NOTICE OF PERMIT

Andy Esposito

Plant Manager

Blacklidge Emulsions, Inc., Plant #3

2701 East 2nd Avenue

Tampa, FL 33605

Re:  Hillsborough County - AP

Dear Mr. Esposito:

Enclosed is Permit Number 0570461-021-AC to convert the two (2) existing asphalt emulsion tanks (E7 and E8) to hot asphalt tanks, issued pursuant to Section 403.087, Florida Statutes.

Any party to this order (permit) has the right to seek judicial review of the permit pursuant to Section 120.68, Florida Statutes, by the filing of a Notice of Appeal pursuant to Rule 9.110, Florida Rules of Appellate Procedure, with the Clerk of the EPC in the Legal Department at 3629 Queen Palm Dr., Tampa, FL  33619; and by filing a copy of the Notice of Appeal accompanied by the applicable filing fees with the appropriate District Court of Appeal.  The Notice of Appeal must be filed within 30 days from the date this Notice is filed with the clerk of the EPC.

Executed in Tampa, Florida.

Sincerely,

Richard D. Garrity, Ph.D.

Executive Director

RDG/KRZ/krz

cc:  Dean H. Meyers, P.E. - General Environmental Engineering, Inc.

CERTIFICATE OF SERVICE

This is to certify that this NOTICE OF PERMIT and all copies were mailed before the close of business on _________________________ to the listed persons.

Clerk Stamp

FILED, on this date, pursuant to Section 120.52(7), Florida Statutes, with the designated clerk, receipt of which is hereby acknowledged.

___________________________   _____________

Clerk	Date

PERMITTEE:	PERMIT/CERTIFICATION

Blacklidge Emulsions, Inc.	Permit No.: 0570461-021-AC

Plant #3	County: Hillsborough

2701 East 2nd Avenue	Expiration Date:  May 31, 2015

Tampa, Florida 33605	Project: Conversion of Tanks E7 and E8

This permit is issued under the provisions of Chapter 403, Florida Statutes (F.S.), and Florida Administrative Code (F.A.C.) Chapters 62-204, 62-210, 62-212, 62-213, 62-296, 62-297, and Chapter 62-4.  The above named permittee is hereby authorized to perform the work or operate the facility shown on the application and approved drawing(s), plans and other documents, attached hereto or on file with the Environmental Protection Commission (EPC) of Hillsborough County and made a part hereof and specifically described as follows:

This air construction (AC) permit authorizes the facility to convert the two (2) existing asphalt emulsion tanks to hot asphalt tanks.  The emissions from the two tanks will be controlled by the existing control device, Air-Clear TM Fiber-Bed Mist Collection system and Carbon Adsorber Unit. A new ducting system will be installed, which will vent the emissions from the two storage tanks to the control device.  Also, this permit re-incorporates the natural gas fired heaters as regulated emission units under EU 004, since they are an integral component of the hot storage tanks.

Blacklidge Emulsions, Inc., Plant #3 is an asphalt blending, mixing, and transfer facility that produces paving asphalt products.  These products include polymer modified asphalt, recycled asphalt blends, and asphalt emulsions.  The facility receives liquid asphalt cement and stores it in one of five tanks, H1 through H5.  Various raw materials are received in drums, bags, or super sacks.  Polymer modified asphalt is made in either mix tank P1 or P2 (both horizontal mix tanks) by adding polymer and other additives to the liquid asphalt.  Once this process has taken place, the material is then transferred to one of the H series tanks.  Tanks T1 through T3 are “cook-off” tanks, which remove the water from off-specification asphalt emulsions.  Tanks T4 through T6 are RA tanks, and Tank T7 may contain either anti-strip additive or flux oil additive that is stored at ambient temperature.

The particulate matter and volatile organic compounds emitted from the heated asphalt cook-off, mixing, and holding tanks and truck loading station are controlled by the Air-Clear TM Fiber-Bed Mist Collection system and Carbon Adsorber Unit.  The unit has a design exhaust flow rate of 2,600 cfm and operates up to a maximum temperature of 140 °F.  The Vapor Phase Carbon Adsorber Unit

containing at least 1,600 pounds of activated carbon and designed for airflow up to 4,000 cfm is connected to the exhaust of the Air-ClearTM fiber-bed mist collection system.  An induced-draft fan located downstream of the carbon adsorber provides sufficient air flow through the air pollution and odor control system to evacuate the headspace of all heated asphalt tanks during tank loading or unloading, mixing and blending, and maintain negative pressures within the tank when the tank hatch is opened to add polymer, aggregate, or other additives to manufacture paving asphalt products.

Besides the two emulsified asphalt tanks (E7 and E8), which will be converted to hot asphalt tanks under this AC project, the facility has five (5) emulsified asphalt tanks (E1 thru E5) that are generally kept at ambient temperature.  However, these tanks may be heated, as necessary, to maintain product quality during cold weather conditions.  These tanks are not vented to the air pollution control device.  The facility also has auxiliary soap, water, and chemical additive tanks to support the emulsified asphalt manufacturing process.

Heat for the asphalt storage and blending tanks is provided by electric heaters or natural gas (NG) burners.  Under the issuance of the renewal permit (Permit No. 0570461-020-AF, issued on March 6, 2013) EU 004 (Natural Gas Burners) was determined to be categorically exempt from air permitting pursuant to Rule 62-210.300(3)(a)34., F.A.C., since the burners collectively burn less than 375 MMSCF of natural gas per year.  However, during the review of this project, EPC staff re-evaluated the previous determination, and determined that since the NG burners are an integral component of the hot storage tanks, which are regulated emission units at this facility, they cannot be exempted from permitting. Therefore, the emissions from the NG burners are included in the facility-wide PTE and EU 004 (Natural Gas Heaters) is being re-activated in this AC permit

The total heat input for all natural gas heaters is 24 MMBtu/hr.  The PM and VOC potential-to-emit for this facility are 3.8 tpy and 16.6 tpy, respectively.  This facility is subject to Rule 62-296.320, F.A.C., since it is a source of PM, VOC, and a potential source of odors.  The facility is also subject to Chapter 1-3, Rules of the EPC, which limits visible emissions to a 5% opacity standard.

Tank T7, which can store anti-strip additive or flux oil for the manufacture of RA asphalt, is exempt from air permitting pursuant to Rule 62-210.300(3)(b)1. F.A.C.

Location:  2701 East 2nd Avenue, Tampa, Hillsborough County

UTM Coordinates: 17-359.5 E  3093.2 N

NEDS No.:  0570461

| EU ID# | Description | Capacity |
| --- | --- | --- |
| 002 – Tanks and Truck Loading | Raw/Product Asphalt Storage Tank (H1) Raw/Product Asphalt Storage Tank (H2) Raw/Product Asphalt Storage Tank (H3) Raw/Product Asphalt Storage Tank (H4) Raw/Product Asphalt Storage Tank (H5) “Cook-off” Tank (T1) “Cook-off” Tank (T2) “Cook-off” Tank (T3) Recycled Asphalt Tank (T4) Recycled Asphalt Tank (T5) Recycled Asphalt Tank (T6) Recycled Asphalt Additive Tank (T7) - exempt Polymer Mix Tank (P1) Polymer Mix Tank (P2) Hot Asphalt Storage Tank (E7) - new Hot Asphalt Storage Tank (E8) - new Asphalt Product Truck Loading | 25,000 gallons 25,000 gallons 25,000 gallons 25,000 gallons 25,000 gallons 5,000 gallons 5,000 gallons 10,000 gallons 15,000 gallons 15,000 gallons 18,000 gallons 8,000 gallons 25,000 gallons 25,000 gallons 40,000 gallons 40,000 gallons |
| 003 – Asphalt Emulsion Tanks | Asphalt Emulsion Tank (E1) Asphalt Emulsion Tank (E2) Asphalt Emulsion Tank (E3) Asphalt Emulsion Tank (E4) Asphalt Emulsion Tank (E5) | 13,500 gallons 13,500 gallons 29,000 gallons 29,000 gallons 13,500 gallons |
| 004 – Natural Gas Heaters | H1 Natural Gas-fired Tank Heater H2 Natural Gas-fired Tank Heater H3 Natural Gas-fired Tank Heater H4 Natural Gas-fired Tank Heater H5 Natural Gas-fired Tank Heater T1 Natural Gas-fired Tank Heater T2 Natural Gas-fired Tank Heater T3 Natural Gas-fired Tank Heater T4 Natural Gas-fired Tank Heater T5 Natural Gas-fired Tank Heater T6 Natural Gas-fired Tank Heater E7 Natural Gas-fired Tank Heater - new E8 Natural Gas-fired Tank Heater - new P1 Natural Gas-fired Tank Heater P2 Natural Gas-fired Tank Heater E4 Asphalt Emulsion Tank Heater | 1 MMBtu/hr 1 MMBtu/hr 1 MMBtu/hr 1 MMBtu/hr 1 MMBtu/hr 1 MMBtu/hr 1 MMBtu/hr 2 MMBtu/hr 2 MMBtu/hr 2 MMBtu/hr 2 MMBtu/hr 2 MMBtu/hr 2 MMBtu/hr 2 MMBtu/hr 2 MMBtu/hr 1 MMBtu/hr |

References Permit Nos.: 0570461-017-AC and 0570461-020-AF

- A part of this permit is the attached General Conditions. [Rule 62-4.160, F.A.C.]

- Issuance of this permit does not relieve the permittee from complying with applicable emission limiting standards or other requirements of Chapters 62-204, 62-210, 62-212, 62-296 and 62-297, F.A.C., or any other requirements under federal, state, or local law. [Rule 62-210.300, F.A.C.]

- All applicable rules of the Environmental Protection Commission of Hillsborough County including design discharge limitations specified in the application shall be adhered to.  The permit holder may also need to comply with county, municipal, federal, or other state regulations prior to construction. [Rule 62-4.070(7), F.A.C.]

- The use of property, facilities, equipment, processes, products, or compounds, or any other act that causes or materially contributes to a public nuisance is prohibited, pursuant to the Hillsborough County Environmental Protection Act, Section 16, Chapter 84-446, Laws of Florida, as Amended.

- The facility is permitted to operate continuously; 8,760 hours per 12-consecutive month period.  [Rule 62-210.200(247) (Potential to Emit), F.A.C.]

- As requested by the permittee, in order to limit the potential-to-emit, the facility shall not produce more than 98,000 tons of paving asphalt product in any 12-consecutive month period.

[Rule 62-210.200(247) and 62-4.070(3), F.A.C. and Permit Nos. 0570461-017/020-AC/AF]

- As requested by the permittee, in order to limit the potential-to-emit, the facility’s operations shall be limited as follows:

  - The temperature of the asphalt material mixing and storage tanks shall not exceed 375 °F; however, temperature spikes up to 400 oF are allowed during mixing and blending operations.

  - Only asphalt cement as presented in the Air Construction Permit Application shall be used.  The use of lower boiling point asphalt cement or cutback asphalt without prior approval by the EPC is prohibited;

[Rules 62-4.070(3), F.A.C. and Permit Nos. 0570461-017/020-AC/AF]

- The maximum allowable visible emissions from the Tank & Truck Loading activities (EU 002) and the Asphalt Emulsion Tanks (EU 003) shall not be equal to or greater than 5% opacity.

| Emission Unit | Description | VE Limit (% opacity) |
| --- | --- | --- |
| 002 | Tanks and Truck Loading | 5 |
| 003 | Asphalt Emulsion Tanks (E1-E5) | 5 |

[Rule 62-4.070(3), F.A.C., Chapter 1-3.52, Rules of the EPCHC and Permit Application received April 11, 2014]

- The facility shall maintain the system to control Visible Emissions and Odors from EU 002.  The system shall be configured with the following specifications:

A.  An Air-Clear TM fiber-bed mist collection system with a maximum exhaust flow of 2,600 acfm;

B.  As recommended by the manufacturer, the inlet temperature of the Air-ClearTM system shall not exceed 140° Fahrenheit (F).  In addition, since the cooling of the exhaust prior to the Air-ClearTM system is imperative in effectively collecting the submicron particulates that cause opacity, the permittee shall make all attempts to minimize the exhaust temperature to the extent practical;

C.  The facility shall install, calibrate, operate, and maintain an inlet thermometer and inlet 	temperature chart recorder in degrees Fahrenheit for the Air-ClearTM fiber-bed mist collection 	system;

D.  The facility shall install, calibrate, operate, and maintain a pressure drop measuring device 	in inches of water across the main fiber-bed diffusion filters of the Air-ClearTM system.  As 	recommended by the manufacturer, the pressure drop across the main fiber-bed filters shall be 	in the range of 0 – 3 inches water gauge (w.g.) when new; the filters should be changed when 	the pressure drop approaches 12 inches w.g.

E.  The facility shall install, calibrate, operate, and maintain a static pressure gauge at the inlet 	of the Air-ClearTM fiber-bed mist collection system that is integrated with the variable 	frequency drive fan to verify adequate air flow through the system.

F.  Prior to each instance of the addition of polymer additive to the polymer mix tanks, the 	facility shall verify adequate air flow as measured by the static pressure gauge;

G.  A complete spare filter set (enough to replace all the filters in the unit) for the Air-Clear TM 	system shall be kept on site or at the Montgomery Street site;

H.  A Vapor Phase Carbon Adsorber Unit containing at least 1,600 pounds of activated carbon 	and designed for an airflow of up to 4,000 acfm shall be connected to the exhaust of the Air-	ClearTM fiber-bed mist collection system;

I.  The Carbon Adsorber Unit shall be monitored and maintained as prescribed in the Carbon 	Adsorber OM&M Plan (Attachment C) to maintain the efficiency of the odor control system;

J. As per the manufacturer’s recommendations, the facility shall measure the inlet and outlet 	temperature of the Carbon Adsorber Unit immediately upon installation of a new carbon 	canister and monitor the outlet temperature every four (4) hours for the first twenty-four (24) 	hours of operation.

[Rule 62-4.070(3), F.A.C., Permit Nos. 0570461-017/020-AC/AF, Permit Application received April 11, 2014, and Carbon Adsorber and Air ClearTM O&M Manuals]

- [RESERVED]

- No person shall store, pump, handle, process, load, unload, or use in any process or installation any VOC/OS without applying known and existing vapor emission control devices or systems deemed necessary and ordered by the Environmental Protection Commission of Hillsborough County.  The following procedures shall be utilized to minimize pollutant emissions:

- Maintain covers, lids, etc., on all containers of VOC/OS when they are not being handled, tapped, etc.

- Prevent excessive air turbulence across exposed VOC/OS.

- Where possible and practical, procure/fabricate a tightly fitting cover for any open trough, basin, bath, etc., of VOC/OS so that it can be covered when not in use.

- All fittings, valve lines, etc., shall be properly maintained.

- All VOC/OS spills shall be attended to in a timely manner and the waste properly disposed of, recycled, etc.

[Rule 62-296.320(1), F.A.C.]

- The permittee shall not cause, suffer, allow or permit the discharge of air pollutants which cause or contribute to an objectionable odor.  An objectionable odor is any odor present in the outdoor atmosphere which by itself or in combination with other odors, is or may be harmful or injurious to human health or welfare, which unreasonably interferes with the comfortable use and enjoyment of life or property, or which creates a nuisance.  During tank loading/truck loadout and tank to tank transfer/blending operations, a representative of Blacklidge Emulsions shall monitor daily the downwind property boundary for odors.  If objectionable odors are noted, the operation(s) shall be curtailed or halted.  If the Environmental Protection Commission of Hillsborough County receives odor complaints attributed to this facility and they are determined to be a nuisance, the permittee will need to evaluate their operations and propose corrective actions that may include installing or implementing additional odor control measures for this facility.

[Rules 62-4.070(3), 62-296.320, F.A.C., and Chapter 1-3.22, Paragraph 3, Rules of the EPCHC]

- All reasonable precautions shall be taken to prevent and control generation of unconfined emissions of particulate matter in accordance with the provision in Rule 62-296.320, F.A.C.  These provisions are applicable to any source, including, but not limited to, vehicular movement, transportation of materials, construction, alterations, demolition or wrecking, or industrial related activities such as loading, unloading, storing, and handling.  Reasonable precautions shall include, but not limited to, the following:

- Maintenance of parking areas and yards.  Use water or chemical dust suppressants, if necessary;

- Removal of particulate matter from paved areas and work areas under the control of the owner/operator;

- Monitor each tank/truck loading operation and observe the associated equipment and exhaust stack for visible emissions;

- Curtail or halt operation if visible emissions are noted.

[Rule 62-296.320, F.A.C.]

- The permittee shall not circumvent any air pollution control device, or allow the emissions of air pollutants without the applicable air pollution control device operating properly.

[Rules 62-4.070(3), and 62-210.650, F.A.C.]

- Excess emissions which are caused entirely or in part by poor maintenance, poor operation, or any other equipment or process failure which may reasonably be prevented during startup, shutdown, or malfunction shall be prohibited. The pollution control equipment (fiber-bed mist eliminator, odor control system, and associated equipment) shall be maintained in good repair to perform adequately the function for which it was intended.  The permittee shall lower the temperature in the tanks as necessary to comply with the limitations in Specific Condition Nos. 8 and 9.  Maintenance shall include, but is not limited to, monthly inspections and repair or replacement of faulty equipment when necessary.

[Rules 62-4.070(3), 62-210.700(4), and 62-210.700(5), F.A.C.]

- In order to demonstrate compliance with Specific Condition No. 8, the permittee shall conduct a visible emission test on the carbon absorber exhaust stack within thirty (30) days of storing hot asphalt product in the modified storage tanks (E7 and E8), controlled by the Air-ClearTM fiber-bed mist collection system.

[Rules 62-297.310(7)(a), F.A.C., Chapter 1-3.52, Rules of the EPCHC and Permit Application received April 11, 2014]

- Concurrent with the annual VE test for EU 002, perform an odor evaluation (test team’s opinion whether objectionable odors are being emitted from the facility).  The odor evaluation shall be conducted during normal tank loading/truck loadout and tank to tank transfer/blending operations.  The odor evaluation report shall include the names of the odor evaluation team members, date, time, wind direction, observations, tank loading/truck loadout and tank to tank transfer/blending operations taking place at the time, and any corrective actions taken.  [Rule 62-297.310(4)(a)2 and (8), F.A.C.]

- Visible emissions tests shall be conducted by a certified observer in accordance with EPA Method 9 for a minimum of thirty (30) minutes in duration and shall include the period during which the highest opacity emissions can reasonably be expected to occur.

[Rules 62-297.310(4)(a)2, and 62-297.401(9)(a) F.A.C., and Chapter 1-3.52, Rules of the EPCHC]

- The permittee shall notify the Air Compliance Section of the EPC at least 15 days prior to the date on which each formal compliance test is to begin, of the date, time, and place of each such test, and the test contact person who will be responsible for coordinating and having such test conducted.  [Rule 62-297.310(7)(a)(9), F.A.C.]

- The permittee will submit two (2) copies of the compliance test report required by this permit to the Air Compliance Section of the EPC within forty-five (45) days after the test is complete.  The test report shall contain sufficient detail on the source tested and the test procedures used to allow the EPC to determine if the test was properly conducted and the test results properly computed.

[Rule 62-297.310(8), F.A.C.]

- When the Environmental Protection Commission of Hillsborough County (EPC) after investigation, has good reason (such as complaints, increased visible emissions, or questionable maintenance of control equipment) to believe that any applicable emission standard contained in Rules 62-204, 62-210, 62-212, 62-296, or 62-297, F.A.C., or in a permit issued pursuant to those rules is being violated, it shall require the owner or operator of the source to conduct compliance tests which identify the nature and quantity of pollutant emissions from the source and to provide a report on the results of said tests to the EPC. [Rule 62-297.310(7)(b), F.A.C.]

- The permittee shall maintain a monthly log of operations.  The log shall include, at a minimum, the following items and be kept for a minimum of three (3) years and made available for inspection to representatives of the Environmental Protection Commission of Hillsborough County upon request.

- Daily maximum temperature of each blend batch (RA asphalt, polymer modified asphalt, and emulsified asphalt) of paving asphalt product;

- Total weight of emulsified asphalt produced, in short tons, on a monthly basis and the rolling total for the previous 12-consecutive month period;

- Total weight of all paving asphalt produced, in short tons, on a monthly basis and the rolling total for the previous 12-consecutive month period;

- Copies of the inlet temperature charts to the Air-ClearTM system as required in Specific Condition No. 9.C.;

- Weekly pressure drop, in inches of water, of the main fiber-bed diffusion filters for the Air-ClearTM system;

- Weekly static pressure gauge reading for the inlet of the Air-ClearTM system;

- Record of odor monitoring downwind of property per Specific Condition No. 9.H. that shall include date, time, wind direction, observations, tank loading/truck loadout, tank to tank transfer/blending operations taking place at the time, operator initials, and corrective actions taken;

- Any maintenance/repair performed on the Air-ClearTM and Carbon Adsorber system.

[Rule 62-4.070(3) and 62-4.160(14)(b), F.A.C., Permit Nos. 0570461-017/020-AC/AF and Permit Application received April 11, 2014]

- The permittee shall submit to the Environmental Protection Commission of Hillsborough County, by April 1st of each calendar year, a completed DEP Form 62-210.900(5), "Annual Operating Report (AOR) for Air Pollutant Emitting Facility", for the preceding calendar year.

[Rule 62-210.370(3), F.A.C.]

- If the permittee wishes to transfer this permit to another owner, an "Application for Transfer of Permit" (DEP Form 62-210.900(7)) shall be submitted, in duplicate, to the Environmental Protection Commission of Hillsborough County within 30 days after the sale or legal transfer of the permitted facility. [Rule 62-4.120, F.A.C.]

- The permittee shall provide timely notification to the Environmental Protection Commission of Hillsborough County prior to implementing any changes that may result in a modification to this permit pursuant to Rule 62-210.200, F.A.C., Modification.  The changes do not include normal maintenance, but may include, and are not limited to, the following, and may also require prior authorization before implementation:

- Alteration or replacement of any equipment or major component of such equipment;

- Installation or addition of any equipment which is a source of air pollution;

- Use of any type of asphalt or chemical other than those authorized by this permit.

Note:	Items A. and B. are not applicable to routine maintenance, repair, or replacement of component parts of an air emissions unit. [Rules 62-210.300 and 62-4.070(3), F.A.C.]

- A complete application for an air operation permit shall be submitted to the Environmental Protection Commission of Hillsborough County within 60 days of completion of the visible emissions test required under Specific Condition No. 16 or at least 90 days prior to the expiration date of this permit, whichever occurs first.

[Rules 62-4.070(3), 62-4.090, 62-210.300(2), and 62-210.900, F.A.C.]

ENVIRONMENTAL PROTECTION COMMISSION

OF HILLSBOROUGH COUNTY

_________________________________

Richard D. Garrity, Ph.D.

Executive Director