Environmental

Compliance

Plan

Prepared for:

Pro-Mac Machine, Inc.

6156 15th Street East

Bradenton, Florida 34203

Prepared by:

Environmental Engineering Consultants, Inc.

5119 N. Florida Avenue

Tampa, Florida 33603

EEC # 2014035

May 21, 2014

Table of Contents

I.	Overview							Page 3

II.	B-Series Industravac						Page 4

III,	Site Specific Operation and Maintenance Plan		Page 5

Appendix A - Industravac Dimensional Data			Page 6

Appendix B – Daily Operational Checklist			Page 8

Appendix C – Monthly Operational Checklist		Page 9

Appendix D – Generic Operation and Maintenance Plan 	Page 10

from Manufacturer

- Overview

General Information:

Pro-Mac Machines, Inc. will be authorized to operate a beryllium machine shop according to their up-coming air construction permit. They produce high tolerance components manufactured from beryllium and other specialty metals. Equipment used in beryllium machining is a SHARP Model 4328SX, Vertical Milling Machine, as backup contingency vertical milling machines are the Cincinnati Milacron Arrow 500 and the Cincinnati Milacron Arrow 750 and the only lathe to be used is the Cincinnati Milacron Falcon 300.

Dust and chips generated by the SHARP Model 4328SX, Vertical Milling Machine or by one of the other 3 machines are collected by vacuum pickup lines at each tool point, only one machine will operate at a time. Beryllium emissions generated from the machining will be controlled by an emission control system which consists of a Spencer Industravac Model SB-515 vacuum type dust collector with baghouse, cyclone and an inline HEPA filter (99.9% design efficiency) and exhausted through the vent.

II.	B-Series Industravac

General Information:

The B-Series Industravac is manufactured by The Spencer Turbine Company and was installed by Machine Tool Finance. The Spencer Industravac Model SB-515 vacuum type dust collector consists of baghouse, cyclone, and inline HEPA filter (99.9% design efficiency). Please see Figure 1 in Appendix A for the location of the control devices. The Spencer Industravac operates at a design air flow rate of 600 SCFM.

Specific Information:

Vacuum Producer:		3500 RPM

5, 6, or 7 stages

6” dia. Outlet

Standard overhung construction

Motor:				Open drip-proof 1.15 service factor

230 volt 3 phase 60 hertz

3500 RPM, 15 Horsepower

Separator:			75 sq. ft. Filter bag area

Cotton sateen filter bag material

29 filter bags

Filter bag catalog number 4380-N

Manual bag shaker

7.5” cu. Ft. capacity removable dirt can

Available Options:		Cotton Twill

Equalizing line with one liner dirt can

Discharge silencer

Discharge throttling valve

Materials of Construction:	Casing ASTM A569 hot rolled steel

Impellers: 5052-H34 Aluminum

Shaft: AISI 1045 hot rolled bar steel

Finish: Semi-gloss enamel over red oxide primer

Separator:			Shell and dirt can: ASTM A569 hot rolled steel

Intake section lining: Abrasive resistant steel

Finish: Semi-gloss enamel over red oxide

II.	B-Series Industravac continued:

General Information continued:

Performance:			Average sound pressure level: 80 dBA at 3 feet

Maximum vibration: 1.5 mils peak to peak

Average filtration efficiency: 99.9% at 3 Microns

III. 	Site Specific Operation and Maintenance Plan

The site specific maintenance and inspections are broken down into daily and monthly requirements listed below.  The overall or generic Operation and Maintenance Plan is found in Appendix D – Generic Operation and Maintenance Plan and is from page 7 of the Factory Manual.  Source is this link; http://www.spencerturbine.com/files/UU4.1.pdf.

Note: These requirements are for only the days and months when Beryllium will be or has been machined.

Daily:		Upon startup the outlets of the vacuum system shall be checked for

vacuum flow.

All vacuum lines that are not in use shall be plugged.

The baghouse and HEPA filter shall be checked daily upon startup by

looking through the observation window.  Please see Appendix A for

Sketch and Appendix B for Daily Operational Checklist.

Monthly	Checks shall be performed by opening the observation door on the

baghouse (Appendix A) and visually inspecting the bag collection system.

Visually inspect the collection can to determine the remaining capacity.  Record these observations in Appendix C – Emission Control System Operation Monthly Checklist.

Appendix A

Industravac Dimensional Data

Appendix B

Daily Operational Checklist

Example

| Spencer Vacuum Daily Check List |  |  |  |
| --- | --- | --- | --- |
| Date | Time | Viewer | Site Glass Viewing (Description) |

Appendix C

Monthly Operational Checklist

Monthly Operational Checklist

1. Open observation door on baghouse and check condition of bags		_______

2. Check remaining capacity of dirt collection can				_______

3. Perform all daily operational checks						_______

______________________					______________________

Date								Signature

Appendix D

Generic Operation and Maintenance Plan from Manufacturer