##### PERMITTEE

Solid Waste Authority of Palm Beach County

7501 North Jog Road

West Palm Beach, Florida 33412

##### Permitting Authority

Florida Department of Environmental Protection

Division of Air Resource Management

Office of Permitting and Compliance

2600 Blairstone Road

Tallahassee, Florida 32399

##### PROJECT

Air Permit No. 0990234-032-AC

Minor Source Air Construction Permit

Palm Beach Renewable Energy Facility No. 2

The Solid Waste Authority (SWA) of Palm Beach County (PBC) proposed to expand the authorized slate of fuels at the new Palm Beach Renewable Energy Facility No. 2 (PBCREF No. 2) and clarify the emissions testing procedures and requirements for this facility.

##### NOTICE AND PUBLICATION

The Department distributed a draft minor air construction permit package on November 13, 2014.  The applicant published the Public Notice in the Palm Beach Post on November 18, 2014.  The Department received the proof of publication on November 19, 2014.  No requests for administrative hearings or requests for extensions of time to file a petition for administrative hearing were received.

# COMMENTS

Comments from the Public

- Commenter No. 1:  On November 29, 2014, the Department received comments from Drew Martin, the Conservation Chair of the Loxahatchee Group of the Sierra Club.  The following summarizes the comments and the Department’s response.

  - The commenter is concerned about possible increases in air pollution resulting from this permit.  Toxic emissions from items such as tires are a particular concern.

Response:  Originally authorized in 2010, the expansion is nearing completion.  Upon startup, the new municipal waste combustors (MWC) will operate in accordance with the emissions standards, control equipment, and monitoring requirements that represent the best available control technology (BACT) and are the most stringent to-date in the U.S.  The acceptance of different categories of non-hazardous wastes will not cause the facility to emit in excess of these stringent standards.

In 2012, the legislature passed House Bill (HB) 503 that amended Section 403.707(1), Florida Statutes (F.S.), which now states:

“For the purpose of permitting under this chapter, the department shall allow waste-to-energy (WTE) facilities to maximize acceptance and processing of nonhazardous solid and liquid waste”.

Tires have excellent energy content and are squarely within the types of waste (or fuels) for which the statute was framed.  Tires will be thoroughly combusted and the units will be subject to the same stringent standards for carbon monoxide (CO), sulfur dioxide (SO2), nitrogen oxides (NOX), particulate matter (PM/PM10/PM2.5), and hazardous air pollutant (HAP) that apply when burning conventional municipal solid waste (MSW).  The Department has already permitted similar fuel/waste diversification projects at several WTE facilities and has received no indication of substantial changes in emissions or exceedances of permitted emission limits.

It is important to note that this permit authorizes a greater proportion of non-MSW wastes combusted at the facility in segregated loads.  However, the permit does not authorize an increase in the permitted capacity of the facility or the amount of waste that may be combusted.  In addition, no pollutant emission limits in previous air construction permits were changed as a result of this permitting action.  The change in the permitted mix of wastes at the facility is not expected to appreciably affect the emissions from the facility or the facility’s ability to comply with all applicable emissions standards.

Extensive pollution monitoring requirements are also required by the facility’s original air construction permit.  Emissions of CO, NOX, and SO2 from each MWC unit and mercury from one of the MWC unit will be monitored and recorded by continuous emissions monitoring systems (CEMS).  Visible emissions (opacity) from each unit will be monitored and recorded by a continuous opacity monitoring system (COMS).  The facility will have to demonstrate compliance with all emissions standards once it begins operation, and it will have to demonstrate subsequent compliance through regular emissions testing or continuous monitoring.

  - The commenter notes that an increase in tonnage into the plant may result in increased truck traffic and associated air emissions.

Response:  Again, it is important to note that the permit revision does not authorize an increase in the original permitted capacity of the facility or the amount of waste that may be combusted.

  - The commenter is concerned about the burning of recyclable materials and about the destruction of expired products, which may still be useful but for artificially short sell-by dates.

Response:  These issues are outside the scope of the Department’s authority in this air permitting matter.

  - The commenter requests to know the level of mercury, arsenic or other toxic materials being released into the air and how these increases will impact these pollutants and the amounts released.

Response:  Mercury and other HAP emissions, controls and monitoring requirements are detailed in the Department’s original Technical Evaluation and Preliminary Determination dated November 15, 2010:  Link to Technical Evaluation.  Hazardous air pollutants are regulated in accordance with the New Source Performance Standards specified in Title 40 of the Code of Federal Regulations, Part 60, Subpart Eb for Large Municipal Waste Combustors.  This regulation establishes emissions standards, operating limits and monitoring requirements to control HAP emissions.  Specifically, mercury and HAP emissions are controlled by activated carbon injection, lime scrubbing, selective catalytic reduction and the collection of the resulting fly ash particles by fabric filters.  The expanded fuel slate will not meaningfully change the expected emissions of these well-controlled pollutants.

No changes were made to the permit as a result of these comments.

- Commenter Nos. 2 and 3:  On December 1, 2014, the Department received comments from Dinah Garson and Charles Brown, both of whom live within a mile of the facility.  The two letters raised the same concerns.  The following summarizes the comments and the Department’s response.

  - The commenters state that when they moved into their neighborhood (e.g. Ms. Garson in 2009), they were promised that the Palm Beach Renewable Energy Park would cease operations in the near future.

Response:  This issue is outside the scope of the Department’s authority in this air permitting matter.

  - The commenters note that odor and “noxious fumes” are evident when driving past the existing Palm Beach Renewable Energy Park on the nearby Florida’s Turnpike, as well as in their neighborhood.

Response:  With regard to this permitting action, it is unlikely that the odor complaints are related to the new PBCREF No. 2, which has yet to start combusting MSW.  In general, odors are controlled in MWC units by maintaining significant negative pressure on the tipping floor where the MSW is deposited before it is fed into the waste combustors.  Consequently, there is always a strong inward draft across the tipping floor with the draft providing the combustion air for the MWC, which destroys the odors through combustion.  Once the new PBCREF No. 2 becomes fully operational (spring of 2015), all the combustible MSW received by the SWA will be processed through the PBCREF Nos. 1 and 2.  This should significantly reduce the potential for odor generation from landfilling unprocessed MSW.

To ensure odor complaints are timely identified and resolved, individuals should report the complaints to the Palm Beach County Health Department at 561-355-3136. Complaints specific to the Solid Waste Authority facilities should be directed to the Department’s Southeast District Office at 561-681-6600.  The Department’s Southeast District Office is actively engaging the Solid Waste Authority for a solution to resolve odor issues related to the associated landfill.  To address odor issues, the Solid Waste Authority of Palm Beach County has agreed to implement and/or investigate the following measures.

- Install pin wells and horizontal collectors to capture fugitive landfill gas close to the surface and direct it a control device such as a flare to control odor.

- Utilize alternative landfill cover materials to minimize odors emanating from the landfill.  Some landfill cover may have contained wallboard, which contains sulfur compounds that can generate H2S via anaerobic digestion of these materials.

- Evaluate the installation of a permanent impervious cap on additional areas of the landfill to control odors.

- At the woody waste composting facility, divert all woody waste to the new PBCREF No. 2 beginning in the first quarter of 2015.  This will effectively eliminate woody waste processing activities as a source of potential odors.  In the meantime: 1) reduce the height and age of piles or windrows of woody waste; 2) introduce oxygen by turning piles to minimize anaerobic processes thereby minimizing odors; and, 3) allow no more than woody waste material 1,500 tons of material to accumulate before size reduction takes place.

- At the woody waste composting facility, excavate and backfill bio-filter beds.

- At the bio-solids facility, minimize the amount of time the doors are open for the receipt of wastewater residuals (bio-solids).  Also minimize the waiting time of the delivery trucks queuing up to deliver wastewater residuals to the facility.

- Use odor control misters as needed at lift stations and open manholes in the leachate collection system.  Also, minimize open manholes during maintenance activities on the leachate collection system.

  - The commenters state that the community has made known its opposition to the SWA’s desire to bring in waste from outside the county.  The commenters also state that the SWA has not been transparent in its plans to seek wastes from a wider area.

Response:  This issue is outside the scope of the Department’s authority in this air permitting matter.

  - The commenters request that the PBCREF No. 2 not be permitted to bring in waste from outside of the county.  The commenters ask the DEP meet with and try to accommodate the wishes of the residents.

Response:  Again, the issue of bringing in waste from outside the county is outside the scope of the Department’s authority in this air permitting matter.

No changes were made to the permit as a result of these comments.

Comments from the Applicant

On November 25, 2014, the Department received comments from the applicant.  The following summarizes the comments and the Department’s response.

- The applicant recommended minor edits to provide clarity to the draft permit conditions.

- Page 3 of 28, first sentence above table: Change PBREF-1 to PBREP and delete “facility”.

- Page 8 of 28, Section 3.A.2.d, the heading:  Change “Activate” to “Activated” [CI System and FF Baghouse].

- Page 8 of 28, Section 3.A.3 (Aqueous Ammonia or Urea Storage Tank), third sentence: Revise to state:  The tank “shall be” designed and fabricated.

- Page 18 of 28, Section 3.A.35.d.: Change the first sentence to state that, “Each day the weight of each load of segregated non-MSW material …” Change the second sentence to state, “At the end of each calendar month, the resultant monthly total weight of each type of segregated load of non-MSW material.”

Response:  These recommendations to clarify and correct the conditions are reasonable and will be made in the final permit.

# CONCLUSION

The final action is to issue the permit with the minor changes, corrections and clarifications as described above.