TECHNICAL EVALUATION

&

PRELIMINARY DETERMINATION

APPLICANT

Southeastern Construction and Maintenance, Inc.

1150 Pebbledale Road

Mulberry, FL  33860

Facility ID No. 1050451

PROJECT

Project No. 1050451-001-AC

Application for Minor Air Construction Permit

Project Name: After-the-Fact Initial Construction Permit

COUNTY

Polk County, Florida

PERMITTING AUTHORITY

Florida Department of Environmental Protection

Air Resource Management

Southwest District Office

Temple Terrace, Florida 33637-0926

April 22, 2014

Prepared by Nancy E. Knight

General Project INFORMATION

Air Pollution Regulations

Projects at stationary sources with the potential to emit air pollution are subject to the applicable environmental laws specified in Section 403 of the Florida Statutes (F.S.).  The statutes authorize the Department of Environmental Protection (Department) to establish regulations regarding air quality as part of the Florida Administrative Code (F.A.C.), which includes the following applicable chapters:  62-4 (Permits); 62-204 (Air Pollution Control – General Provisions); 62-210 (Stationary Sources – General Requirements); 62-212 (Stationary Sources – Preconstruction Review); 62-213 (Operation Permits for Major Sources of Air Pollution); 62-296 (Stationary Sources - Emission Standards); and 62-297 (Stationary Sources – Emissions Monitoring).  Specifically, air construction permits are required pursuant to Rules 62-4, 62-210 and 62-212, F.A.C.

In addition, the U. S. Environmental Protection Agency (EPA) establishes air quality regulations in Title 40 of the Code of Federal Regulations (CFR).  Part 60 specifies New Source Performance Standards (NSPS) for numerous industrial categories.  Part 61 specifies National Emission Standards for Hazardous Air Pollutants (NESHAP) based on specific pollutants.  Part 63 specifies NESHAP based on the Maximum Achievable Control Technology (MACT) for numerous industrial categories.  The Department adopts these federal regulations on a quarterly basis in Rule 62-204.800, F.A.C.

Glossary of Common Terms

Because of the technical nature of the project, the permit contains numerous acronyms and abbreviations, which are defined in Appendix A of this permit.

- Facility Description:

This facility designs, fabricates, paints, assembles/constructs, repairs and maintains equipment, 	components and structures made from carbon steel.

II.  	Project Description:

A.	Applicant:

Steven Howell, Sr. Vice President

Southeastern Construction and Maintenance, Inc.

1150 Pebbledale Road

Mulberry, FL  33860

B.	Professional Engineer:

Dale C. Konas

EnviroTrac Ltd., Inc.

5309 56th Commerce Park Blvd.

Tampa, FL  33610

C.	Project Location:

1150 Pebbledale Road

Mulberry, FL  33860

D. 	Project Summary:

This is an after-the-fact initial construction permit for a facility  that designs, fabricates, paints, assembles/constructs, repairs and maintains equipment, components and structures made from carbon steel, as further described in the permit.

E.	Application Information:

Application Received on:   4/17/2014

Application Complete on:  4/17/2014

III.	Applicable State and Federal Regulations

This project is subject to the preconstruction review requirements of Chapter 403, Florida Statutes and Chapters 62-204 through 62-297, Florida Administrative Code (F.A.C.), as indicated below.

| Subject to: | Y/N | Comments |
| --- | --- | --- |
| Rule 62-210.300, F.A.C. – Permits Required | Y | subject to general permitting requirements |
| Rule 62-212.400, F.A.C. - Prevention of Significant Deterioration | N | facility is not a PSD major source |
| Rule 62-296.320(4), F.A.C. - General Particulate Emission Limiting Standards | Y | painting operations could be a source of unconfined particulate matter emissions |
| Rules 62-296.320(1) and (2), F.A.C. - General Pollutant Emission Limiting Standards (VOCs and Odor) | Y | painting operations are a source of VOC and odors |
| Rule 62-296.401-418, F.A.C. - Stationary Source Emission Standards | N | there is no applicable source |
| Rule 62-296.500, F.A.C. - Reasonably Available Control Technology (VOC) | N | Polk County is not an air quality maintenance area for ozone |
| Rule 62-296.700, F.A.C. - Reasonably Available Control Technology (PM) | N | there is no applicable source category |
| Rule 62-204.800, F.A.C. - Standards of Performance for New Stationary Sources (NSPS) | N | there is no applicable source category |
| Rule 62-204.800, F.A.C. National Emission Standard for Hazardous Air Pollutants (NESHAPS – 40 CFR 61) | N | there is no applicable source category |
| Rule 62-204.800, F.A.C. National Emission Standard for Hazardous Air Pollutants for Source Categories a.k.a. MACT (NESHAPS –40 CFR 63) | N | there is no applicable source category |
| Chapter 62-213, F.A.C. - Operation Permits for Major Sources of Air Pollution | N | facility is a synthetic non-Title V source |
| Rule 62-297.310, F.A.C. - General Compliance Test Requirements, F.A.C. | N | Special Compliance VE testing may be required by the Department for EU No. 002. |

IV.	Summary of Emissions

| Pollutant | EU No. and brief description | Potential Emissions (TPY) | Allowable Emissions (TPY) |
| --- | --- | --- | --- |
| PM | 001- Open Exterior Sand Blasting | 23.11 |  |
|  | 002 – Metal Shot Blasting Area | 0.0053 |  |
|  | Total | 23.1 |  |
| PM10 | 001- Open Exterior Sand Blasting | 11.12 |  |
|  | 002 – Metal Shot Blasting Area | 0.0034 |  |
|  | Total | 11.1 |  |
| VOC | 003 – Surface Coating Operations | 43.0 | 43.0 |
|  | Total | 43.0 | 43.0 |
| Total HAP | 003 – Surface Coating Operations | 24.0 | 24.0 |
|  | Total | 24.0 | 24.0 |
| Individual HAP (xylene) | 003 – Surface Coating Operations | 8.0 | 8.0 |
|  | Total | 8.0 | 8.0 |
| Individual HAP (phenols) | 003 – Surface Coating Operations | 8.0 | 8.0 |
|  | Total | 8.0 | 8.0 |
| VE Limitation |  |  |  |
| VE | 002 – Metal Shot Blasting Area |  | - 20% - Rule 62-296.320(4)(b), F.A.C. - 5% - As an indicator of proper dust collector operation and maintenance. |

1 PM emissions determined using the permitted throughput rate of 854.32 tons per year of sand and an emission factor from Table 13.2.6-1 of AP-42 of 54 pounds of PM per ton of sand (wind speed of 5 mph).

2 PM10 emissions determined by using the permitted throughput rate of 854.32 tons per year of sand and an emission factor from Table 13.2.6-1 of AP-42 of 26 pounds of PM10 per ton of sand.

3 PM emissions determined using the permitted throughput rate of 96.94 tons per year of steel shot and an emission factor from Paragraph 13.2.6.3 of AP-42 that states that PM emissions from abrasive blasting using shot are ~10% of total PM emissions from abrasive blasting with sand (5.4 pounds of PM per ton of shot (wind speed of 5 mph) and a control efficiency of 98% for the Torit cartridge dust collector.

4 PM10 emissions determined using the permitted throughput rate of 96.94 tons per year of steel shot and an emission factor from Paragraph 13.2.6.3 of AP-42 that states that PM emissions from abrasive blasting using shot are ~10% of total PM emissions from abrasive blasting with sand (5.4 pounds of PM per ton of shot (wind speed of 5 mph) and a control efficiency of 98% for the Torit cartridge dust collector.

V.	Conclusions

The emission limits proposed by the applicant will meet all of the requirements of Chapters 62-204 through 297, F.A.C.

The General and Specific Conditions listed in the proposed permit (attached) will assure compliance with all the applicable requirements of Chapters 62-204 through 297, F.A.C.

VI.	Preliminary Determination

The Department makes a preliminary determination that the proposed project will comply with all applicable state and federal air pollution regulations as conditioned by the draft permit.  This determination is based on a technical review of the complete application, reasonable assurances provided by the applicant, and the conditions specified in the draft permit.  Additional details of this analysis may be obtained by contacting the project engineer at the Florida Department of Environmental Protection Air Resource Management, Southwest District 13051 North Telecom Parkway, Temple Terrace, Florida 33637-7600.

Pursuant to Section 403.087, Florida Statutes and Section 62-4.070, Florida Administrative Code, the Department hereby gives notice of its intent to issue a permit to construct the aforementioned air pollution source in accordance with the draft permit and its conditions as stipulated (see attached).