### TECHNICAL EVALUATION

#### APPLICANT

Madison County Public Works Department

Post Office Box 237

##### Madison, Florida 32341

Facility ID No. 7775321

#### PROJECT

# Project No. 7775321-003-AC/MM

Non-Title V Air Construction Permit

#### COUNTY

Madison

PERMITTING AUTHORITY

Florida Department of Environmental Protection

Northeast District Office

Waste and Air Resource Management

8800 Baymeadows Way West, Suite 100

Jacksonville, Florida  32256

COMPLIANCE AUTHORITY

Florida Department of Environmental Protection

Northeast District Office

Compliance Assurance

8800 Baymeadows Way West, Suite 100

Jacksonville, Florida  32256

October 2, 2014

1.  General Project INFORMATION

Air Pollution Regulations

Projects at stationary sources with the potential to emit air pollution are subject to the applicable environmental laws specified in Section 403 of the Florida Statutes (F.S.).  The statutes authorize the Department of Environmental Protection (Department) to establish regulations regarding air quality as part of the Florida Administrative Code (F.A.C.), which includes the following applicable chapters:  62-4 (Permits); 62-204 (Air Pollution Control – General Provisions); 62-210 (Stationary Sources – General Requirements); 62-212 (Stationary Sources – Preconstruction Review); 62-213 (Operation Permits for Major Sources of Air Pollution); 62-296 (Stationary Sources - Emission Standards); and 62-297 (Stationary Sources – Emissions Monitoring).  Specifically, air construction permits are required pursuant to Rules 62-4, 62-210 and 62-212, F.A.C.

In addition, the U. S. Environmental Protection Agency (EPA) establishes air quality regulations in Title 40 of the Code of Federal Regulations (CFR).  Part 60 specifies New Source Performance Standards (NSPS) for numerous industrial categories.  Part 61 specifies National Emission Standards for Hazardous Air Pollutants (NESHAP) based on specific pollutants.  Part 63 specifies NESHAP based on the Maximum Achievable Control Technology (MACT) for numerous industrial categories.  The Department adopts these federal regulations on a quarterly basis in Rule 62-204.800, F.A.C.

Glossary of Common Terms

Because of the technical nature of the project, the permit contains numerous acronyms and abbreviations, which are defined in Appendix A of this permit.

Facility Process Description and Location

The existing Madison County Public Works Department (WACS No.101815) collects and stores wood wastes consisting of trees, logs, large brush, stumps relatively free of soil, unbagged leaves, tree surgeon debris, and clean dry lumber such as pallets generated by the public and collected from solid waste/recycling center throughout Madison County. .

Madison County Public Works Department (WACS No.101815) is located in Madison County at the end of SW Industrial Ext, Madison County, Florida

Facility Regulatory Categories

- The facility is a not a major source of hazardous air pollutants (HAP).

- The facility does not operate units subject to the acid rain provisions of the Clean Air Act.

- The facility is not a Title V major source of air pollution in accordance with Chapter 213, F.A.C.

- The facility is not a major stationary source in accordance with Rule 62-212.400, F.A.C. for the Prevention of Significant Deterioration (PSD) of Air Quality.

Processing Schedule

9/23/14	Department received an application for construction of a relocatable air curtain incinerator

Project Description

The applicant has proposed to construct an air curtain incinerator (Manufacturer: New Concept Products, Model: CP 2400 HD) (Standard Industrial Classification No. 4953) for the burning of 100 percent wood wastes consisting of trees, logs, large brush, stumps relatively free of soil, unbagged leaves, tree surgeon debris, and clean dry lumber such as pallets generated by the public and collected from solid waste/recycling center throughout Madison County.

The proposed operation will be conducted by the City of Madison Public Works Storage Area (WACS No.101815). The relocatable air curtain incinerator will be operated at various locations within Madison County, Florida including the following three principal locations:

Site 1	Rocky Ford Burn Site located at 2505 NE Rocky Ford Road, Madison County, Florida. Latitude/longitude coordinates are 30º30’18”N; 83º23’52”W.

Site 2	West Farm Burn Site located at corner of Old County Camp Road and SE Ash Way, Madison, County, Florida. Latitude/longitude coordinates are 30º27’19”N; 83º23’40”W.

Site 3	City of Madison Site located at the end of SW Industrial Ext, Madison County, Florida.

Latitude/longitude coordinates are 30º26’18”N; 83º24’41”W.

Permitting History:

On August 4, 2006 the Department issued Air Construction Permit No. 7775321-001-AC to Madison County Publics Works Department for the construction of a relocatable air curtain incinerator.

On June 27, 2007, Madison County Department of Public Works requested its first time extension of the expiration date of air construction Permit No. 7775321-001-AC to allow additional time for the county to procure the equipment, complete construction, test the equipment and apply for an air operation permit. The Department approved the request and the expiration date of the permit was extended from August 4, 2007 to August 4, 2008.

On June 7, 2008, October 2, 2009 and October 29, 2013 the Department approved the requests for time extensions and granted time extensions to the facility for the completion of the project.

Considering the time laps between the initial permit issuance and several changes and amendments of the rules that regulate air curtain incinerators, a determination was made to modify Construction Permit No. 7775321-001-AC in order to incorporate the rule changes and additional facility requested operating locations.

Potential to Emit: Emissions factors from Los Alamos National Laboratories and AP-42 are used to estimate the potential emissions of both the wastes burning and engine operation. Table below shows the emissions factors used and the potential emissions of the unit.

| Pollutant | Wood Burning Emissions Factor (lb/ton of wood waste) | PTE of Wood Waste Burning (Ton/year) | Engine Operation Emissions factor (lb/hp-hr) | PTE of Engine Operation (Ton/year) | Total Emissions (Ton/year) |
| --- | --- | --- | --- | --- | --- |
| PM/PM10 | 2.0 | 24.0 | 0.0022 | 0.2 | 24.2 |
| SO2 | 0.1 | 1.2 | 0.0021 | 0.2 | 1.4 |
| NOx | 2.0 | 24.0 | 0.031 | 2.3 | 26.3 |
| CO | 1.4 | 16.8 | 0.0067 | 0.5 | 17.3 |
| VOC | 3.8 | 45.6 | 0.0025 | 0.2 | 45.8 |
| HAPs | 0.4 | 4.2 | 0.000046 | 0.0 | 4.2 |

PTE estimations are based on maximum operational hours of 3,000 hours/year and maximum incineration rate of 8 tons/hour.

3.   Application Review

The applicant has not proposed any changes to the equipment or operation of the facility nor to the conditions of their current operating permit. Additionally, there have been no changes to the rules applicable to this facility.

State Requirements

- Chapter 62-256, F.A.C		Open Burning

- Chapter 62-296, F.A.C		Stationary Sources – Emissions Standards

- Rule 62-296.320, F.A.C		General Pollutant Emissions Limiting Standards

- Rule 62-296.401(7), F.A.C	Air Curtain Incinerator

- Chapter 62-297, F.A.C		Stationary Sources – Emissions Monitoring

- Rule 62-297.310, F.A.C		General Test Requirements

- Rule 62-297.320, F.A.C		Standards for Persons Engaged in Visible Emissions Observations.

Federal NSPS Provisions

New Source Performances Standards (NSPS) 40 CFR 60 Subpart CCCC – Air Curtain Incinerator

The unit also has to meet the requirements of 40 CFR 60.2245 through 60.2260 (part of NSPS, Subpart CCCC - New Source Performance Standards for Commercial and Industrial Solid Waste Incineration (CISWI) Units.) for Air Curtain Incinerators.

Preliminary Determination

The Department makes a determination that the proposed project will comply with all applicable state and federal air pollution regulations as conditioned by the draft permit. This determination is based on a technical review of the complete application, reasonable assurances provided by the applicant, and the conditions specified in the draft permit. John Phillips is the project engineer responsible for reviewing the application and drafting the permit.  Additional details of this analysis may be obtained by contacting the project engineer at the Florida Department of Environmental Protection, Northeast District Office, 8800 Baymeadows Way, Suite 100, Jacksonville, FL 32256, Phone: 904/256-1700