TECHNICAL EVALUATION

&

PRELIMINARY DETERMINATION

APPLICANT

Lagan Construction, LLC

2500 Hunter Place, Suite 201

Woodbridge, Virginia 22192

NAS Whiting Field - South

Facility ID No. 7775859

PROJECT

Project No. 7775859-001-AC

Application for Minor Source Air Construction Permit

New Hot-Mix Asphalt Concrete Plant

COUNTY

Santa Rosa County, Florida

PERMITTING AUTHORITY

Florida Department of Environmental Protection

Northwest District Permitting Program

160 W. Government Street, Suite 308

Pensacola, Florida 32502-5740

July 16, 2015

- General Project INFORMATION

Air Pollution Regulations

Projects at stationary sources with the potential to emit air pollution are subject to the applicable environmental laws specified in Section 403 of the Florida Statutes (F.S.).  The statutes authorize the Department of Environmental Protection (Department) to establish regulations regarding air quality as part of the Florida Administrative Code (F.A.C.), which includes the following applicable chapters:  62-4 (Permits); 62-204 (Air Pollution Control – General Provisions); 62-210 (Stationary Sources – General Requirements); 62-212 (Stationary Sources – Preconstruction Review); 62-213 (Operation Permits for Major Sources of Air Pollution); 62-296 (Stationary Sources - Emission Standards); and 62-297 (Stationary Sources – Emissions Monitoring).  Specifically, air construction permits are required pursuant to Chapters 62-4, 62-210 and 62-212, F.A.C.

In addition, the U. S. Environmental Protection Agency (EPA) establishes air quality regulations in Title 40 of the Code of Federal Regulations (CFR).  Part 60 specifies New Source Performance Standards (NSPS) for numerous industrial categories.  Part 61 specifies National Emission Standards for Hazardous Air Pollutants (NESHAP) based on specific pollutants.  Part 63 specifies NESHAP based on the Maximum Achievable Control Technology (MACT) for numerous industrial categories.  The Department adopts these federal regulations in Rule 62-204.800, F.A.C.

Glossary of Common Terms

Because of the technical nature of the project, the permit contains numerous acronyms and abbreviations, which are defined in Appendix A of this permit.

Facility Description and Location

Lagan Construction, LLC (Lagan) proposes to construct a new asphalt concrete plant, a synthetic minor facility, which is categorized under Standard Industrial Classification Code No. 2951, Asphalt Paving Mixtures and Blocks.  The new facility will be located in Santa Rosa County at NAS Whiting Field – South, on SR 87A, about 6 miles north of Milton, Florida.  The UTM coordinates of the new facility are Zone 16, 499.63 km East, and 3396.34 km North.  This site is in an area that is in attainment (or designated as unclassifiable) for all air pollutants subject to Ambient Air Quality Standards (AAQS).

Facility Regulatory Categories

- The facility is not a major source of hazardous air pollutants (HAP).

- The facility is not a Title V major source of air pollution in accordance with Chapter 62-213, F.A.C.

- The facility is not a major stationary source in accordance with Rule 62-212.400, F.A.C. for the Prevention of Significant Deterioration (PSD) of Air Quality.

- Project Description

Lagan proposes to install and operate a portable asphalt plant for repaving two runways at NAS Whiting Field.  The proposed asphalt plant will be an Astec portable unified counterflow drum mix hot-mix asphalt concrete plant, Model No. D-PVUCF-8446. The plant will contain an aggregate feed system, a drum dryer, an asphalt cement storage tank, and an asphalt product surge bin.  The drum/dryer oil burner, Model No. WhisperJet WJ-75-O, will have a maximum firing rate of 75 MM Btu/hr at the maximum plant operating rate of 250 tons of product produced per hour.  Particulate matter emissions from the hot-mix asphalt concrete plant will be controlled by a portable Astec reverse pulse baghouse, Model No. PEBH-43, with 960 aramid felt bags.  The air flow through the baghouse will be 43,000 cubic feet per minute, the total cloth area will be 7,903 ft2, and the air-to-cloth ratio will be approximately 5.4:1.

The maximum recycled asphalt pavement (RAP) usage rate will be 80% for the product mix.  The fines from the baghouse will be recycled into the asphalt mix.  Fugitive emissions at the facility will result from uncontrolled process emissions, aggregate stockpiles and vehicular traffic.  Front end loaders will be used to transfer aggregate from storage piles to the aggregate feed system.  Aggregate will be proportioned by calibrated feeders onto a common collecting belt, and then conveyed to the drying drum.  After the aggregate is thoroughly dried, it will be combined with RAP and baghouse fines, preheated to drive off moisture, and then mixed with hot liquid asphalt in the mixing drum to create the asphalt concrete product.  The final product is conveyed by a fully enclosed conveyor to the product surge bin for discharge into trucks.

Lagan will maintain at least one water truck (two thousand gallon bowser/sprayer) at the staging area to control unconfined particulate matter during the stockpiling and loading operations.  If conditions require, .additional water bowers/sprayers will be utilized.

Electric power for the plant will be provided by two portable diesel powered generators.  By applicant’s request the facility will burn diesel fuel oil or No.2 fuel oil with a sulfur content of 0.5 % or less, by weight.

Proposed repairs at NAS Whiting field include; repairs of approximately 300,000 square yards of asphalt pavement and shoulder surfaces, and various drainage ditches at Runways 5/23 and 14/32.  Repairs also include sweeping and preparing pavement surfaces, milling cold in-place asphalt or comparable process, variable depth overlay and airfield markings, saw cutting, barricades, construction fencing, surveying, repair of existing airfield shoulder surfaces and construction of new shoulder surfaces, grading/earthwork/compaction and sodding at runway and shoulder edges, and drainage ditch repair and/or reconstruction.  New airfield markings (striping and numbers) will be applied in accordance with current regulations.  Lagan is projecting the following quantities and operating schedule for the project.  There are three types of asphaltic material which will be produced in the plant:

- 32,000 tons of Cold-Mix Recycled Material @ 100 tons / Hour (80% RAP + 20% virgin aggregates)

- 45,000 tons of Hot-Mix Asphalt, Regulating Layer @ 140 tons / Hour (30% RAP + 70% virgin aggregates)

- 33,000 tons of Hot-Mix Asphalt, Surface Layer @ 160 tons / Hour (100% virgin aggregates)

In total Lagan is projecting 110,000 tons of asphaltic material to be produced through the plant and estimate this will require 850 hours of production.  Therefore, the average production rate will be approximately 130 tons/hour for 850 hours.  However, to be conservative, Lagan is basing the emissions estimates on 300,000 tons of asphaltic material to be produced in approximately 1160 hours of production.  Lagan will be using two generators to power the plant.  During “production mode”, the plant will be powered by a 783 hp (554 kW) portable diesel generator.  The plant is projected to operate in “production mode” for approximately 1160 hours during project duration.  Each day, after production is completed, Lagan will switch the plant from the 783 hp generator to “standby mode” and utilize a 384 hp portable diesel generator to maintain power to the burner and control house. The plant will operate on the 384 hp generator in “standby mode” for approximately 1800 hours during the project duration.  Additionally, Lagan intends to operate a crusher for processing RAP, only if required.  This crusher will be rented when necessary.  Please note that this permit application also includes only emissions from the engine associated with the crusher.

- Recycled asphalt pavement (RAP) will be generated by milling machines.  A relocatable RAP crushing system, operating under its own Air General Permit, may be brought on site periodically to provide RAP for the asphalt mix.

- The facility will be regulated by New Source Performance Standards 40 CFR part 60 Subpart I, Standards of Performance for Hot-Mix Asphalt Facilities, and Rule 62-210.300(3)(c)2., F.A.C., Conditional Exemptions from Title V Permitting.  The facility will be a synthetic minor source.

- Per applicant request, product throughput will be limited to 300,000 tons of asphalt product produced per year; total fuel oil consumption, including the asphalt plant, the portable RAP crusher engine, the two portable generator engines, and other stationary oil burning equipment, will be limited to 730,000 gallons per year; and the fuel oil sulfur content will be limited to 0.5% or less, by weight.  These limitations requested by the applicant are stricter than the limits required by Rule 62-210.300(3)(c)2., F.A.C.

- Processing Schedule

July 10, 2015	Received the application for a minor source air pollution construction permit.

PSD Applicability

General PSD Applicability

For areas currently in attainment with the AAQS or areas otherwise designated as unclassifiable, the Department regulates major stationary sources of air pollution in accordance with Florida’s PSD preconstruction review program as defined in Rule 62-212.400, F.A.C.  Under preconstruction review, the Department first must determine if a project is subject to the PSD requirements (“PSD applicability review”) and, if so, must conduct a PSD preconstruction review.  A PSD applicability review is required for projects at new and existing major stationary sources.  In addition, proposed projects at existing minor sources are subject to a PSD applicability review to determine whether potential emissions from the proposed project itself will exceed the PSD major stationary source thresholds.  A facility is considered a major stationary source with respect to PSD if it emits or has the potential to emit:

- 5 tons per year or more of lead;

- 250 tons per year or more of any regulated air pollutant; or

- 100 tons per year or more of any regulated air pollutant and the facility belongs to one of the following 28 PSD-major facility categories:  fossil fuel-fired steam electric plants of more than 250 million British thermal units per hour heat input, coal cleaning plants (with thermal dryers), Kraft pulp mills, Portland cement plants, primary zinc smelters, iron and steel mill plants, primary aluminum ore reduction plants, primary copper smelters, municipal incinerators capable of charging more than 250 tons of refuse per day, hydrofluoric, sulfuric, and nitric acid plants, petroleum refineries, lime plants, phosphate rock processing plants, coke oven batteries, sulfur recovery plants, carbon black plants (furnace process), primary lead smelters, fuel conversion plants, sintering plants, secondary metal production plants, chemical process plants, fossil fuel boilers (or combinations thereof) totaling more than 250 million British thermal units per hour heat input, petroleum storage and transfer units with a total storage capacity exceeding 300,000 barrels, taconite ore processing plants, glass fiber processing plants and charcoal production plants.

Once it is determined that a project is subject to PSD preconstruction review, the project emissions are compared to the “significant emission rates” defined in Rule 62-210.200, F.A.C. for the following pollutants:  carbon monoxide (CO); nitrogen oxides (NOX); sulfur dioxide (SO2); particulate matter (PM); particulate matter with a mean particle diameter of 10 microns or less (PM10); particulate matter with a mean particle diameter of 2.5 microns or less (PM2.5); volatile organic compounds (VOC); lead (Pb); fluorides (F); sulfuric acid mist (SAM); hydrogen sulfide (H2S); total reduced sulfur (TRS), including H2S; reduced sulfur compounds, including H2S; municipal waste combustor organics measured as total tetra- through octa-chlorinated dibenzo-p-dioxins and dibenzofurans; municipal waste combustor metals measured as particulate matter; municipal waste combustor acid gases measured as SO2 and hydrogen chloride (HCl); municipal solid waste landfills emissions measured as non-methane organic compounds (NMOC); and mercury (Hg).  In addition, significant emissions rate also means any emissions rate or any net emissions increase associated with a major stationary source or major modification which would construct within 10 kilometers of a Class I area and have an impact on such area equal to or greater than 1 μg/m3, 24-hour average.

If the potential emission equals or exceeds the defined significant emissions rate of a PSD pollutant, the project is considered “significant” for the pollutant and the applicant must employ the Best Available Control Technology (BACT) to minimize the emissions and evaluate the air quality impacts.  Although a facility or project may be major with respect to PSD for only one regulated pollutant, it may be required to install BACT controls for several “significant” regulated pollutants.

PSD Applicability for Project

As provided in the application, the following table summarizes potential emissions and PSD applicability for the project.

| Pollutant | Potential Emissions | Proposed Project Potential Emissions | PSD Major Threshold | Significant Emissions Rate | Subject To PSD? |
| --- | --- | --- | --- | --- | --- |
| CO | None - new | 24.9 | 250 | 100 | No |
| NOX | None - new | 16.3 | 250 | 40 | No |
| PMtotal/ PM10 | None - new | 7.9/4.1 | 250 | 25 | No |
| SO2 | None - new | 18.7 | 250 | 40 | No |
| VOC | None - new | 8.8 | 250 | 40 | No |
| HAP (Combined) |  | 1.4 |  |  |  |
| Formaldehyde |  | 0.13 |  |  |  |

Emission factors for PMtotal and PM10 are from AP-42, Table 11.1-3 (Baghouse with fabric filter).  Emission factors for NOx and CO are from AP-42, Table 11.1-7.  The emission factor for VOC is from AP-42 Table 11.1-8, and the emission factor for SO2 is from AP-42, Table 1.3-1.

This is a new facility.  The PSD major stationary source threshold for this type of facility is 250 TPY of a regulated pollutant.  As shown in the table above, the potential emissions from the proposed project do not equal or exceed the PSD major stationary source thresholds for this type of facility.  Based on these facts it is determined that the project is not subject to PSD preconstruction review

- APPLICATION review

The applicant proposes to install and operate a portable hot-mix asphalt concrete plant at NAS Whiting Field in Santa Rosa County for a planned runway paving project.

Application Fee

This is an AC/1E (two emissions units: 2 X $1,000 = $2,000.00 fee) for a source of emissions more than 5 but less than 25 tons per year.

Brief Discussion of Emissions

Potential emissions are calculated based on regulated activities associated with asphalt production, and include both ducted process emissions, generated during product manufacturing, and fugitive production-related emissions, resulting from the transport of asphalt concrete from drum mix to the surge bin and loading operations.  Fugitive emissions from yard storage piles and vehicular traffic are not included in the estimate of potential emissions.

For purposes of estimating emissions, Lagan is conservatively considering all asphalt produced to be hot-mix asphalt; although more than a quarter of the asphalt mix produced at the site will be emulsified asphalt (cold-mix recycled material) which emits only small amounts of volatile organic compounds.

The generator engines are being treated similarly to crusher engines, i.e., their emissions and fuel consumption are being considered as part of the whole asphalt plant facility.

State Requirements

The facility will be regulated by Rule 62-210.300(3)(c)2., F.A.C., Conditional Exemptions from Title V Permitting.

Federal NSPS Provisions

This emissions unit is subject to New Source Performance Standards 40 CFR part 60 Subpart I, Standards of Performance for Hot-Mix Asphalt Facilities.

NSPS subpart IIII is not applicable at this time.  Applicant has represented that the asphalt batch plant and its generators will not be at the same location for more than 12 consecutive months.  The portable electric generators’ diesel engines at this facility are non-road engines as defined in 40CFR §1068.30.  However, if an engine remains at the same location for more than 12 consecutive months, then it will be considered a stationary RICE, and may become subject to NSPS subpart IIII.

Federal NESHAP Provisions

Not applicable at this time.  Applicant has represented that the asphalt batch plant and its generators will not be at the same location for more than 12 consecutive months.  The portable electric generators’ diesel engines at this facility are non-road engines as defined in 40CFR §1068.30.  However, if an engine remains at the same location for more than 12 consecutive months, then it will be considered a stationary RICE, and may become subject to NESHAP subpart ZZZZ.

Other Draft Permit Requirements

There are no additional draft permit requirements.

- Preliminary Determination

The Department makes a preliminary determination that the proposed project will comply with all applicable state and federal air pollution regulations as conditioned by the draft permit.  This determination is based on a technical review of the complete application, reasonable assurances provided by the applicant, and the conditions specified in the draft permit.  No air quality modeling analysis is required because the project does not result in a significant increase in emissions.  Howard Ard is the project engineer responsible for reviewing the application and drafting the permit.  Additional details of this analysis may be obtained by contacting the project engineer at the Department’s Northwest District Office, 160 W. Government Street, Suite 308, Pensacola, Florida 32502-5740, phone 850-595-0622, or by e-mail howard.ard@dep.state.fl.us .