|  | Florida Department of Environmental Protection Northwest District 160 W. Government Street, Suite 308 Pensacola, Florida 32502-5740 | Rick Scott Governor Carlos Lopez-Cantera Lt. Governor Jonathan P. Steverson Interim Secretary |
| --- | --- | --- |

DRAFT

Permittee

| Lagan Construction, LLC 2500 Hunter Place, Suite 201 Woodbridge, Virginia 22192 Authorized Representative: Mr. Donald K. Bloodworth, Regional Manager | Air Permit No. 7775859-001-AC Permit Expires: (5 years after issuance) Minor Air Construction Permit NAS Whiting Field - South New Hot-Mix Asphalt Concrete Plant |
| --- | --- |

Project

This is the final air construction permit, which authorizes Lagan Construction to install a portable hot-mix asphalt concrete plant at NAS Whiting Field for a planned runway paving project.  The proposed work will create a new asphalt concrete manufacturing facility (Standard Industrial Classification No. 2951).  The new facility will be located in Santa Rosa County at NAS Whiting Field – South, on SR 87A, about 6 miles north of Milton, Florida.  The UTM coordinates of the new facility are Zone 16, 499.63 km East and 3396.34 km North.

This final permit is organized into the following sections:  Section 1 (General Information); Section 2 (Administrative Requirements); Section 3 (Emissions Unit Specific Conditions); and Section 4 (Appendices).  Because of the technical nature of the project, the permit contains numerous acronyms and abbreviations, which are defined in Appendix A of Section 4 of this permit.  As noted in the Final Determination provided with this final permit, only minor changes and clarifications were made to the draft permit.

Statement of Basis

This air pollution construction permit is issued under the provisions of:  Chapter 403 of the Florida Statutes (F.S.) and Chapters 62-4, 62-204, 62-210, 62-212, 62-296 and 62-297 of the Florida Administrative Code (F.A.C.).  The permittee is authorized to conduct the proposed work in accordance with the conditions of this permit.  This project is subject to the general preconstruction review requirements in Rule 62-212.300, F.A.C. and is not subject to the preconstruction review requirements for major stationary sources in Rule 62-212.400, F.A.C. for the Prevention of Significant Deterioration (PSD) of Air Quality.

Upon issuance of this final permit, any party to this order has the right to seek judicial review of it under Section 120.68 of the Florida Statutes by filing a notice of appeal under Rule 9.110 of the Florida Rules of Appellate Procedure with the clerk of the Department of Environmental Protection in the Office of General Counsel (Mail Station #35, 3900 Commonwealth Boulevard, Tallahassee, Florida, 32399-3000) and by filing a copy of the notice of appeal accompanied by the applicable filing fees with the appropriate District Court of Appeal.  The notice must be filed within 30 days after this order is filed with the clerk of the Department.

Executed in Pensacola, Florida

___________________________________		_______________

Elizabeth Mullins Orr

Permitting Program Administrator

CERTIFICATE OF SERVICE

The undersigned duly designated agency clerk hereby certifies that this Air Permit package was sent by electronic mail, or a link to these documents made available electronically on a publicly accessible server, with received receipt requested before the close of business on the date indicated below to the following persons.

# Donald K. Bloodworth, Regional Manager, Lagan Construction, donald.bloodworth@laganconstruction.com

# Graeme Clarke, Project Manager, Lagan Construction, graeme.clarke@laganconstruction.com

# Geoff Tizard, P.E., EA Engineering Science and Technology, Inc., PBC, gtizard@eaest.com

# Naveen Devata, EA Engineering Science and Technology, Inc., PBC, ndevata@eaest.com

# FILED, on this date, pursuant to Section 120.52(7), Florida Statutes, with the designated agency clerk, receipt of which is hereby acknowledged.

________________________________		______________

Clerk										Date

FACILITY AND PROJECT DESCRIPTION

Lagan proposes to install and operate a portable asphalt plant for repaving two runways at NAS Whiting Field.  The proposed asphalt plant will be an Astec portable unified counterflow drum mix hot-mix asphalt concrete plant, Model No. D-PVUCF-8446. The plant will contain an aggregate feed system, a drum dryer, an asphalt cement storage tank, and an asphalt product surge bin.  The drum/dryer oil burner, Model No. WhisperJet

WJ-75-O, will have a maximum firing rate of 75 MM Btu/hr at the maximum plant operating rate of 250 tons of product produced per hour.  Particulate matter emissions from the hot-mix asphalt concrete plant will be controlled by a portable Astec reverse pulse baghouse, Model No. PEBH-43, with 960 aramid felt bags.  The air flow through the baghouse will be 43,000 cubic feet per minute, the total cloth area will be 7,903 ft2, and the air-to-cloth ratio will be approximately 5.4:1.

The maximum recycled asphalt pavement (RAP) usage rate will be 80% for the product mix.  The fines from the baghouse will be recycled into the asphalt mix.  Fugitive emissions at the facility will result from uncontrolled process emissions, aggregate stockpiles and vehicular traffic.  Front end loaders will be used to transfer aggregate from storage piles to the aggregate feed system.  Aggregate will be proportioned by calibrated feeders onto a common collecting belt, and then conveyed to the drying drum.  After the aggregate is thoroughly dried, it will be combined with RAP and baghouse fines, preheated to drive off moisture, and then mixed with hot liquid asphalt in the mixing drum to create the asphalt concrete product.  The final product is conveyed by a fully enclosed conveyor to the product surge bin for discharge into trucks.

Electric power for the plant will be provided by two portable diesel powered generators.  Recycled asphalt pavement will be generated on-site by milling machines.  A relocatable RAP crushing system, operating under its own Air General Permit, may be brought on site periodically to provide RAP for the asphalt mix.

The facility will be regulated by New Source Performance Standards 40 CFR part 60 Subpart I, Standards of Performance for Hot-Mix Asphalt Facilities, and Rule 62-210.300(3)(c)2., F.A.C., Conditional Exemptions from Title V Permitting.  The facility will be a synthetic minor source.  By applicant’s request the product throughput will be limited to 300,000 tons of asphalt product produced per year; total fuel oil consumption, including the asphalt plant, the portable RAP crusher engine, the two portable generator engines, and other stationary oil burning equipment, will be limited to 730,000 gallons per year; and the fuel oil sulfur content will be limited to 0.5% or less, by weight.  These limitations are stricter than the limits required by Rule 62-210.300(3)(c)2., F.A.C.

This project will add the following emissions units.

| Facility ID No. 7775859 |  |
| --- | --- |
| ID No. | Emission Unit Description |  |
| 001 | Asphalt Concrete Plant with Baghouse |  |
| 002 | Two New Diesel Fueled Generator Engines (RICE) |  |
| 003 | Portable RAP Crushing System |  |

Facility Regulatory Classification

- The facility is not a major source of hazardous air pollutants (HAP).

- The facility does not operate units subject to the acid rain provisions of the Clean Air Act (CAA).

- The facility is not a Title V major source of air pollution in accordance with Chapter 62-213, F.A.C.

- The facility is not a major stationary source in accordance with Rule 62-212.400(PSD), F.A.C.

- Permitting Authority:  The permitting authority for this project is the Northwest District Permitting Program of the Department of Environmental Protection (Department).  The District’s mailing address is 160 W. Government Street, Suite 308, Pensacola, Florida 32502-5740.  All documents related to applications for permits to operate an emissions unit shall be submitted to the District Office at the above address or by electronic mail at epost_nwdwasteair@dep.state.fl.us.

- Compliance Authority:  All documents related to compliance activities such as reports, tests, and notifications shall be submitted to the Northwest District Office Compliance Assurance Program at the above address or by electronic mail at nwdair@dep.state.fl.us.

- Appendices:  The following Appendices are attached as a part of this permit:  Appendix A (Citation Formats and Glossary of Common Terms); Appendix B (General Conditions); Appendix C (Common Conditions); and Appendix D (Common Testing Requirements).

- Applicable Regulations, Forms and Application Procedures:  Unless otherwise specified in this permit, the construction and operation of the subject emissions units shall be in accordance with the capacities and specifications stated in the application.  The facility is subject to all applicable provisions of: Chapter 403, F.S.; and Chapters 62-4, 62-204, 62-210, 62-212, 62-213, 62-296 and 62-297, F.A.C.  Issuance of this permit does not relieve the permittee from compliance with any applicable federal, state, or local permitting or regulations.

- Asbestos:  This permit does not authorize any demolition or renovation of the facility or its parts or components which involves asbestos removal.  This permit does not constitute a waiver of any of the requirements of Chapter 62-257, F.A.C., and 40 CFR 61, Subpart M, National Emission Standard for Asbestos, adopted and incorporated by reference in Rule 62-204.800, F.A.C.  Compliance with Chapter 62-257, F.A.C., and 40 CFR 61, Subpart M, Section 61.145, is required for any asbestos demolition or renovation at the source.  [40 CFR 61; Rule 62-204.800, F.A.C.; and, Chapter 62-257, F.A.C.]

- New or Additional Conditions:  For good cause shown and after notice and an administrative hearing, if requested, the Department may require the permittee to conform to new or additional conditions.  The Department shall allow the permittee a reasonable time to conform to the new or additional conditions, and on application of the permittee, the Department may grant additional time.  [Rule 62-4.080, F.A.C.]

- Modifications:  The permittee shall notify the Permitting and Compliance Authorities upon commencement of construction.  Notification may be submitted by electronic mail to the Compliance Authority at nwdair@dep.state.fl.us and copied to the Permitting Authority at epost_nwdwasteair@dep.state.fl.us.  No new emissions unit shall be constructed and no existing emissions unit shall be modified without obtaining an air construction permit from the Department.  Such permit shall be obtained prior to beginning construction or modification.  [Rules 62-210.300(1) and 62-212.300(1)(a), F.A.C.]

- Construction and Expiration.  The expiration date shown on the first page of this permit provides time to complete the physical construction activities authorized by this permit, complete any necessary compliance testing, and obtain an operation permit.  Notwithstanding this expiration date, all specific emissions limitations and operating requirements established by this permit shall remain in effect until the facility or emissions unit is permanently shut down.  For good cause, the permittee may request that a permit be extended.  Pursuant to Rule 62-4.080(3), F.A.C., such a request shall be submitted to the Permitting Authority in writing before the permit expires.  [Rules 62-4.070(4), 62-4.080 & 62-210.300(1), F.A.C.]

- Application for Non-Title V Permit.  An application for an operation permit shall be submitted with the compliance test results and appropriate fee when applicable.  These are to be submitted within 75 days after initial operation.  The permittee shall obtain an operating permit for this source before the expiration of the construction permit if the permittee desires to continue operation.  You can obtain a permit application form or apply for permit renewal electronically at the following web address: http://www.dep.state.fl.us/air/rules/forms/application.htm .

[Rules 62.4-030 and 62-210.300(1)(a) & (2), F.A.C.]

- Source Obligation:

- At such time that a particular source or modification becomes a major stationary source or major modification (as these terms were defined at the time the source obtained the enforceable limitation) solely by virtue of a relaxation in any enforceable limitation which was established after August 7, 1980, on the capacity of the source or modification otherwise to emit a pollutant, such as a restriction on hours of operation, then the requirements of subsections 62-212.400(4) through (12), F.A.C., shall apply to the source or modification as though construction had not yet commenced on the source or modification.

- At such time that a particular source or modification becomes a major stationary source or major modification (as these terms were defined at the time the source obtained the enforceable limitation) solely by exceeding its projected actual emissions, then the requirements of subsections 62-212.400(4) through (12), F.A.C., shall apply to the source or modification as though construction had not yet commenced on the source or modification.

[Rule 62-212.400(12), F.A.C.]

This section of the permit addresses the following emissions unit.

| EU No. | Emission Unit Description |
| --- | --- |
| 001 | Asphalt Concrete Plant with Baghouse |

The portable counter-flow drum mix hot-mix asphalt concrete plant is comprised of an aggregate feed system, a drum dryer, an asphalt cement storage tank, and an asphalt product surge bin.  Front end loaders will be used to transfer aggregate from storage piles to the aggregate feed system.  Aggregate is fed onto a common collecting belt, and then conveyed to the drying drum.  After the aggregate is thoroughly dried, it is combined with Recycled Asphalt Product and baghouse fines, preheated to drive off moisture, and then mixed with hot liquid asphalt in the mixing drum to create the asphalt concrete product.  The final product is conveyed by a fully enclosed conveyor to the product surge bin for discharge into trucks.  Particulate matter emissions from the hot-mix asphalt concrete plant will be controlled by a baghouse.

Equipment

- Asphalt Concrete Plant with Baghouse:  The permittee is authorized to install a 250 TPH Hot-Mix asphalt concrete plant with a baghouse.  Equipment to be installed for this emissions unit include:

- Astec UniDrum portable dryer/drum asphalt mixer, with a 75 MM Btu/hr burner.

- Pulse jet baghouse, with a 150 HP electrically powered exhaust fan.

- Fuel oil fired hot oil heater.

- Asphalt cement storage tank.

- Asphalt product surge bin.

[Application No. 7775859-001-AC]

Performance Restrictions

- Permitted Capacity:  For compliance testing purposes only, the maximum allowable operating rate of the asphalt plant is 250 tons of asphalt concrete mix produced per hour.

[Rule 62-210.200((“Potential to Emit”)), F.A.C.; and Application No. 7775859-001-AC]

- Production:  The production rate of asphalt concrete (RAP or Non-RAP containing) shall not exceed 300,000 tons in any consecutive 12-month period.

[Rule 62-210.200(PTE), F.A.C.; and requested by applicant in Application No. 7775859-001-AC]

{Permitting Note: This limitation requested by the applicant is stricter than the limit required by Rule 62-210.300(3)(c)2.a., F.A.C.}

- Authorized Fuel:  The facility may use diesel fuel oil or No. 2 fuel oil.  The total combined amount of fuel oil consumed by the asphalt concrete plant, including the dryer burner, hot oil heater, portable RAP crusher engine, generator engines, and other stationary oil burning equipment, shall not exceed 730,000 gallons in any consecutive 12-month period.  The facility may not use fuel oil that has a sulfur content of more than 0.5% by weight.

[Rule 62-210.200(PTE), F.A.C., and requested by applicant in Application No. 7775859-001-AC]

{Permitting Note: These limitations requested by the applicant are stricter than the limits required by Rule 62-210.300(3)(c)2.c., F.A.C.}

- Hours of Operation:  The hours of operation are not limited (8760 hours per year).

[Rules 62-4.070(3) and 62-210.200(PTE), F.A.C.]

##### Emissions Standards

- Emissions Standards:

- Particulate Matter (PM):  Particulate matter emissions, associated with the asphalt concrete plant, shall not exceed 0.04 grains per dry standard cubic foot averaged over a three-hour period.

[Rules 62-210.300(3)(c)2.d., 62-204.800(8)(b)(13), F.A.C; and 40CFR 60.92(a)(1) (Subpart I)]

b.	Visible Emissions (VE):  Visible emissions, associated with the asphalt concrete plant shall be less than 20% opacity.  [Rules 62-210.300(3)(c)2.f., 62-204.800(8)(b)(13), F.A.C.; and 40 CFR 60.92(a)(2)]

##### Testing Requirements

- Initial Compliance Tests:  The emissions unit shall be tested to demonstrate initial compliance with the emissions standards for PM and VE.  The initial tests shall be conducted within 60 days after achieving permitted capacity, but not later than 180 days after initial operation of the unit.  Notification of compliance testing and completed test reports may be submitted by electronic mail to nwdair@dep.state.fl.us .

[Rules 62-4.070 and 62-297.310(8)(b)1, F.A.C.]

- Annual Compliance Tests:  During each calendar year (January 1st to December 31st), the emissions unit shall be tested to demonstrate compliance with the emissions standards for PM and VE.  Notification of compliance testing and completed test reports may be submitted by electronic mail to nwdair@dep.state.fl.us.  [Rule 62-297.310(8)(a), F.A.C.]

- Test Requirements:  The permittee shall notify the Compliance Authority in writing at least 15 days prior to any required tests.  Notification of compliance testing and completed test reports may be submitted by electronic mail to nwdair@dep.state.fl.us.  Tests shall be conducted in accordance with the applicable requirements specified in Appendix D (Common Testing Requirements) of this permit.

[Rule 62-297.310(9), F.A.C.]

- Test Methods:  Required tests shall be performed in accordance with the following reference methods.

| Method | Description of Method and Comments |
| --- | --- |
| 5 | Method for Determining Particulate Matter Emissions |
| 9 | Visual Determination of the Opacity of Emissions from Stationary Sources |

The above methods are described in Appendix A of 40 CFR 60 and are adopted by reference in Rule 62-204.800, F.A.C.  No other methods may be used unless prior written approval is received from the Department.  [Rule 62-204.800, F.A.C.; and Appendix A of 40 CFR 60]

Monitoring Requirements

- Baghouse Operating Parameters:  The operation of the baghouse shall be properly adjusted to maintain a 2 to 6 iWC pressure drop across the bags at all times to control particulate emissions.  The pressure drop shall be checked and recorded twice per day.  [Rule 62-4.070(3), F.A.C., and Application No. 7775859-001-AC]

Records and Reports

- Test Reports:  The Permittee shall prepare and submit reports for all required tests in accordance with the requirements specified in Appendix D (Common Testing Requirements) of this permit.  For each test run, the report shall also indicate the operating rate.  [Rule 62-297.310(10), F.A.C.]

- The Permittee shall record the baghouse pressure drop twice per day, and maintain the data in readily available form for inspection by the Department.  [Rule 62-4.070(3), F.A.C.]

- The Permittee shall maintain records to document the monthly and the 12-month rolling totals of tons of asphalt concrete produced and gallons of fuel oil consumed by the asphalt plant, including the dryer burner, hot oil heater, portable RAP crusher engine, diesel powered generator engines, and any other stationary oil burning equipment.  [Rules 62-4.070(3), F.A.C. and Application No. 7775859-001-AC]

- The Permittee shall maintain records of fuel supplier certifications for each shipment of fuel oil received.  The fuel oil supplier certification shall include the following information:

- The name of the supplier;

- A statement from the supplier that the fuel oil complies with the fuel oil sulfur content specification.

These records shall be maintained by the facility for a minimum of five years following the date of such record.  [Rules 62-4.070(3), F.A.C. and Application No. 7775859-001-AC]

PORTABLE RAP CRUSHERS

- Any portable RAP crusher operating at this facility is subject to all the terms and conditions of its current Air General (AG) Permit and any denoted applicable condition of this Permit.  A portable RAP crusher shall be tested in accordance with its current AG permit requirements.  [Rule 62-4.070(3), F.A.C.]

Location, Relocation, Co-Location

- Relocation:  Prior to any relocation of the asphalt concrete plant, the owner or operator must submit for Department approval a Notification of Intent to Relocate Air Pollutant Emitting Facility [DEP Form 62-210.900(6)] and a $250 minor modification application fee to the appropriate District office at least 30 days prior to the relocation.    You can obtain a permit application form or apply for permit renewal electronically at:  http://www.dep.state.fl.us/air/emission/permitting.htm.  The supplemental requirements of the form include:

- Provide a scale map (e.g., the relevant portion of a USGS topographic map) showing the proposed new location of the facility and points of air pollutant emissions in relation to roads and other features of the surrounding area.

- If relocating to a different DEP district or local air permitting office area of jurisdiction, provide a copy of the most recent compliance test report.

The Department shall require Permittee to publish public notice of Proposed Agency Action, which shall be provided by the Department prior to issuance of authorization for the relocation.  Proof of publication may be submitted by electronic mail to nwdair@dep.state.fl.us.

[Rule 62-210.900(6), F.A.C.]

- This permit is a conditional exemption from Title V permitting in accordance with Rule 62-210.300(3)(c)2, F.A.C.  An asphalt plant claiming this exemption from Title V air permitting shall not collocate with, or relocate to, any Title V source; nor shall it create a Title V source in combination with any other collocated facilities, emissions units, or pollutant-emitting activities, including any such facility, emissions unit, or activity that is otherwise exempt from permitting.  [Rule 62-210.300(3)(c)2.j., F.A.C]

NSPS REQUIREMENTS

- 40 CFR 60 Requirements Incorporated:  All requirements of Title 40, Code of Federal Regulations, part 60, subpart A, General Provisions, and Subpart I, Standards of Performance for Hot-Mix Asphalt Facilities, adopted and incorporated by reference at Rule 62-204.800(8), F.A.C., which are applicable to this source shall be adhered to at all times.

This section of the permit addresses the following emissions unit.

| EU No. | Emission Unit Description |
| --- | --- |
| 002 | Two Portable Diesel Fueled Generator Engines (RICE) |

This emissions unit consists of two diesel fueled, reciprocating internal combustion engines (RICE), which are used to power electrical generators.  These engines are included in this permit as a regulated emissions unit because emissions of one or more regulated air pollutant is greater than 5 tons per year.  They are being treated similarly to crusher engines, i.e., their emissions and fuel consumption are being considered as part of the whole asphalt plant facility.

These engines are considered non-road (portable) engines as defined in 40 CFR 1068.30; and are, therefore, not subject to 40 CFR 63 Subpart ZZZZ (RICE) or 40 CFR 60 Subpart IIII.  However, a portable engine will be considered stationary, and will be subject to these federal rules, if it remains at a location for more than 12 consecutive months.  Applicant has represented that the asphalt batch plant and its generators will not be at the same location for more than 12 consecutive months.

Operating Authorization

- This facility may operate the following diesel powered generators to provide electrical power to this facility:

| Description | Engine Type | Heat Input (MMBtu/hr) | Fuel Type | Kw | Brake hp | Mfg/Model Year |
| --- | --- | --- | --- | --- | --- | --- |
| Generator 1 | Non-road | 2.69 | Diesel | 286 | 384 | new |
| Generator 2 | Non-road | 5.2 | Diesel | 554 | 783 | new |

[Application No. 7775859-001-AC]

Essential Potential to Emit (PTE) Parameters

- Fuel Types and Usage.  The total combined amount of fuel oil consumed by these generator engines, the asphalt plant, the portable RAP crusher, and other stationary oil burning equipment, shall not exceed 730,000 gallons in any consecutive 12-month period.  The facility may not use fuel oil that has a sulfur content of more than 0.5% by weight.  [Rules 62-4.070(3), 62-210.200(PTE), F.A.C.; and requested by applicant in Application No. 7775859-001-AC]

Records and Reports

- Engine Operation Records.  In order to document compliance for these generator engines, the Permittee shall record the following while these generator engines are on site:

  - Engine manufacturer, serial number, model year, fuel type, engine horsepower, generator output, and any other identifying information so each generator engine can be clearly distinguished from another engine;

  - Date brought on site, date removed from site;

  - Hours of operation: per day*, per month and per rolling 12-month total.

* If the generator engines have run-time hour meters, it is acceptable to log, on the last day of each month, the run hours reading and not have a daily hour’s entry in the log.

This information shall be available no later than the last working day of the month following the end of each calendar month and shall be made available for inspection upon request by the Department.

[Rules 62-4.070(3) and 62-210.200(PTE), F.A.C.]

- The Permittee shall maintain records to document the monthly and the 12-month rolling totals of gallons of fuel oil consumed by the asphalt plant, including the dryer burner, hot oil heater, portable RAP crusher engine, diesel powered generator engines, and any other stationary oil burning equipment.

[Rules 62-4.070(3), F.A.C. and Application No. 7775859-001-AC]

- The Permittee shall maintain records of fuel supplier certifications for each shipment of fuel oil received.  The fuel oil supplier certification shall include the following information:

- The name of the supplier;

- A statement from the supplier that the fuel oil complies with the diesel fuel maximum sulfur content specification.

These records shall be maintained by the facility for a minimum of five years following the date of such record.  [Rules 62-4.070(3), F.A.C. and Application No. 7775859-001-AC]

Additional NSPS and NESHAP Requirements

- If any engine at the facility becomes subject to 40 CFR 60 Subpart IIII, 40 CFR 60 Subpart JJJJ, or 40 CFR 63 Subpart ZZZZ, the affected engine(s) shall comply with all the limitations and requirements of that Subpart.  The permittee shall apply for a construction permit to incorporate applicable portions of the federal regulations into the facility’s permit.  [Rules 62-4.070(3), 62-210.300(3)(a)35., 62-204.800(9) & (11), F.A.C.]

{Permitting Note: The engine in a portable unit, such as a portable RICE powered generator, is not considered a stationary engine unless the portable unit stays in the same location at the facility for more than 12 consecutive months.  See 40 CFR 60.4200(a)(2) and 40 CFR 1068.30}

This section of the permit addresses the following emissions unit.

| EU No. | Emission Unit Description |
| --- | --- |
| 003 | Portable RAP Crushing System |

A portable RAP crusher may be brought on site as needed and will operate under its current air general permit issued by the Department.  The portable crusher is subject to New Source Performance Standards (NSPS), Subpart OOO—Standards of Performance for Nonmetallic Mineral Processing Plants.

The engine is a non-road engine as defined in 40 CFR §1068.30.  40 CFR 63, Subpart ZZZZ (RICE) and 40 CFR 60, Subpart IIII apply to stationary compression ignition internal combustion engines that are not non-road engines.  Applicant has represented that the asphalt batch plant will not be at the same location for more than 12 consecutive months.

According to Rule 62-210.310(5)(e)5, F.A.C., no nonmetallic mineral processing plant using this air general permit shall perform a task routinely done at the individually permitted facility, such as crushing recycled asphalt pavement (rap) at an asphalt plant, unless operation of the nonmetallic mineral processing plant is authorized by the air construction permit or non-Title V air operation permit, as applicable, for the permitted facility.

The following specific conditions apply to the emissions unit listed above:

Operating Authorization

- This facility may operate a nonmetallic mineral processing plant on this site as allowed by its current air general permit and this permit.  The crushing system is subject to its current air general permit.  In case of conflict, this permit shall supersede the air general permit.

[Rule 62-210.310(5)(e)5, F.A.C., and Application No. 7775859-001-AC]

Essential Potential to Emit (PTE) Parameters

- Capacity.  The maximum permitted operating rate for the RAP crushing system is 200,000 tons per consecutive 12 months.  [Rule 62-210.200(PTE), F.A.C., and Application No. 7775859-001-AC]

- Fuel Types and Usage.  The total combined amount of fuel oil consumed by the asphalt plant, including the portable RAP crusher engine, the generator engines, and other stationary oil burning equipment, shall not exceed 730,000 gallons in any consecutive 12-month period.  The facility may not use fuel oil that has a sulfur content of more than 0.5% by weight.

[Rule 62-210.200(PTE), F.A.C., and requested by applicant in Application No. 7775859-001-AC]

- Florida Air Permit Requirement.  Authorization for Crusher Operations.  The operation of any currently Air General (AG) permitted crusher unit is allowed as long as the following criteria are complied with:

- Collocation Requirements.  Collocation of the crusher does not create a Title V source;

- Requirements.  The crusher meets the requirements of this permit and its AG permit.

- Notification.  The portable crusher owner or operator shall notify the Department’s Northwest District in writing prior to the relocation of a crusher to this facility’s site, in addition to meeting the relocation notification requirements of Rule 62-210.310(c)(5)3.f, F.A.C.

[Rule 62-4.070(3), F.A.C.]

Emissions Standards

- Emissions Standards.  This specific condition does not apply if the portable crushing system is not subject to 40 CFR 60 subpart OOO:

| Brief Description | Maximum Percent Opacity |  |
| --- | --- | --- |
| Portable Crusher | 15%1 | 12%2 |
| *Hopper (Non-40 CFR 60 Subpart OOO) | <20% |  |
| All conveyor Belt Transfer Points | 10%1 | 07%2 |
| All Screens | 10%1 | 07%2 |
| All other Non-NSPS Points | <20% |  |
| Crusher Diesel Engine | <20% |  |

1 For crushers and affected pieces of equipment (e.g., screens) that commenced construction, modification or reconstruction after 8/31/1983, but before 4/22/2008.

2 For crushers and affected pieces of equipment (e.g., screens) that commenced construction, modification or reconstruction on or after 4/22/2008.

[Rules 62-210.200 (PTE) and 62-296.320(4)(b), F.A.C.; 40 CFR 60.672]

{*Permitting Note:  Visible emissions from the transfer of material into the hopper and onto the coarse and fine storage piles are covered under the facility-wide visible emission limitation of 20%.  These transfer points do not require visible emissions testing.}

- Opacity Standard - Exceptions.  This specific condition does not apply if the portable crushing system is not subject to 40 CFR 60 subpart OOO:  The opacity standards stated above, apply at all times except during periods of startup, shutdown, and malfunction of the emissions unit.  [40 CFR 60.11(c), and Rule 62-204.800(8), F.A.C.]

- Subpart A – General Provisions.  This specific condition does not apply if the portable crushing system is not subject to 40 CFR 60 subpart OOO:  The owner or operator shall comply with the applicable requirements of 40 CFR 60 subpart A – General Provisions.  [40 CFR 60.1(a) and Rule 62-204.800(8), F.A.C.]

Testing Requirements

- Visible Emissions Test Requirements.  This specific condition does not apply if the portable crushing system is not subject to 40 CFR 60 subpart OOO:  In order to determine compliance with the visible emission limitations for the crushing system that is brought on site, Permittee shall maintain on site either documentation of the most recent annual visible emissions test that complies with the crushing system’s General Permit requirements prior to locating at this site or shall test on site.  The most recent test shall not be older than the previous federal fiscal year.  [Rule 62-4.070(3), F.A.C.]

- Off-Site Test Requirements.  This specific condition does not apply if the portable crushing system is not subject to 40 CFR 60 subpart OOO:  An off-site test may be accepted by the Department provided the required documentation listed in Records and Reports below can be provided to the Compliance Authority with the test report.  In addition, the crushing system must have been tested while operating with the same or more emission points and equipment that will be operating while on site.  If an off-site visible emissions test is used to demonstrate compliance, the crusher shall not be operated at rates greater than 110% of the actual processing rate measured during that test.  [Rule 62-4.070(3), F.A.C.]

- On-Site Test Requirements.  This specific condition does not apply if the portable concrete crushing system is not subject to 40 CFR 60 subpart OOO:  If adequate documentation is not available, Permittee shall test for visible emissions as soon as possible but no later than 30 days of placing the crushing system into operation after the effective date of this permit.  Once tested, the crushing system may not operate in a configuration that has more pieces of equipment and/or emission points than were operating during the test unless a new compliance test is conducted with the greater number of pieces of equipment and/or emission points.  If the crusher remains on site, the crushing system shall be tested for visible emissions annually during each federal fiscal year (October 1 – September 30) that the crushing system is on site.  [Rules 62-4.070(3) and 62-297.310 (7)(a), F.A.C.]

- Visible Emissions Testing Requirements.  This specific condition does not apply if the portable concrete crushing system is not subject to 40 CFR 60 subpart OOO:  On-site visible emissions testing shall be conducted in accordance with the following:

- The duration of the Method 9 observations must be 30 minutes (five six-minute averages).  Compliance with the VE opacity limit must be based on the average of the five six-minute averages.

[Rules 62-297.310(4), and 62-297.401, F.A.C.; and 40 CFR 60.675 (c) and (e)(2)]

- Test Method.  This specific condition does not apply if the portable concrete crushing system is not subject to 40 CFR 60 Subpart OOO:  Required tests shall be performed in accordance with the following reference methods.

| Method(s) | Description of Method and Comments |
| --- | --- |
| 9 | Visual Determination of the Opacity of Emissions from Stationary Sources |

The above method is described in Appendix A of 40 CFR 60 and is adopted by reference in Rule 62-204.800, F.A.C.  No other method may be used unless prior written approval is received from the Department.

[Rules 62-204.800 and 62-297.401, F.A.C.; and 40 CFR 60, Appendix A]

Monitoring Requirements

- Monitoring Requirements.  This specific condition does not apply if the portable concrete crushing system is not subject to 40 CFR 60 subpart OOO.  If any affected piece(s) of equipment of the concrete crushing system (i.e., crusher, screen or conveyor belt) was constructed, modified or began reconstruction on or after April 22, 2008, a monthly inspection must be performed to check that water is flowing to discharge spray nozzles of the wet suppression system.  The owner or operator must initiate corrective action within 24 hours and complete corrective action as expediently as practical if water is not flowing properly during the inspection.

[40 CFR 60.674(b)]

Notification Requirements

- On-Site Test Notification.  Permittee shall notify the Compliance Authority in writing at least 15 days prior to any required on-site tests.  The notification must include the following information:  the date, time and location of each test; the name and telephone number of the facility’s contact person who will be responsible for coordinating the test; and, the name, company and telephone number of the person conducting the test.  The notification should also include the relevant emissions unit ID No(s)., test method(s) to be used, and pollutants to be tested.  Notification may be submitted by electronic mail to nwdair@dep.state.fl.us.

[Rules 62-4.070(3) and 62-297.310(7)(a)9., F.A.C.]

Records and Reports

- On-Site Test Report Requirements.  Permittee shall prepare and submit to the Compliance Authority reports for all required tests in accordance with the requirements specified below.  Reports may be submitted by electronic mail to nwdair@dep.state.fl.us.  The test report must include the following:

- owner name;

- current Air General Permit number;

- manufacturer’s maximum rated capacity for any material (i.e., the rated TPH capacity of the material with the highest processing rate);

- date each affected piece of equipment of the crushing system commenced construction, modification or reconstruction;

- type of material processed;

- actual material handling rate during the test period (tons/hour);

- configuration of the crushing system during the test to include specific pieces of equipment, emission points and/or a process flow diagram; and,

- a copy of the operating logs for the day and month the test was conducted.

[Rules 62-4.070(3) and 62-297.310(8), F.A.C.]

- Off-Site RAP Crushing System Test Reports.  When demonstrating compliance by the visible emissions test, permittee shall maintain a copy of the most recent VE test report for each crushing system that operated on site.  [Rule 62-4.070(3) F.A.C.]

- Crusher Operation Records.  In order to document crusher compliance, Permittee shall require the crushing contractor to record and provide to the Permittee the following, while the crushing system is on site:

  - Facility name, facility ID No., emissions unit ID No., crushing system owner’s name, Florida General or non-Title V Air Operation Permit No., crushing system serial number and any other identification information so each crushing system can be clearly distinguished from another crushing system;

  - Operating configuration while on-site to include specific pieces of equipment, emission points;

  - Date of most recent visible emissions test and a copy of the test report;

  - Date brought on site, date removed from site;

  - Hours of operation: per day*, per month and per rolling 12-month total; and,

  - Concrete throughput in tons: per day, per month and per rolling 12-month total.

* If the crushers’ engines have run-time hour meters, it is acceptable to log, on the last day of each month, the run hours reading and not have a daily hour’s entry in the log.

This information shall be available no later than the last working day of the month following the end of each calendar month and shall be made available for inspection upon request by the Department.

[Rule 62-4.070(3), F.A.C.]

- Monitoring Records.  This specific condition does not apply if the portable concrete crushing system is not subject to 40 CFR 60 subpart OOO:  If any affected piece(s) of equipment of the concrete crushing system was constructed, modified or began reconstruction on or after April 22, 2008, the owner or operator must record each inspection of the water spray nozzles, including the date of each inspection and any corrective actions taken.  [40 CFR 60, 60.674(b)]

- The Permittee shall maintain records to document the monthly and the 12-month rolling totals of gallons of fuel oil consumed by the asphalt plant, including the dryer burner, hot oil heater, portable RAP crusher engine, diesel powered generator engines, and any other stationary oil burning equipment.

[Rules 62-4.070(3), F.A.C. and Application No. 7775859-001-AC]

- The Permittee shall maintain records of fuel oil supplier certifications for each shipment of fuel oil received.  The fuel supplier certification shall include the following information:

  - The name of the supplier;

  - A statement from the supplier that the fuel oil complies with the fuel oil maximum sulfur content specification.

These records shall be maintained by the facility for a minimum of five years following the date of such record.  [Rules 62-4.070(3), F.A.C. and Application No. 7775859-001-AC]

Additional NSPS and NESHAP Requirements

- If any engine at the facility becomes subject to 40 CFR 60 Subpart IIII, 40 CFR 60 Subpart JJJJ, or 40 CFR 63 Subpart ZZZZ, the affected engine(s) shall comply with all the limitations and requirements of that Subpart.  The permittee shall apply for a construction permit to incorporate applicable portions of the federal regulations into the facility’s permit.  [Rules 62-4.070(3), 62-210.300(3)(a)35., 62-204.800(9) & (11), F.A.C.]

{Permitting Note: The engine in a portable unit, such as a rock crusher, is not considered a stationary engine unless the portable unit stays in the same location at the facility for more than 12 consecutive months.  See 40 CFR 60.4200(a)(2) and 40 CFR 1068.30. }