Title V Air Operation Permit Renewal

Permit No. 0250476-015-AV

Applicant

The applicant for this project is Miami-Dade Water and Sewer Department.  The applicant’s responsible official and mailing address are: Robert Fergen, Chief of WWTP Operations and Maintenance, Miami-Dade Water and Sewer Department, PO Box 330316, Miami, Florida.

Facility Description

The Central District Wastewater Treatment Plant Cogeneration Facility (CDWWTP) is engaged in wastewater treatment activities and is publicly owned.  The first treatment facilities at the Virginia Key site became operational in 1956 with a capacity of 47 millions of gallons per day (mgd).  Since that time numerous additions and modifications to the plant have been made, including the change from aeration to oxygenation in the original plant.  The facility consists of two parallel wastewater treatment trains: named Plant No. 1 and Plant No. 2. Plant 1 is rated at 60 mgd annual average daily flow and Plant 2 is rated for 83 mgd annual average daily flow.  The total facility is permitted to treat 143 mgd annual average daily flow.  The liquid process is designated as Emissions Unit (EU) 008 which consists of aerated grit chambers, oxygenation tanks, sludge concentration tanks, secondary clarifiers, and chlorine contact basins.

Electricity for in-plant operations is produced by four nominal 1.2 Megawatt (MW) Superior Digester Gas Electric Cogenerator engines (model 16GTLD) designated as EU021, EU022, EU 024, and EU025.  Backup emergency power is provided by three nominal 2.5 MW diesel engine-driven electric standby generators (model EMD 20-645E4B) designated as EU 013, 014, and 015; and two nominal 2.865 MW diesel engine electric standby generators, (model 20-645F4B) designated as EU 019 and 020.  A 37 HP Diesel Engine, designated at EU 023, is available to provide compressed air to start the emergency generators in the event all electrical power is lost at the facility.  In addition, there are eleven digester gas flares at the facility designated as EU 018.

There is an agreement with the local utility, that the facility provide peak shaving or non-emergency demand response, in exchange for financial incentives to the facility.  Because of this agreement, per Federal Rule 40 CFR 63 Subpart ZZZZ, the existing diesel driven standby generators (EU 013, 014, 015, 019 and 020) are considered to be non-emergency engines, and therefore require the installation of oxidation catalysts.  The 37 HP Diesel Engine (EU023) is also regulated by the Federal Regulation 40 CFR 63 Subpart ZZZZ.

Also, included in this permit are miscellaneous insignificant emissions units and/or activities (see Appendix I, List of Insignificant Emissions Units and/or Activities).

regulated Emissions Unit IDentification Numbers and descriptions

| EU No. | Brief Description |
| --- | --- |
| Regulated Emissions Units |  |
| 013 | 2.5 MW Diesel Engine Electric Generator #1 |
| 014 | 2.5 MW Diesel Engine Electric Generator #2 |
| 015 | 2.5 MW Diesel Engine Electric Generator #3 |
| 019 | 2.865 MW Diesel Engine Electric Generator #4 |
| 020 | 2.865 MW Diesel Engine Electric Generator #5 |
| 021 | 1.2 MW Digester Gas Electric Co-generator (#3) |
| 022 | 1.2 MW Digester Gas Electric Co-generator (#4) |
| 023 | 37 HP Emergency Air Compressor Diesel Engine |
| 024 | 1.2 MW Digester Gas Electric Co-generator (#1) |
| 025 | 1.2 MW Digester Gas Electric Co-generator (#2) |
| 027 | Gasoline Dispensing Facility |
| Unregulated Emissions Units and/or Activities |  |
| 008 | Wastewater Treatment Plant - Liquid Processes |
| 017 | Wastewater Treatment Plant – Solids Handling Processes |
| 018 | Wastewater Treatment Plant – Digester Gas Flares |

Applicable Regulations

Based on the Title V air operation permit renewal application received on August 9, 2019, this facility is not a major source of hazardous air pollutants (HAP).  The existing facility is a prevention of significant deterioration (PSD) major source of air pollutants in accordance with Rule 62-212.400, F.A.C. A summary of applicable regulations is shown in the following table:

| Regulation | EU No(s). |
| --- | --- |
| Federal Rule Citations |  |
| 40 CFR 60, Subpart A, NSPS General Provisions | 021, 022, 024, 025 |
| 40 CFR 60, Subpart JJJJ, NSPS for Spark Ignition Engines | 021, 022, 024, 025 |
| 40 CFR 63, Subpart A, NESHAP General Provisions | 013, 014, 015, 019, 020, 021, 022, 023, 024, 025 |
| 40 CFR 63, Subpart ZZZZ, NESHAP for Reciprocating Engines | 013, 014, 015, 019, 020, 021,022, 023, 024, 025, 027 |
| 40 CFR 63, Subpart CCCCCC, Gasoline Dispensing Facilities | 027 |
| Section 112(r) of CAA Prevention of Accidental Releases |  |
| State Rule Citations |  |
| 62-4, F.A.C., Permits |  |
| 62-204, F.A.C., Air Pollution Control - General Provisions |  |
| 62-210, F.A.C., Stationary Sources - General Requirements |  |
| 62-212, F.A.C., Stationary Sources - Preconstruction Review |  |
| 62-213, F.A.C., Title V Major Source |  |
| 62-296, F.A.C., Stationary Sources - Emission Standards 62-296.570(4)(a)3. 62-296.570(4)(b)7. | 013, 014, 015, 019, 020 |
| 62-297, F.A.C., Stationary Sources - Emissions Monitoring |  |

Project DESCRIPTION

The purpose of this permitting project is to renew the Title V permit for the above referenced facility.

Processing Schedule and Related Documents

Application for a Title V Air Operation Permit Renewal received August 9, 2019

Additional information request dated September 16, 2019

Partial Additional information response received January 3, 2020

Partial Additional information response received January 31, 2020

Partial Additional information response received February 26, 2020

Additional information response received March 10, 2020

Primary Regulatory requirements

Primary Standard Industrial Classification (SIC) Code:  4952 – Sewerage Systems.

Primary North American Industry Classification System (NAICS):  221320. – Sewage Treatment Facilities.

Secondary Standard Industrial Classification (SIC) Code:  4941 – Water Supply.

Secondary North American Industry Classification System (NAICS):  221310. – Water Supply and Irrigation Systems.

HAP:  The facility is not identified as a major source of hazardous air pollutants (HAP).

Title IV:  The facility does not operate units subject to the acid rain provisions of the Clean Air Act.

Title V:  The facility is a Title V major source of air pollution in accordance with Chapter 62-213, Florida Administrative Code (F.A.C.).

PSD:  The facility is a Prevention of Significant Deterioration (PSD)-major source of air pollution in accordance with Rule 62-212.400, F.A.C.

NSPS:  The facility operates units subject to the New Source Performance Standards (NSPS) of 40 Code of Federal Regulations (CFR) 60.

EUs 021, 022, 024, 025: 40 CFR 60, Subpart JJJJ, Standards of Performance for Stationary Spark Ignition Internal Combustion Engines.

NESHAP:  The facility operates units subject to the National Emissions Standards for Hazardous Air Pollutants (NESHAP) of 40 CFR 63.

EUs 013, 014, 015, 019, 020, 021, 022, 023, 024, 025: 40 CFR 63, Subpart ZZZZ, National Emissions Standards for Hazardous Air Pollutants for Stationary Reciprocating Internal Combustion Engines

EU 027: 40 CFR 63, Subpart CCCCCC, National Emissions Standards for Hazardous Air Pollutants for Gasoline Dispensing Facilities.

The Central District Wastewater Treatment Plant is presently not subject to Subpart VVV, as only 40 CFR 63.1580(a)(1) and (3) are true to the facility at this time and not 40 CFR 62.1580(a)(2).  The Central District Wastewater Treatment Plant is not a 40 CFR 63.1581 (a) "Group 1 POTW treatment plant" as defined in 40 CFR 63.1595, and has not and is not a major source of HAP.  In accordance with 40 CFR 63.1581(b), the facility is a "Group 2 POTW treatment plant" as defined in 40 CFR 63.1595.

CAIR:  The facility is not subject to the Clean Air Interstate Rule (CAIR) set forth in Rule 62-296.470,

F.A.C.

CAM:  Compliance Assurance Monitoring (CAM) does not apply to any of the units at the facility.  Each emissions unit is subject to 111 or 112 standards promulgated after 11/15/90.

PROJECT REVIEW

- Facility-wide Condition 9 was added to the permit due to the changes to the Title V Permit format.

FW9.	Semi-Annual Monitoring Reports.  The permittee shall monitor compliance with the terms and conditions of this permit and shall submit reports of any deviations from the requirements of these conditions at least every six (6) months.  All instances of deviations from permit requirements must be clearly identified in such reports, including reference to the specific requirement and the duration of such deviation.  All reports shall be accompanied by a certification by a responsible official, pursuant to subsection 62-213.420(4), F.A.C.  (See also Conditions RR2. – RR4. of Appendix RR, Facility-wide Reporting Requirements, for additional reporting requirements related to deviations.)

[Rule 62-213.440(1)(b)3.a., F.A.C.; Rule 2.501, JEPB]

{Permitting Note:  EPA has clarified that, pursuant to 40 CFR 70.6(a)(3), the word “monitoring” is used in a broad sense and means monitoring (i.e., paying attention to) the compliance of the source with all emissions limitations, standards, and work practices specified in the permit.}

- The Permitting Note to the Common Testing Requirements permit condition has been added due to the changes to the Title V Permit format.

Common Testing Requirements.  Unless otherwise specified, tests shall be conducted in accordance with the requirements and procedures specified in Appendix TR, Facility-Wide Testing Requirements, of this permit.  [Rule 62-297.310, F.A.C.]

{Permitting Note:  Air compliance test notifications can now be completed online in the Department’s Business Portal.  To access this online process, go to http://www.fldepportal.com/go/home and sign in (or register if you’re a new user) from the link in the upper right corner of the page.  On the Welcome page select the Submit option, then select Registration/Notification, and then click on Air Compliance Test Notifications.  Once in the process, just carefully read the instructions on each screen (and under the Help tabs) to complete the notification.}

- Three relocatable Lochinvar Copper – Fin CB 2067 (or equivalent) 2.2 MMBtu/hr propane fired hot water boilers used to maintain the temperatures in the biosolids digesters to ensure biogas production and proper volatile reduction of solids required for the beneficial reuse of the biosolids for land application while the renovation of the digesters is taking place have been added to Appendix I.

- Emissions Unit 027 has been added to the permit for the existing, but previously identified in Appendix I, gasoline dispensing facility.  This emissions unit has been removed from Appendix I as it is subject to 40 CFR 63 subpart CCCCCC – Gasoline Dispensing Facilities.

- A permitting note has been added to the permit subsections for the facility to provide in the submitted permit application for the next Title V renewal or air construction permit modification, whichever occurs first, the facility-wide potential individual and total HAP emissions.

{Permitting Note: During the next Title V renewal or air construction permit modification, whichever comes first, the facility-wide potential individual (i.e., formaldehyde, etc.) and total HAP emissions need to be provided by the facility in the submitted permit application.}

Conclusion

This project renews Title V air operation permit No. 0250476-012-AV, which was effective on March 24, 2015.  This Title V air operation permit renewal is issued under the provisions of Chapter 403, Florida Statues (F.S.), and Chapters 62-4, 62-210, and 62-213, F.A.C.