September 23, 2005

Mr. Richard Dodd, P.E., President

GAC Contractors, Inc.

4116 North Highway 231

Panama City, Florida  32404

Dear Mr. Dodd,

Enclosed is a copy of the report completed by Mr. Richard S. Brookins Jr. describing his inspection of your Panama City facility on Friday, September 16, 2005.  The cooperation and assistance of Ms. Lisa Schiek and Mr. Steve Churchwell during the inspection is greatly appreciated.

This letter applies only to activities covered by the Air Resources Management Program.  It indicates a state of compliance for the sources specifically listed in the report.  However, please note the Recommendation section of the report.

If you have any questions, please contact Mr. Richard S. Brookins Jr. at (850) 872-4375 x108 or e-mail at Richard.Brookins@dep.state.fl.us.

Sincerely,

Josie Penton

Environmental Manager

INSPECTION REPORT

AIR POLLUTANT EMISSION SOURCES

| FACILITY: | Gulf Asphalt Cement | DISTRICT: 10 |  |  | COUNTY: 03 |
| --- | --- | --- | --- | --- | --- |
|  | 4116 N. Highway 231 | CONTACT: | Steve Churchwell |  |  |
|  | Panama City | TELEPHONE: |  | (850) 769-0316 |  |
| Email Address: Dodd@GACcontractors.com |  |  |  |  |  |

| AIRS ID#: 0050008 | PERMIT#: 0050008-002-AO | EXPIRATION DATE: 07/22/2009 |
| --- | --- | --- |
| Permit Renewal Application/Notification Due Date: 05/23/2009 |  |  |

| INSPECTION DATE | AUDIT TYPE | COMPLIANCE STATUS |
| --- | --- | --- |
| 09/16/2005 | II | In Compliance |
| SKY | WIND | TEMPERATURE |
| Clear | 1-3 mph | High 80’s |

SOURCE DESCRIPTION:  This facility consists of a 189 ton per hour asphalt cement batch plant.  Both virgin material and RAP are utilized in the production process.  Particular emissions are controlled by a baghouse.

INSPECTION COMMENTS:

I arrived at the facility and was greeted by Ms. Lisa Schier.  Ms. Schier provided copies of requested records for review.  After the record review, Mr. Steve Churchwell  accompanied me on the inspection of the facility and provided access to the fuel oil sulfur content verification reports.  The facility was not operating at the time of the inspection.

There were no objectionable odors noticed during the inspection.  [Facility-wide Condition 2]

There were no fugitive emissions in excess of 20% noticed during the inspection.   [Facility-wide Condition 3]

There were no open containers of VOC’s or other Organic Solvents noticed during the inspection.  [Facility-wide Condition 4]

Reasonable precautions to prevent fugitive emissions were judged to be adequate at the time of the inspection.  However, since the facility yard is part dirt and part asphalt, please see the Recommendation section of the report.  [Facility-wide Condition 6]

A copy of the facility’s Operation and Maintenance Plan was available for review at the time of inspection.  [Facility-wide Condition 11]

EU001	ASPHALT PLANT

According to records provided, the facility produced 58,922 tons of asphalt cement for the 12-month period ending August 31, 2005.  This is within the facility’s permitted limit of 500,000 tons in any consecutive 12-month period.  [Specific Condition A.2]

### According to the records provided, the facility operated for 545.58 hours for the 12-month period ending August 31, 2005.  This is within the facility’s permitted limit of 3000 hours in any consecutive 12-month period.  [Specific Condition A.3]

According to records provided, the facility used 134,880 gallons of fuel oil for the burner during the 12-month period ending August 31, 2005.  This is within the facility’s limit of 1.2 million gallons in any consecutive 12-month period.  Information was provided which showed fuel oil deliveries to be less than 0.05% sulfur.  [Specific Condition A.4]

Since the facility was not operating, visible emissions were 0% at the time of the inspection.  [Specific Condition A.6]

### Scale calibration records were available for review.  [Specific Condition A.8]

Monthly and 12-month rolling totals of asphalt concrete produced, fuel oil used, and hours of operation were available for inspection.  [Specific Condition A.11]

EU002	ASPHALT HEATER:

According to records provided, the facility used 34,957 gallons of fuel oil for the asphalt heater during the 12-month period ending August 31, 2005.  Information was provided which showed fuel oil deliveries to be less than 0.05% sulfur.  [Specific Condition B.1]

RECOMMENDATIONS:

At the time of inspection the asphalt heater was operating on natural gas due to the recent fuel oil shortage caused by Hurricane Katrina.  Natural gas is not a permitted fuel for this emission unit and if the facility wishes to continue to have the capability to burn natural gas after the fuel crisis is over, a permit modification should be requested from the District office.

During the dry times of the year more frequent watering of the general yard and roadway areas will be necessary to prevent excessive fugitive emissions.  Ensure that a water truck is available for this purpose at the facility.   As an alternative to using water the facility may want to investigate the potential use of organic dust suppressing chemicals.

| INSPECTOR’S NAME | SIGNATURE | DATE |
| --- | --- | --- |
| Richard S. Brookins Jr. |  |  |
| Environmental Specialist |  | September 19, 2005 |