| AIR POLLUTION SOURCE INSPECTION REPORT FORM DEPARTMENT OF ENVIRONMENTAL PROTECTION |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| FACILITY: Ajax Paving Industries – Palmetto Plant |  |  |  |  | PAGE: 1 OF 2 |  |  |
| FACILITY ADDRESS: 12165 US 41 N |  |  | CITY: Palmetto, FL 34221 |  |  |  | COUNTY: Manatee |
| MAILING ADDRESS: 510 Gene Green Road |  |  |  | CITY: Nokomis | STATE: FL |  | ZIP: 34275 |
| INSP. DATE: 8/27/2010 | TIME IN: 10:45 a.m. |  |  | TIME OUT: 11:45 a.m. | INSP TYPE: INS2 |  | STATUS: IN |
| IDENTIFICATION NO: 0810063 |  |  | EU ID # Facility Wide |  |  |  |  |
| SOURCE/FACILITY DESCRIPTION: Asphalt Plant |  |  |  |  |  |  |  |
| CONTACT(S): Tim Gearheart, Plant Foreman |  |  |  |  |  |  |  |
| I, James Burkholder (JB), and Joe Panetta (JP) went to Ajax Paving Industries Palmetto Plant to conduct an unannounced facility inspection. The purpose of this inspection was to perform a records review and a facility walkthrough to verify the facility is operating according to permit. This facility operates a Standard Havens, Incorporated Drum Mix Asphalt Plant with a design capacity to produce a maximum of 300.0 tons/hour of hot mix asphalt concrete. In addition to processing virgin raw materials, this plant also processes Recycle Asphalt Pavement (RAP). Also included at this plant is a Meeker Oil Heating System, used to keep the liquid asphalt at the proper temperature. When JP and I, JB, arrived on site we met with Mr. Time Gearheart, Plant Foreman of Ajax Paving. The daily, monthly, and 12 month rolling averages for the asphalt plant and RAP crusher were requested. The daily records are hand written during operation and are then typed up and kept in a 3 ring binder on site. The daily records for August 26, 2010 were: Start Time: 07:30 (military time); Total Production: 751 tons. End Time: 10:30 (military time); Production Rate: 250.33 tons/hr. Hours operated: 3 hours; Fuel Usage: 2438.48 Therms of Natural Gas. On November 24, 2009 annual compliance testing was performed at 250 tons per hour of raw virgin material; therefore, the new permitted production limit is now set at 275 tons per hour. A spot check of daily records from December of 2009 to August of 2010 showed that the facility typically operates at approximately 250 tons per hour and under the limit of 275 tons per hour. Monthly and 12-month rolling numbers are kept on the same spreadsheet. The following 12-month data was collected: Date (month); 12-mon tonnage; Fuel Oil (gal); Hours Oper.; Hot Oil (gal); Nat Gas (ft3); Lime (tons); Fiber (tons); Crushed RAP (tons); Crusher fuel (gal). Aug 09’; 277,358; 0.0; 1219.8; 0.0; 7390973; 30.2; 13.8; 0.0; 0.0. July 10’; 282,012; 0.0; 1221.0; 0.0; 7161426; 283.1; 113.1; 44234.9; 2149.5. |
| INSPECTOR(S): James Burkholder (JB); Joe Panetta (JP) |  | TIME EXPENDED: JB – 170 mins JP – 90 mins |  |  |  | REPORT DATE: 8/31/2010 |  |

| DEPARTMENT OF ENVIRONMENTAL PROTECTION CONTINUATION PAGE 2 OF 2 |  |  |  |
| --- | --- | --- | --- |
| FACILITY: Ajax Paving Industries, Inc. Palmetto Plant |  | INSPECTION DATE: 8/27/2010 |  |
| The 12 month rolling records show the facility is well under the 500,000 ton annual throughput limit and 1.2 million gallons of fuel limit. The facility operates the drum mix asphalt plant on natural gas and the crushers are operated by diesel fuel. The facility also keeps a daily RAP crusher log containing hours operated, tons produced, average ton/hr, and fuel used for each crusher. The 12 month rolling fuel oil and hot oil usage shows 0.0 gallons. The facility only operates the hot drum mix asphalt plant on natural gas. The last time fuel oil was fired was approximately 2 years ago only for compliance testing purposes. The asphalt storage silos have heating capabilities; however, the facility does not currently operate the Meeker Oil Heater system to heat the silos. With economic concerns and product demand Ajax typically makes enough product to fill orders and does not store asphalt. The quick turnover allows the asphalt to remain hot and the heater system is not used. The facility grounds were well wetted due to recent rainy conditions in the state of Florida. Previous on-site inspections did not reveal any problems with fugitive emissions from the yard or storage piles. At the time of inspection a crusher was not on site and the facility has not had a crusher on site since June 28, 2010. A portable lime silo and fiber feeder were on site but not in use. The facility has current Enforcement for unauthorized use of a crusher, fiber feeder, and lime silo. The case is still open but the proper correction actions have been taken. A file review for the Full Compliance Evaluation showed all reports were submitted timely and compliance testing is being conducted once per federal fiscal year. At the time of inspection the facility appeared to be in compliance with all permit conditions. |  |  |  |
| INSPECTOR: James Burkholder (JB) and Joe Panetta (JP) | REPORT DATE: 8/31/2010 |  | FACILITY ID NUMBER: 0810063 |