| 1030077 88752 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FACILITY: Times Publishing Company |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | PERMIT ID: 989 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| St. Petersburg Times Printing Plant |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | DISTRICT: Southwest |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ADDRESS: 1301 34th Street North |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CONTACT PHONE: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| St. Petersburg, FL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 727-893-8281 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ARMS NO: 1030077 004 |  |  |  |  |  |  |  | PERMIT NO: |  |  |  |  |  |  |  |  |  |  |  | Expiration Date: 9/10/2018 Renewal Date: 7/10/2018 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | 1030077-008-AF |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Test Due Date: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EMISSION UNIT DESCRIPTION : Rack repair painting operations and imaging/platemaking. Damaged newspaper racks are repaired and then repainted in the rack repair paint spray booth, which is located in the 3131 building |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CMS INSPECTION DATE: |  |  |  |  |  |  | ARMS INSPECTION TYPE: |  |  |  |  |  |  |  |  |  |  |  |  |  |  | COMPLIANCE STATUS: |  |  |  |  |  |  |  |  |  |  |  |  |
|  | May 15, 2014 |  |  |  |  |  |  |  |  | INS1 |  |  | INS2 |  |  | INS3 |  |  | FUI |  |  |  |  |  | IN |  |  | MNC |  |  |  |  | SNC |  |
| INSPECTION TYPE: |  |  |  |  | Initial |  |  |  |  |  | Re-inspection |  |  |  |  |  | Complaint |  |  |  |  |  |  | Drive-by |  |  |  |  |  | Quarterly |  |  |  |  |
|  |  |  | A. General Review: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1. |  | Permit File Review |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  | No |
|  |  | Comments: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2. |  | Introduction and Entry |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  | No |
|  |  | Comments: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3. |  | Is the Responsible Official/Authorized Representative still: Benjamin H. Hayes? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  | No |
|  |  | Comments: The Responsible Official/Authorized Representative’s e-mail is: bhayes@tampabay.com |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4. |  | Is the facility contact still: Johnathan Kemp? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  | No |
|  |  | Comments: The facility contact’s e-mail is: jkemp@tampabay.com? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| IN | MNC | SNC | B. Specific Conditions: |
| --- | --- | --- | --- |
|                    |  |  | 8. If, for any reason, the permittee does not comply with or will be unable to comply with any condition or limitation specified in this permit, the permittee shall immediately provide the Department with the following information: a. A description of and cause of noncompliance; and b. The period of noncompliance, including dates and times; or, if not corrected, the anticipated time the noncompliance is expected to continue, and steps being taken to reduce, eliminate, and prevent recurrence of the noncompliance. The permittee shall be responsible for any and all damages which may result and may be subject to enforcement action by the Department for penalties or for revocation of this permit. IN MNC SNC N/A Comments: There were no periods of noncompliance reported during compliance year 12. This permit or a copy thereof shall be kept at the work site of the permitted activity. IN MNC SNC N/A Comments: A copy of the permit was was not kept at the worksite. SECTION 4. APPENDIX C - Common Conditions [Rule 62-296.320(4)(c), F.A.C.] 2. Circumvention: The permittee shall not circumvent the air pollution control equipment or allow the emission of air pollutants without this equipment operating properly. [Rule 62-210.650, F.A.C.] IN MNC SNC N/A Comments: There was no circumvention observed during the compliance year 6. VOC or OS Emissions: No person shall store, pump, handle, process, load, unload or use in any process or installation, volatile organic compounds (VOC) or organic solvents (OS) without applying known and existing vapor emission control devices or systems deemed necessary and ordered by the Department. [Rule 62-296.320(1), F.A.C.] IN MNC SNC N/A Comments: - During the time of inspection there was no indication/observance of leakage from the fittings or pipes associated with the 9 printing presses and no solvent odors were detected. - The rag disposal container lids were sealed. If hydraulic fluid from the presses is observed to be leaking, it is cleaned immediately. - To prevent fugitive VOC emissions, the facility uses cleanup socks and rags to clean up the hydraulic fluid that are disposed of in sealed containers. The rags and cleanup socks are then recycled. 7. Objectionable Odor Prohibited: No person shall cause, suffer, allow or permit the discharge of air pollutants, which cause or contribute to an objectionable odor. An “objectionable odor” means any odor present in the outdoor atmosphere which by itself or in combination with other odors, is or may be harmful or injurious to human health or welfare, which unreasonably interferes with the comfortable use and enjoyment of life or property, or which creates a nuisance. [Rules 62-296.320(2) and 62-210.200(Definitions), F.A.C.] IN MNC SNC N/A Comments: An upwind/downwind survey of the facility was conducted. The observed parameters were: Downwind odor level detected- none ; Wind direction – SE/7 Upwind odor level detected-none 9. Unconfined Particulate Emissions: a. No person shall cause, let, permit, suffer or allow the emissions of unconfined particulate matter from any activity, including vehicular movement; transportation of materials; construction, alteration, demolition or wrecking; or industrially related activities such as loading, unloading, storing or handling; without taking reasonable precautions to prevent such emissions. b. Any permit issued to a facility with emissions of unconfined particulate matter shall specify the reasonable precautions to be taken by that facility to control the emissions of unconfined particulate matter. c. Reasonable precautions include the following: (1) Paving and maintenance of roads, parking areas and yards. (2) Application of water or chemicals to control emissions from such activities as demolition of buildings, grading roads, construction, and land clearing. (3) Application of asphalt, water, oil, chemicals or other dust suppressants to unpaved roads, yards, open stock piles and similar activities. (4) Removal of particulate matter from roads and other paved areas under the control of the owner or operator of the facility to prevent reentrainment, and from buildings or work areas to prevent particulate from becoming airborne. (5) Landscaping or planting of vegetation. (6) Use of hoods, fans, filters, and similar equipment to contain, capture and/or vent particulate matter. (7) Confining abrasive blasting where possible. (8) Enclosure or covering of conveyor systems. IN MNC SNC N/A Comments: There were no unconfined emissions observed. The roads are paved in the plant area and there is vegetation/plants and trees around the main building Permit Administrative Requirements x7. General Standards: Objectionable Odor - No person shall cause, suffer, allow or permit the discharge of air pollutants, which cause or contribute to an objectionable odor. An objectionable odor is any odor present in the outdoor atmosphere, which by itself or in combination with other odors, is or may be harmful or injurious to human health or welfare, which unreasonably interferes with the comfortable use and enjoyment of life or property, or which creates a nuisance. If the Department and/or Pinellas County Air Quality Division (PCAQD) receives a valid odor complaint, the Department and/or PCAQD reserves the right to require that odor control measures/work practices be implemented. [Rules 62-4.070(3), 62-210.200 (“Objectionable Odor”) and 62-296.320(2), F.A.C.; Pinellas County Code, Section 58-178] IN MNC SNC N/A Comments: There were no objectionable odors detected during the inspection. x8 Annual Operating Report: On or before April 1 of each year, the permittee shall submit a completed DEP Form 62-210.900(5), "Annual Operating Report for Air Pollutant Emitting Facility" (AOR) for the preceding calendar year. The report may be submitted electronically in accordance with the instructions received with the AOR package sent by the Department, or a hardcopy may be sent to the Compliance Authority. [Rule 62-210.370(3), F.A.C.] IN MNC SNC N/A Comments: The annual operating report for calendar year 2013 was submitted on _3/21/14. ***Assigned for edit checks only 9 Operation Permit Renewal Application: A completed application for renewal of the operation permit shall be submitted to the Permitting Authority with a copy to PCDEM (Compliance Authority) no later than 60 days prior to the expiration date of the operation permit. [Rules 62-4.030, 62-4.050, 62-4.070(3), 62-4.090, 62-210.300(2), and 62-210.900, F.A.C.] IN MNC SNC N/A Comments: Expiration Date: 9/10/2018 Renewal Date: 7/10/2018 This section of the permit addresses emission units (EU 004). Rack Repair Paint Spray Booth & Imaging and Plate Making: EU004 consists of the rack repair painting operations and imaging/platemaking. Damaged newspaper racks are repaired and then repainted in the rack repair paint spray booth, which is located in the 3131 building. The use of paints and solvents in the painting operations result in the emissions of VOCs, HAPs and particulate matter. The paint spray booth is equipped with an exhaust fan and fabric filters to control overspray emissions. The paint spray booth is a Binks special floor type spray booth with a 30” fan and 5 hp motor. The fan is specified to draw 10,800 cfm through a “paint pocket” fabric filter. The filter is rated to provide 95% removal of paint solidA.1. Permitted Capacity - The maximum production rate for the Lithographic presses shall not exceed 540,000 newspapers/hr. [Rules 62-4.160(2), 62-204.800, 62-210.200 (Potential to Emit), 62-296.405, F.A.C.; and, Construction Permit No. 1030077-003-AC.] IN MNC SNC N/A Comments: The facility meets the provisions of this condition. The fabric filters are changed out every 2-3 days. The hourly run was = 57,675 newspapers/hr. Press #3 was the only one operating during this time period. A.3. Visible Emissions (VE) Limitation - Visible emissions shall not exceed 20% opacity, however, in order to provide reasonable assurance that the filter system is being properly operated, visible emissions due to paint overspray should not exceed 5 percent opacity. Exceedance of the 5% limit shall not be considered a violation in and of itself, but an indication that additional control precautions and/or practices may be necessary. [Rules 62-4.070(3) & 62-296.320(4)(b)1., F.A.C.] IN MNC SNC N/A Comments: There were no visible emissions observed during the inspection. The last VE test for the BACT engineering Model SLC5-5-S Central Dust Collection System was performed on 5/31/13 and received by the Air Quality Division on 6/11/13. 30 minute run and 0% opacity. A.4. VOC and HAP Emissions: - VOC emissions from all press operations, imaging/platemaking, and rack repair painting operations shall not exceed 98 tons per consecutive 12-month period. - Total HAP emissions from all press operations, imaging/platemaking, and rack repair painting operations shall not exceed 16.0 tons per consecutive 12-month period, with no individual HAP emission exceeding 8.0 tons. Supporting documentation (MSDS, purchase orders, certificates of analysis, U.S. EPA “VOC DATA SHEETS”, etc.) shall be kept at the facility for each ink, paint, coating, solvent, and other material containing VOCs and HAPs which includes sufficient information to determine VOC and total HAP emissions. In addition, sufficient records shall be maintained so that monthly individual HP emissions can be calculated, if requested. (Permitting Note: The VOC emission limit of Specific Condition A.4.a. is very close to the Title V threshold of 100 ton/yr. Please be aware that if the Title V threshold is exceeded, the facility will revert to Title V and be required to obtain a Title V permit) [Rule 62-210.370, F.A.C.; Initial FESOP Application submitted 10/25/2013] IN MNC SNC N/A Comments: (Permitting Note: The VOC emission limit of Specific Condition A.4.a. is very close to the Title V threshold of 100 ton/yr. Please be aware that if the Title V threshold is exceeded, the facility will revert to Title V and be required to obtain a Title V permit) A.6. Monthly Log for Material Usage- A monthly log shall be established and maintained to document compliance with Condition No. A.4. At a minimum, the log shall identify and quantify each ink, solvent, and other material containing VOCs and/or HAPs that were used in the press operations and rack repair painting operations. Documentation of solvents consumed may use a mass balance method to determine usage (amount used minus amount collected for disposal or recycle, less solids). The monthly log shall be updated and completed by the 20th day of the following month and shall include a t a minimum the following: - Facility name, Facility ID, Emission Unit ID, Source Description, and Date (Month, Year); - A monthly total usage of each ink, paint, coating, solvent, and other VOC and/or HAP containing material used (gallons); - The VOC and/or HAP content for each material (pounds/gallon); - A monthly sum of VOC emissions (tons); e A monthly sum of HAP emissions (tons); f. A most recent rolling 12-consecutive month sum of VOC emissions (tons); and g. A most recent rolling 12-consecutive month sum of HAP emissions (tons). If any month results in VOC emissions being within 10% of the allowable emission limit in Specific Condition No. A.4.a. for the prior consecutive 12-month period (i.e., over 88 tons), the monthly log for VOC emissions shall be kept daily. IN MNC SNC N/A Comments: The monthly log for material usage was reviewed and it meets the provisions of this condition. or Daily Log for Recordkeeping - The option of using purchases as a proxy for usage is not allowed for this daily recordkeeping. The total daily values for the month shall then be added to the previous 11-month period total emissions for VOC. The daily recordkeeping shall remain in effect until the consecutive 12-month period total VOC emissions are below 88 tons. These logs shall be completed within three (3) business days. IN MNC SNC N/A Comments: This log was reviewed and meets the provision of this condition, A.7. Record Retention - Records required by this permit shall be maintained on-site or electronically available on-site for a minimum of the most recent five (5) year period and made available to the Department and the PCAQD upon request. [Rule 62-4.070(3), F.A.C.] IN MNC SNC N/A Comments: The records are kept on-site for a minimum of the most 5 year period.. |
|  |  |  | #### Valid Permit [Rule 62-210.300] |
|  |  |  | #### Changes to Facility/emission unit [Rule 62-210.300] Does the emission unit description above match what the facility is operating (Number of emission units or points, model number and serial number, etc.) Yes No Comments: |

|  |  | C. Other: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Pollution Prevention Activities - P2 Handouts Provided: P2 Brochure; P2 Manual; P2 Checklist - Have any emissions reductions occurred Yes / No Chemical Substitution; Equipment Changes; Process Changes Chemical/Material Reuse; On-site Recycling; Other: Comments: There were no pollution prevention activities during the compliance year |  |  |  |  |  |  |  |
| Closing Conference: |  |  |  |  |  |  |  |
| Inspector(s): Jeffrey Morris, Pinellas County, Air Quality Division |  |  |  |  |  |  |  |
| Signature(s): Date: |  |  |  |  |  |  | Date: |
|  | # ACCESS? |  |  | # ARMS? |  |  |  |

| AIRS ID 1030077 | OWNER Times Publishing Company |  | FACILITY NAME St. Petersburg Times Printing Plant |
| --- | --- | --- | --- |
| TITLE V |  | SYNTHETIC MINOR | DATE OF THIS FCE May 15, 2014 |
| TITLE V MEGA-SITE* |  | OTHER | DATE OF LAST FCE June 19, 2012 |

*Facility with a large number of complex emissions units.  It is more reasonable to evaluate a Title V Mega-Site once every 3 years instead of once every 2 years.

Review of All Required Reports

| PERIODIC REPORTS |  | COMMENTS |
| --- | --- | --- |
| DONE N/A | Annual operating report | 2013 AOR received by AQD on 2/24/14. It is currently being held for edit checks. FDEP is processing it. Because the facility has been redesignated as a synthetic minor in 2014, the EAOR will be processed by AQD in 2015. |
| DONE N/A | Statement of compliance | Received on 3/3/14 and reviewed on 3/4/14 |
| DONE N/A | Annual |  |
| DONE N/A | Semi-annual |  |
| DONE N/A | Quarterly |  |
| DONE N/A | Other : |  |
| DONE N/A | Other : |  |

| CONTINUOUS EMISSION MONITOR REPORTS |  | COMMENTS |
| --- | --- | --- |
| DONE N/A | Quarterly excess emissions |  |
| DONE N/A | Semi-annual |  |
| DONE N/A | RATA |  |
| DONE N/A | CGA |  |
| DONE N/A | Other : |  |
| DONE N/A | Other : |  |

Assessment of Control Device and Process Operating Conditions

| OFF-SITE ASSESSMENT |  | (Describe the off-site assessment in comments) |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ON-SITE ASSESSMENT |  | (Document the on-site inspection below) |  |  |  |
| DATE OF INSPECTION | DATE OF INSPECTION REPORT |  | My office maintains the inspection report... |  |  |
|  |  |  | ...in the ARMS database through EASIIR. | ...with the paper or electronic compliance files | ...in another location (specify). |
| May 15, 2014 | May 15, 2014 |  |  |  |  |

Review of Tests and Records

| TESTS, OBSERVATIONS AND RECORDS |  | COMMENTS |
| --- | --- | --- |
| DONE N/A | Visible emission observation(s) | Notified 5/16/13 and performed on 5/31/13 |
| DONE N/A | Review of facility records and logs | Copy of 12-mo consecutive total with inspection report |
| DONE N/A | Assessment of process parameters (feed rates, process rates, raw material compositions, etc.) |  |
| DONE N/A | Assessment of control equipment performance parameters (water flow rates, pressure drops, temperatures, ESP power levels, etc.) | Received a copy of the Baghouse O&M Plan Daily O&M Inspection Checklist |
| DONE N/A | Stack test(s) |  |
| DONE N/A | Other : |  |
| DONE N/A | Other : |  |

Compliance Monitoring (CM) Information

| CM ELEMENT | My office maintains this information... |  |  |  |
| --- | --- | --- | --- | --- |
|  | ...electronically, in the ARMS database. | ...in the permit. | ...in the inspection report. | ...in another location (specify). |
| Facility information |  |  |  |  |
| Applicable requirements |  |  |  |  |
| Inventory of emission units |  |  |  |  |
| Enforcement history |  |  |  |  |
| Compliance activities |  |  |  |  |
| Findings and recommendations |  |  |  |  |
| COMMENTS |  |  |  |  |

Other Comments

Prepared by:	   Jeffrey Morris		Date:

Reviewed by:			Date:

1.  Complete the form as much as possible during for inspection; 2.  The remainder of checklist, for activities in the FY, are filled in no later than 9/15;  3.  Between 9/15 and 9/30, complete the FCE by closing out the FCS project in ARMS.  The INSP > FCS record only accepts a completion date and a comment; it does not accept information on the lower portion of the form.  If an FCS project already exists for the fiscal year in question, enter a completion date and comment in the existing record, do not create a duplicate FCS record.