| 1030077 88755 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FACILITY: Times Publishing Company |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | PERMIT ID: 4056 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| St. Petersburg Times Printing Plant |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | DISTRICT: Southwest |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ADDRESS: 1301 34th Street North |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CONTACT PHONE: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| St. Petersburg, FL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 727-893-8281 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ARMS NO: 1030077 007 |  |  |  |  |  |  |  | PERMIT NO: |  |  |  |  |  |  |  |  |  |  |  | Expiration Date: 9/10/2018 Renewal Date: 7/10/2018 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | 1030077-008-AF |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Test Due Date: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EMISSION UNIT DESCRIPTION : 10,000 gallon fleet gasoline fuel dispensing operations at the printing facility. The fueling point is used for fleet vehicles only. Gasoline dispensing averages 709 gallons per month; Also on-site is a 10,000 gallon diesel tank |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CMS INSPECTION DATE: |  |  |  |  |  |  | ARMS INSPECTION TYPE: |  |  |  |  |  |  |  |  |  |  |  |  |  |  | COMPLIANCE STATUS: |  |  |  |  |  |  |  |  |  |  |  |  |
|  | May 15, 2014 |  |  |  |  |  |  |  |  | INS1 |  |  | INS2 |  |  | INS3 |  |  | FUI |  |  |  |  |  | IN |  |  | MNC |  |  |  |  | SNC |  |
| INSPECTION TYPE: |  |  |  |  | Initial |  |  |  |  |  | Re-inspection |  |  |  |  |  | Complaint |  |  |  |  |  |  | Drive-by |  |  |  |  |  | Quarterly |  |  |  |  |
|  |  |  | A. General Review: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1. |  | Permit File Review |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  | No |
|  |  | Comments: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2. |  | Introduction and Entry |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  | No |
|  |  | Comments: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3. |  | Is the Responsible Official/Authorized Representative still: Benjamin H. Hayes? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  | No |
|  |  | Comments: The Responsible Official/Authorized Representative’s e-mail is: bhayes@tampabay.com |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4. |  | Is the facility contact still: Johnathan Kemp? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  | No |
|  |  | Comments: The facility contact’s e-mail is: jkemp@tampabay.com? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| IN | MNC | SNC | B. Specific Conditions: |
| --- | --- | --- | --- |
|               |  |  | 6. Modifications - Unless otherwise exempt by rule, the permittee shall not initiate any construction, reconstruction, or modification at the facility and shall not install/modify any pollution control device at the facility without obtaining prior authorization from the Department. Modification is defined as: Any physical change or changes in the method of operations or addition to a facility that would result in an increase in the actual emissions of any air pollutant subject to air regulations, including any not previously emitted, from any emission unit or facility. [Rules 62-210.200 - Definition of “Modification” and 62-210.300(1)(a), F.A.C.] IN MNC SNC N/A Comments: There were no modifications of this permit during the compliance year 7. Annual Operating Report - On or before April 1 of each year, the permittee shall submit a completed DEP Form 62-210.900(5), "Annual Operating Report for Air Pollutant Emitting Facility" (AOR) for the preceding calendar year. The report may be submitted electronically in accordance with the instructions received with the AOR package sent by the Department, or a hardcopy may be sent to the Compliance Authority. [Rule 62-210.370(3), F.A.C.] IN MNC SNC N/A Comments: The annual operating report for calendar year 2013 was submitted on 2/24/14. ***Note: FDEP Tallahassee reviews Title V AOR’s 8. Operation Permit Renewal Application - A completed application for renewal of the operation permit shall be submitted to the Permitting Authority with a copy to Pinellas County Air Quality Division (Compliance Authority) no later than 60 days prior to the expiration date of this operation permit. To properly apply for an operation permit, the applicant shall submit the following: a. the appropriate permit application form (see current version of Rule 62-210.900, F.A.C. (Forms and Instructions), and/or FDEP Division of Air Resource Management website at: http://www.dep.state.fl.us/air/); b. the appropriate operation permit application fee from Rule 62-4.050(4)(a), F.A.C.; c. copies of the most recent compliance test reports required by Specific Condition No. B.10, if not previously submitted; d. copies of the most recent month of records/logs specified in Specific Condition Nos. A.7, B.7, B.14, C.11, C.12 and D.4.; and e. any proposed revisions to the most recently approved O & M Plan (Agency name) (see Specific Condition No. B.14.). [Rules 62-4.030, 62-4.050, 62-4.070(3), 62-4.090, 62-210.300(2), and 62-210.900, F.A.C.] IN MNC SNC N/A Comments: Expiration Date: 9/10/2018 Renewal Date: 7/10/2018 9. General Volatile Organic Compounds (VOC) Emissions or Organic Solvents (OS) Emissions - The permittee shall allow no person to store, pump, handle, process, load, unload or use in any process or installation, volatile organic compounds or organic solvents without applying known and existing vapor emission control devices or systems deemed necessary and ordered by the Department. The following procedures shall be utilized to minimize pollutant emissions: - All equipment, pipes, hoses, lids, fittings, etc. shall be operated and maintained in such a manner as to minimize leaks, fugitive emissions, and spills of solvent materials. - All VOC/OS from washings (equipment clean-up) shall be directed into containers that prevent evaporation into the atmosphere. - All VOC containers shall be tightly covered or closed when they are not in use. - Excessive air turbulence across exposed VOCs shall be prevented. - VOC/OS spills shall be immediately confined and cleaned up and wastes placed in closed containers for reuse, recycling or proper disposal as appropriate. [Rule 62-296.320(1), F.A.C. and Construction Permit No. 103077-003-AC] IN MNC SNC N/A Comments: - During the time of inspection there was no indication/observance of leakage from the fittings or pipes associated with the 9 printing presses and no solvent odors were detected. - The rag disposal container lids were sealed. If hydraulic fluid from the presses is observed to be leaking, it is cleaned immediately. - To prevent fugitive VOC emissions, the facility uses cleanup socks and rags to clean up the hydraulic fluid that are disposed of in sealed containers. The rags and cleanup socks are then recycled. 10. General Visible Emissions - No person shall cause, let, permit, suffer or allow to be discharged into the atmosphere the emissions of air pollutants from any activity equal to or greater than 20% opacity. EPA Method 9 is the method of compliance pursuant to Chapter 62-297, F.A.C. This regulation does not impose a specific testing requirement. [Rule 62-296.320(4)(b)1, F.A.C.] IN MNC SNC N/A Comments: No visible emissions were observed with this emission unit 11. Unconfined Particulate Matter - No person shall cause, let, permit, suffer or allow the emissions of unconfined particulate matter from any activity, including vehicular movement; transportation of materials; construction; alteration; demolition or wrecking; or industrially related activities such as loading, unloading, storing or handling; without taking reasonable precautions to prevent such emissions. Reasonable precautions to prevent emissions of unconfined particulate matter at this facility include: a. capturing rack repair paint spray booth overspray emissions with a filter system; b. collecting emissions from the newspaper folders to the fabric filter; and c. general housekeeping. [Rule 62-296.320(4)(c), F.A.C.; Construction Permit No. 1030077-003-AC.] IN MNC SNC N/A Comments: There were no unconfined emissions observed. The roads are paved in the plant area and there is vegetation/plants and trees around the main building 12. Objectionable Odor Prohibited. No person shall cause, suffer, allow or permit the discharge of air pollutants, which cause or contribute to an objectionable odor. An “objectionable odor” means any odor present in the outdoor atmosphere which by itself or in combination with other odors, is or may be harmful or injurious to human health or welfare, which unreasonably interferes with the comfortable use and enjoyment of life or property, or which creates a nuisance. [Rule 62-296.320(2) and 62-210.200(Definitions), F.A.C.; Pinellas County Code Section 58-178; Construction Permit 1030077-003-AC] IN MNC SNC N/A Comments: An upwind/downwind survey of the facility was conducted. The observed parameters were: Downwind odor level detected- none ; Wind direction – W/7mph Upwind odor level detected- none - This section of the permit addresses the following emission unit (EU 007). Fleet Gasoline Fuel Dispensing Operation: This emissions unit is the fleet gasoline fuel dispensing operations at the printing facility. The fueling point is used for fleet vehicles only. Gasoline dispensing averages 709 gallons per month. The affected source includes each gasoline cargo tank during the delivery of product and also includes each storage tank. D.1. Federal Regulatory Requirements – The Fleet Gasoline Fuel Dispensing Operation (EU No. 007) is subject to and shall comply with the applicable requirements* of Federal NESHAP 40 CFR 63 Subpart CCCCCC (National Emission Standards for Hazardous Air Pollutants for Source Category: Gasoline Dispensing Facility), included in the attached Appendix F. NESHAP 40 CFR 63, Subpart CCCCCC, which is made a part of this permit. [Rule 62-204.800(11), F.A.C.; NESHAP 40 CFR 63 Subpart CCCCCC] (* NESHAP Subpart CCCCCC Permitting Note – This gasoline fueling point located at an area source of HAP is subject to the NESHAP Subpart CCCCCC requirements for facilities with monthly throughput of less than 10,000 gallons of gasoline. Most of the key requirements of Subpart CCCCCC are also included in the specific conditions of this subsection as referenced under the conditions in the bracketed rule basis.) IN MNC SNC N/A Comments: The facility is in compliance with provisions of this condition D.2. Permitted Capacity - The maximum throughput of gasoline shall be less than 10,000 gallons per month. [Rule 62-204.800(11), F.A.C.; NESHAP 40 CFR 63.11116] IN MNC SNC N/A Comments: At no time did the maximum gasoline throughput exceed 10,000 gallons/month. A copy of the record is attached to this inspection report. D.3. Methods of Operation - Gasoline to be handled in a manner to minimize vapor releases to the atmosphere for extended periods of time. Measures to be taken include, but are not limited to, the following: - Minimize gasoline spills; - Clean up spills as expeditiously as practicable; - Cover all open gasoline containers and all gasoline storage tank fill-pipes with a gasketed seal when not in use; - Minimize gasoline sent to open waste collection systems that collect and transport gasoline to reclamation and recycling devices, such as oil/water separators. [Rule 62-204.800(11), F.A.C.; NESHAP 40 CFR 63.11116] IN MNC SNC N/A Comments: The facility follows these procedures RecordKeeping and Reporting requirements D.4. Monthly Log - To document compliance with Specific condition D.2., records shall be kept of gasoline and diesel product purchased and dispensed through this emission unit. Records must be available within 24 hours of a request from the appropriate compliance or permitting authority. [Rule 62-204.800(11), F.A.C.; NESHAP 40 CFR 63.11116] IN MNC SNC N/A Comments: The facility keeps this record and a copy of the log is attached to the inspection report. |
|  |  |  | #### Valid Permit [Rule 62-210.300] |
|  |  |  | #### Changes to Facility/emission unit [Rule 62-210.300] Does the emission unit description above match what the facility is operating (Number of emission units or points, model number and serial number, etc.) Yes No Comments: |

|  |  | C. Other: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Pollution Prevention Activities - P2 Handouts Provided: P2 Brochure; P2 Manual; P2 Checklist - Have any emissions reductions occurred Yes / No Chemical Substitution; Equipment Changes; Process Changes Chemical/Material Reuse; On-site Recycling; Other: Comments: There were no pollution prevention activities during the compliance year |  |  |  |  |  |  |  |
| Closing Conference: |  |  |  |  |  |  |  |
| Inspector(s): Jeffrey Morris, Pinellas County, Air Quality Division |  |  |  |  |  |  |  |
| Signature(s): Date: |  |  |  |  |  |  | Date: |
|  | # ACCESS? |  |  | # ARMS? |  |  |  |

| AIRS ID 1030077 | OWNER Times Publishing Company |  | FACILITY NAME St. Petersburg Times Printing Plant |
| --- | --- | --- | --- |
| TITLE V |  | SYNTHETIC MINOR | DATE OF THIS FCE May 15, 2014 |
| TITLE V MEGA-SITE* |  | OTHER | DATE OF LAST FCE June 19, 2012 |

*Facility with a large number of complex emissions units.  It is more reasonable to evaluate a Title V Mega-Site once every 3 years instead of once every 2 years.

Review of All Required Reports

| PERIODIC REPORTS |  | COMMENTS |
| --- | --- | --- |
| DONE N/A | Annual operating report | 2013 AOR received by AQD on 2/24/14. It is currently being held for edit checks. FDEP is processing it. Because the facility has been redesignated as a synthetic minor in 2014, the EAOR will be processed by AQD in 2015. |
| DONE N/A | Statement of compliance | Received on 3/3/14 and reviewed on 3/4/14 |
| DONE N/A | Annual |  |
| DONE N/A | Semi-annual |  |
| DONE N/A | Quarterly |  |
| DONE N/A | Other : |  |
| DONE N/A | Other : |  |

| CONTINUOUS EMISSION MONITOR REPORTS |  | COMMENTS |
| --- | --- | --- |
| DONE N/A | Quarterly excess emissions |  |
| DONE N/A | Semi-annual |  |
| DONE N/A | RATA |  |
| DONE N/A | CGA |  |
| DONE N/A | Other : |  |
| DONE N/A | Other : |  |

Assessment of Control Device and Process Operating Conditions

| OFF-SITE ASSESSMENT |  | (Describe the off-site assessment in comments) |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ON-SITE ASSESSMENT |  | (Document the on-site inspection below) |  |  |  |
| DATE OF INSPECTION | DATE OF INSPECTION REPORT |  | My office maintains the inspection report... |  |  |
|  |  |  | ...in the ARMS database through EASIIR. | ...with the paper or electronic compliance files | ...in another location (specify). |
| May 15, 2014 | May 15, 2014 |  |  |  |  |
| COMMENTS |  |  |  |  |  |

Review of Tests and Records

| TESTS, OBSERVATIONS AND RECORDS |  | COMMENTS |
| --- | --- | --- |
| DONE N/A | Visible emission observation(s) | Notified 5/16/13 and performed on 5/31/13 |
| DONE N/A | Review of facility records and logs | Copy of 12-mo total with inspection report |
| DONE N/A | Assessment of process parameters (feed rates, process rates, raw material compositions, etc.) |  |
| DONE N/A | Assessment of control equipment performance parameters (water flow rates, pressure drops, temperatures, ESP power levels, etc.) | Received a copy of the Baghouse O&M Plan Daily O&M Inspection Checklist |
| DONE N/A | Stack test(s) |  |
| DONE N/A | Other : |  |
| DONE N/A | Other : |  |

Compliance Monitoring (CM) Information

| CM ELEMENT | My office maintains this information... |  |  |  |
| --- | --- | --- | --- | --- |
|  | ...electronically, in the ARMS database. | ...in the permit. | ...in the inspection report. | ...in another location (specify). |
| Facility information |  |  |  |  |
| Applicable requirements |  |  |  |  |
| Inventory of emission units |  |  |  |  |
| Enforcement history |  |  |  |  |
| Compliance activities |  |  |  |  |
| Findings and recommendations |  |  |  |  |
| COMMENTS |  |  |  |  |

Other Comments

Prepared by:	   Jeffrey Morris		Date:

Reviewed by:			Date:

1.  Complete the form as much as possible during for inspection; 2.  The remainder of checklist, for activities in the FY, are filled in no later than 9/15;  3.  Between 9/15 and 9/30, complete the FCE by closing out the FCS project in ARMS.  The INSP > FCS record only accepts a completion date and a comment; it does not accept information on the lower portion of the form.  If an FCS project already exists for the fiscal year in question, enter a completion date and comment in the existing record, do not create a duplicate FCS record.