| 1030209 80601 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FACILITY: Transitions Optical, Inc. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | PERMIT ID: 198 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | DISTRICT: Southwest |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ADDRESS: 9251 Belcher Rd. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CONTACT PHONE: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Pinellas Park, FL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 727-545-0400 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ARMS NO: 1030209 002 |  |  |  |  |  |  |  | PERMIT NO: |  |  |  |  |  |  |  |  |  |  |  | Expiration Date: 4/7/2016 Renewal Date: 2/17/2016 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | 1030209-028-AO |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Test Due Date: 12/24/2015 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EMISSION UNIT DESCRIPTION : Lens Shaping Area 1: Up to 20 pieces of equipment (edgers, grinders, lathes, etc.) controlled by an Airotech reverse pulse fabric filter dust collector, Model 5P-130-8-(10) BA, or a Dust-Hog Supra-Blast, Model SBD8-2, cartridge dust collector as a backup |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CMS INSPECTION DATE: |  |  |  |  |  |  | ARMS INSPECTION TYPE: |  |  |  |  |  |  |  |  |  |  |  |  |  |  | COMPLIANCE STATUS: |  |  |  |  |  |  |  |  |  |  |  |  |
|  | January 17, 2012 |  |  |  |  |  |  |  |  | INS1 |  |  | INS2 |  |  | INS3 |  |  | FUI |  |  |  |  |  | IN |  |  | MNC |  |  |  |  | SNC |  |
| INSPECTION TYPE: |  |  |  |  | Initial |  |  |  |  |  | Re-inspection |  |  |  |  |  | Complaint |  |  |  |  |  |  | Drive-by |  |  |  |  |  | Quarterly |  |  |  |  |
|  |  |  | A. General Review: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1. |  | Permit File Review |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  | No |
|  |  | Comments: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2. |  | Introduction and Entry |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  | No |
|  |  | Comments: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3. |  | Is the Responsible Official/Authorized Representative still: Vincent Sweeney? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  | No |
|  |  | Comments: The Responsible Official/Authorized Representative’s e-mail is: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4. |  | Is the facility contact still: Christine Appel? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  | No |
|  |  | Comments: The facility contact’s e-mail is: CAppel@Transitions.com |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| IN | MNC | SNC | B. Specific Conditions: |
| --- | --- | --- | --- |
|               |  |  | - Inspection note: Permits 1030209-028-AO (expires 04/07/2016) and 1030209-029-AC (expires 11/30/2011) were issued simultaneously. The construction permit modifies the following emissions units. - The description now includes activities previously identified as exempt and removes the descriptions of specific steps in the coating process. - It also removes the reference to a maximum physical production capacity of 30,000,000 lenses/year and a peak of 9,000 lenses/hour. These capacities were not permit limits. - It adds monthly recordkeeping requirements to record usage and percent by weight of VOCs and HAPs and changes the weekly recordkeeping requirements to daily if 90% of the Title V threshold values for HAP emissions are exceeded. - It removes the list of the dated Operation and Maintenance (O & M) plans. - It removes welding from the list of exempt activities B.1. Permitted Capacity - The maximum number of pieces of lens shaping equipment that can operate in each lens shaping area is as follows: a. EU 002 - Lens Shaping Area No. 1 – twenty (20) pieces of equipment. [Rule 62-210.200 (“Potential to Emit”), F.A.C.; Construction Permit 1030209-026-AC] Comments: 4 pieces of equipment were operating during the inspection. B.2. Dust Collector Operation – The primary dust collector in each lens shaping area may not operate simultaneously with the backup dust collector in that area. [Rule 62-4.070(3), F.A.C.; Construction Permit 1030209-026-AC] Comments: During the time of inspection, only the primary dust collector was operating. The facility did not operate the backup dust collector simultaneously with the primary unit. B.3. Circumvention Limitation - No person shall circumvent any air pollution control device, or allow the emission of air pollutants without the applicable air pollution control device (baghouses) operating properly. As an indicator that the particulate control devices are operating properly, the Department has determined there should be no visible emissions (≤5% opacity) from each device. Visible emissions in excess of 5% are not considered a violation in and of itself, but an indicator of improper operation and may indicate a violation of Rule 62-210.650, F.A.C. ##### [Rules 62-4.070(3) and 62-210.650, F.A.C.; Construction Permit 1030209-026-AC] Comments: There was no circumvention observed of the dust collector during the inspection. Based on a Method 22, there were no emissions observed. Therefore, it appears the dust collector was operating properly. B.4. Compliance Tests - Each of the 4 2 dust collectors associated with Emission Unit Nos. 002 and 009 shall be tested for visible emissions at least 105 days prior to and no more than 365 days prior to the expiration date of this permit. Since the Dust-Hog Supra-Blast Dust Collector associated with Emission Unit No. 009 was not operational prior to the effective date of this permit, this dust collector shall be tested within 30 days after being placed back in operation. If the back-up dust collector associated with either EU No. 002 or EU No. 009 has not operated in the five years prior to permit renewal, no testing is required. If either back-up dust collector is subsequently operated after that permit renewal, that dust collector shall be tested within 30 days of operation. The test report(s) shall be submitted within 45 days of testing to the Air Compliance Sections of this office and the Pinellas County Air Quality Division. [Rule 62-297.310, F.A.C.] Comments: - This dust collector was tested on July 6, 2010. - The permit expired on December 22, 2010. The test was performed 169 days prior to permit expiration. The back-up dust collector was not tested since it was not operated in the 5 years prior to permit renewal. - The test was received by Air Quality on July 7, 2010, 1 day after the test. B.5. Test Requirements - Tests shall be conducted in accordance with the applicable requirements specified in Appendix D (Common Testing Requirements) of this document. [Rule 62-297.310, F.A.C.] Comments: The facility meets the test requirement provisions in Appendix D. B.6. Test Method - Required tests shall be performed in accordance with the following reference method. Method; Description of Method and Comments. 9; Visual Determination of the Opacity of Emissions from Stationary Sources. The above method(s) are described in Appendix A of 40 CFR 60 and are adopted by reference in Rule 62-204.800, F.A.C. No other method(s) may be used unless prior written approval is received from the Department. [Rules 62-204.800 and 62-297.100, F.A.C.; and Appendix A of 40 CFR 60] Comments: Transitions Optical performed only the Method 9 test. There was no other method or methods performed. B.7. Operating Requirements During Testing - Each compliance test shall be conducted with all the pieces of equipment located in each area operating simultaneously, if practical. If it is impractical to test with all pieces of equipment operating simultaneously the emissions unit may be tested with fewer pieces of equipment operating. In this case, subsequent operation is limited to the number of pieces of equipment (or fewer) operating during the test until a new test is conducted. Once the unit is so limited, operation with a greater number of pieces of equipment is allowed for no more than 15 consecutive days for the purpose of additional compliance testing to regain the authority to operate with the higher number. In no case shall the maximum number of pieces of equipment that are: (1) located in each lens shaping area or (2) operating simultaneously exceed their associated maximum number as shown in Specific Condition No. B.1., above. Failure to submit: (1) a statement identifying which baghouse was tested; (2) the number of pieces of equipment that were operating simultaneously during the test; and (3) the number of pieces of equipment located in each lens shaping area tested may invalidate the test and fail to provide reasonable assurance of compliance. [Rules 62-297.310(2) and 62-4.070(3), F.A.C.] Comments: - During the VE test only 4 pieces of equipment were simultaneously in operation. - The facility limits itself to using only 4 pieces of equipment based on a review of its daily records during the compliance year. - Transitions Optical identified the south dust collector (EU 002) was used during the test for the lens shaping area #1. - A copy of the VE testing record is attached to the compliance file. B.8. Test Notification - The permittee shall notify the Compliance Authority in writing at least 15 days prior to any required tests. The notification must include the following information: the date, time, and location of each test; the name and telephone number of the facility’s contact person who will be responsible for coordinating the test; and the name, company, and the telephone number of the person conducting the test. [Rules 62-4.070(3) and 62-297.310(7)(a)9., F.A.C.] Comments: Environmental Engineering Consultants, Marvin Scott, sent the notification to Air Quality on June, 21, 2010, 15 days prior to the test. B.9. Notification of Operation Commencement - The permittee shall notify the Compliance Authority in writing within 15 days of the date of commencing operation of the Dust-Hog Supra-Blast Dust Collector associated with Emission Unit No. 009. [Rule 62-4.070, F.A.C.] Comments: N/A. Question pertains to EU 009. B.10. Test Reports - The permittee shall prepare and submit reports for all required tests in accordance with the requirements specified in Appendix D (Common Testing Requirements) of this document. [Rule 62-297.310(8), F.A.C.] Comments: The facility submitted the VE test prior to the expired permit. It meets the provisions of Appendix D. B.11. Recordkeeping Requirements -The permittee shall comply with the following: a. If the most recent visible emission test for Emission Unit No. 002 or Emission Unit No. 009 is not conducted with its associated maximum allowable pieces of equipment operating, then the permittee shall daily record the maximum number of pieces of equipment operating simultaneously for that emission unit. b. In order to ensure the 2 baghouses associated with each emission unit are not operating simultaneously, the permittee shall daily record for each emission unit, which baghouse was operating. [Rule 62-4.070(3), F.A.C.; Construction Permit 1030209-026-AC] Comments: Transitions Optical did not test with the maximum 20 pieces of equipment. Therefore, the company daily records the pieces of equipment operating simultaneously and the particular dust collector in use. The backup and primary dust collectors are never simultaneously in use. B.12. Operation and Maintenance (O & M) Plan for Particulate Matter Control - For air pollution control devices and collection systems, the permittee shall maintain and implement an O & M Plan to include a schedule for the maintenance and inspection of each control device, collection systems, and auxiliary equipment. Records of inspections, maintenance, and performance data of control devices and auxiliary equipment shall be retained by the emissions unit for a minimum of two (2) years and shall be made available to the Compliance Authority upon request. The performance parameters shall include operating rates and efficiencies. Such parameters and records shall include, at a minimum, that shown below. a. Performance Parameters: 1. The operating parameters of each baghouse. 2. The timetable for the routine maintenance of each baghouse as specified by the manufacturer. 3. The timetable for routine periodic observations of each baghouse sufficient to ensure proper operation. 4. A list of the type and quantity of required spare parts for each baghouse, which are stored on the premises. b. Operation and Maintenance Log: 1. When maintenance and observations were performed. 2. What maintenance and observations were performed. 3. Who performed said maintenance and observations. 4. Acceptable parameter ranges for each operational check. [Rule 62-4.070(3), F.A.C. and Pinellas County Code, Sect. 58-128] Comments: The facility’s O&M plan is attached to the compliance file for the Dust Hog Dust Collector. Specifically, the O&M plan contains discussion of the daily, semi and annual observation records that must be maintained by Transitions Optical. A copy of the daily log is attached to the compliance file. |
|  |  |  | #### Valid Permit[Rule 62-210.300] |
|  |  |  | #### Changes to Facility/emission unit [Rule 62-210.300] Does the emission unit description above match what the facility is operating (Number of emission units or points, model number and serial number, etc.) Yes No Comments: |

|  |  | C. Other: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Pollution Prevention Activities - P2 Handouts Provided: P2 Brochure; P2 Manual; P2 Checklist - Have any emissions reductions occurred Yes / No PM emissions continue to be at 0% opacity Chemical Substitution; Equipment Changes; Process Changes Chemical/Material Reuse; On-site Recycling; Other: Comments: |  |  |  |  |  |  |  |
| Closing Conference: |  |  |  |  |  |  |  |
| Inspector(s): Jeffrey Morris, Pinellas County, Air Quality Division |  |  |  |  |  |  |  |
| Signature(s): Date: |  |  |  |  |  |  | Date: |
|  | # ACCESS? |  |  | # ARMS? |  |  |  |

| AIRS ID 1030209 | OWNER Transitions Optical, Inc. |  | FACILITY NAME Transitions Optical, Inc. |
| --- | --- | --- | --- |
| TITLE V |  | SYNTHETIC MINOR | DATE OF THIS FCE January 17, 2012 |
| TITLE V MEGA-SITE* |  | OTHER | DATE OF LAST FCE September 26, 2008 |

*Facility with a large number of complex emissions units.  It is more reasonable to evaluate a Title V Mega-Site once every 3 years instead of once every 2 years.

Review of All Required Reports

| PERIODIC REPORTS |  | COMMENTS |
| --- | --- | --- |
| DONE N/A | Annual operating report | submitted on 3/15/11 and reviewed on 4/5/11 |
| DONE N/A | Statement of compliance | copy of chemical usage; maintenance logs |
| DONE N/A | Annual |  |
| DONE N/A | Semi-annual |  |
| DONE N/A | Quarterly |  |
| DONE N/A | Other : |  |
| DONE N/A | Other : |  |

| CONTINUOUS EMISSION MONITOR REPORTS |  | COMMENTS |
| --- | --- | --- |
| DONE N/A | Quarterly excess emissions |  |
| DONE N/A | Semi-annual |  |
| DONE N/A | RATA |  |
| DONE N/A | CGA |  |
| DONE N/A | Other : |  |
| DONE N/A | Other : |  |

Assessment of Control Device and Process Operating Conditions

| OFF-SITE ASSESSMENT |  | (Describe the off-site assessment in comments) |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ON-SITE ASSESSMENT |  | (Document the on-site inspection below) |  |  |  |
| DATE OF INSPECTION | DATE OF INSPECTION REPORT |  | My office maintains the inspection report... |  |  |
|  |  |  | ...in the ARMS database through EASIIR. | ...with the paper or electronic compliance files | ...in another location (specify). |
| Jan 17, 2012 | Jan 19, 2012 |  |  |  |  |
| COMMENTS |  |  |  |  |  |

Review of Tests and Records

| TESTS, OBSERVATIONS AND RECORDS |  | COMMENTS |
| --- | --- | --- |
| DONE N/A | Visible emission observation(s) | performed 7/6/10 |
| DONE N/A | Review of facility records and logs | copy of chemical usage; maintenance logs |
| DONE N/A | Assessment of process parameters (feed rates, process rates, raw material compositions, etc.) |  |
| DONE N/A | Assessment of control equipment performance parameters (water flow rates, pressure drops, temperatures, ESP power levels, etc.) |  |
| DONE N/A | Stack test(s) |  |
| DONE N/A | Other : |  |
| DONE N/A | Other : |  |

Compliance Monitoring (CM) Information

| CM ELEMENT | My office maintains this information... |  |  |  |
| --- | --- | --- | --- | --- |
|  | ...electronically, in the ARMS database. | ...in the permit. | ...in the inspection report. | ...in another location (specify). |
| Facility information |  |  |  |  |
| Applicable requirements |  |  |  |  |
| Inventory of emission units |  |  |  |  |
| Enforcement history |  |  |  |  |
| Compliance activities |  |  |  |  |
| Findings and recommendations |  |  |  |  |
| COMMENTS |  |  |  |  |

Other Comments

Prepared by:	   Jeffrey Morris		Date:

Reviewed by:			Date:

1.  Complete the form as much as possible during for inspection; 2.  The remainder of checklist, for activities in the FY, are filled in no later than 9/15;  3.  Between 9/15 and 9/30, complete the FCE by closing out the FCS project in ARMS.  The INSP > FCS record only accepts a completion date and a comment; it does not accept information on the lower portion of the form.  If an FCS project already exists for the fiscal year in question, enter a completion date and comment in the existing record, do not create a duplicate FCS record.