| 1030209 75715 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FACILITY: Transitions Optical, Inc. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | PERMIT ID: 1703 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | DISTRICT: Southwest |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ADDRESS: 9251 Belcher Rd. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CONTACT PHONE: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Pinellas Park, FL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 727-545-0400 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ARMS NO: 1030209 009 |  |  |  |  |  |  |  | PERMIT NO: |  |  |  |  |  |  |  |  |  |  |  | Expiration Date: 12/22/2010 Renewal Date: 10/23/2010 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | 1030209-027-AF & 1030209-026-AC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Test Due Date: 9/8/2010 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EMISSION UNIT DESCRIPTION : Lens Shaping Area 3: Can consist of up to 6 pieces of equipment such as edgers, grinders, lathes, etc. Controlled by individual hoods and common, Ultra-Conic cartridge filter, Model UC-412 and backup Dust-Hog Supra-Blast Dust Collector, Model SBD8-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| INSPECTION DATE: |  |  |  |  |  |  | ARMS INSPECTION TYPE: |  |  |  |  |  |  |  |  |  |  |  |  |  |  | COMPLIANCE STATUS: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | February 7, 2011 |  |  |  |  |  |  |  |  | INS1 |  |  | INS2 |  |  | INS3 |  |  | FUI |  |  |  |  |  | IN |  |  | MNC |  |  |  |  |  | SNC |  |  |
| INSPECTION TYPE: |  |  |  |  | Initial |  |  |  |  |  | Re-inspection |  |  |  |  |  | Complaint |  |  |  |  |  |  | Drive-by |  |  |  |  |  |  | Quarterly |  |  |  |  |  |
|  |  |  | A. General Review: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1. |  | Permit File Review |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  | No |  |
|  |  | Comments: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2. |  | Introduction and Entry |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  | No |  |
|  |  | Comments: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3. |  | Is the Responsible Official/Authorized Representative still: Vincent Sweeney? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  | No |  |
|  |  | Comments: The Responsible Official/Authorized Representative’s e-mail is: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4. |  | Is the facility contact still: Christine Appel? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  | No |  |
|  |  | Comments: The facility contact’s e-mail is: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| IN | MNC | SNC |  |  | B. Specific Conditions: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|                  |  |  |  |  | Inspection Note: This facility currently is under two permits (1030209-027-AF and 1030209-026-AC) The construction permit Incorporates construction modification permit Project No. 1030209-024-AC dated March 6, 2007 by removing the requirements of Specific Condition No. 13 in construction permit 1030209-021-AC; Extends the expiration date from 01/10/2009 to 01/10/2012; Changes the due date to submit an annual operating report as shown in Specific Condition No. 8 below as required by Rule 62-210.370(3), F.A.C.; Eliminates references to the individual identities and locations of the nine (9) business units associated with Emission Unit No. 001. Comments: The Gemini line is in the R&D phase. The facility representative Mike James was reminded that the facility must notify Pinellas County Air Quality 60 days prior to startup. 3. VOC & HAP Emission Limitations: As requested by the permittee in the application dated November 7, 2003, the facility is allowed to operate 8,760 hrs./hr. and shall not emit more than the following in any consecutive 12-month period: A. 77.1 tons of total VOC's. B. 9.95 tons of any individual HAP. C. 21.69 tons of total HAP's. Permitting Note: Compliance demonstrated with Emission Unit No. 001 below. [Rules 62-210.200 – Definition (PTE) and 62-212.300(1)(d), F.A.C.] per 1030209-027-AF Comments: The compliance year was from Jan, 10’ – Jan, 11’. Based on the records review the highest individual HAP Methanol emissions facility-wide = 5.93 tons/yr (Aug, 10’). The highest total HAP emissions facility-wide =8.97 tons/yr (Mar, 10’). The highest total VOC emissions facility-wide = 29.63 tons/yr (Mar, 10’). A copy of the records is attached to the file. 4. Visible Emission Limitations: Visible emissions from this facility (i.e., boilers, process heater, emergency generator, etc.) shall not be equal to or exceed 20% opacity. [Rule 62-296.320(4)(b), F.A.C.] per 1030209-027-AF Comments: 0% visible emissions observed during the inspection from the Airotech reverse pulse fabric filter dust collector EU 002 and the Ultra-Conic cartridge filter model UC-412 EU 009 during the time of inspection. The 3 optical edgers and a grinder were in operation. The facility is limited to 5 pieces of equipment for EU 009. 5. Objectionable Odor Limitation: The permittee shall not cause, suffer, allow, or permit the discharge of air pollutants that cause or contribute to an objectionable odor. An "Objectionable Odor" is defined as any odor present in the outdoor atmosphere, which by itself or in combination with other odors, is or may be harmful or injurious to human health or welfare, which unreasonably interferes with the comfortable use and enjoyment of life or property, or which creates a nuisance. [Rule 62-296.320(2), F.A.C. and Pinellas County Code, Section 58-178] per 1030209-027-AF Comments: No objectionable odor detected during the inspection 6. Work Practices: All equipment, pipes, hoses, lids, fittings, etc. shall be operated/maintained in such a manner as to minimize leaks, fugitive emissions, and spills of materials containing VOCs/OSs/HAPs. [Rule 62-296.320(1)(a), F.A.C.] per 1030209-027-AF Comments: The facility followed its work practices. There was no observance of leaks from the 9 step monomer or polycarbonate lens coating process which involves the use of Methanol and Glycol Ethers. No observed leaks with the surface coating chamber. 7. Special Testing Requirement: Should the Department or Pinellas County Department of Environmental Management (PCDEM), after investigation, have good reason (such as complaints or increased visible emissions) to believe any applicable emission standard contained in a Department rule (or in a permit issued pursuant to those rules) is being violated, it may require the owner or operator of the emissions unit to conduct compliance tests which identify the nature and quantity of pollutant emissions from the emissions unit and to provide a report on the results of said tests to the Department and PCDEM. [Rule 62-297.310(7)(b), F.A.C.] per 1030209-027-AF Comments: No special testing required during the compliance year 8. Annual Operating Report (AOR) Requirement: On or before April 1 of each year, submit to the Air Compliance Section of the Department's Southwest District Office and the PCDEM, an Annual Operating Report [DEP Form No. 62-210.900(5)] for the preceding calendar year. Note, the annual operating report for calendar year 2008, shall be submitted by May 1, 2009. The report may be submitted electronically in accordance with the instructions received with the AOR package sent by the Department, or a hardcopy may be submitted. [Rule 62-210.370(3), F.A.C.] per 1030209-027-AF Comments: 09’ AOR received by Air Quality on 3/22/10. This submittal was before the April 1 deadline 9. Operating Permit Application Requirements: In order to incorporate the modifications and changes addressed in this permit into an operating permit, which are expected to occur over approximately a 5 year period: A. Submit* an operating permit application by July 13 of each year following the date of January 18, 2006, for the modifications and changes completed during each 12-month period of May 29 – May 28, until all the modifications and changes are completed. The first application is due July 13, 2006. [Rules 62-4.090 and 62-4.070(3), F.A.C.] per 1030209-026-AC AND 9. Operating Permit Renewal: An application for renewal of this operation permit shall be submitted to the Air Permitting Section of the Southwest District Office of the Department and one application shall be submitted to the PCDEM prior to 60 days before the expiration date of this permit… [Rules 62-4.090, 62-4.070(3) & 62-210.300(2), F.A.C.] per 1030209-027-AF Comments: This facility currently is under two permits (1030209-027-AF and 1030209-026-AC) The construction permit incorporates construction modification permit Project No. 1030209-024-AC dated March 6, 2007 by removing the requirements of Specific Condition No. 13 in construction permit 1030209-021-AC. The facility is aware that it will need to apply for an operation permit 60 days prior to the expiration of the permit. The facility is also aware that they will need to notify 60 days to startup of the Gemini Line. Emission Unit No. 009 C.1. Permitted Capacity: The permittee shall comply with the following: A. A maximum of 6 pieces of equipment, such as edgers, grinders, lathers, etc., shall be allowed to be in Lens Shaping Area No. 3 and in operation at any one time. B. The 2 dust collectors shall not operate simultaneously. [Rule 62-210.200(PTE), F.A.C.; 1030209-021-AC] per 1030209-027-AF - The facility limits itself to only 5 pieces of equipment. determined from the VE test performed on 7/6/10. Only 3 optical edgers and a grinder were in operation - Ultra-Conic cartridge filter, Model UC-412 was in operation for EU 009. The Dust-Hog Supra-Blast Dust Collector, Model SBD8-2 is used as backup 17. Notification of New Baghouse Operation: Within 5 days of placing the new Ultra-Conic Cartridge Filter Dust Collection System in operation, submit a written notification to the Air Compliance Section of the PCDEM stating such date. [Rule 62-4.070(3), F.A.C.] per 1030209-026-AC Comments: The facility has not installed or has plans to install a new Ultra-Conic cartridge filter dust system at this time. 18. Visible Emission Test Requirement: Test the new Ultra-Conic Cartridge Filter Dust Collection System's emissions for visible emissions within 30 days of being placed in operation. [Rule 62-297.310(7)(a)(1), F.A.C.] per 1030209-026-AC Comments: The facility has not installed or has plans to install a new Ultra-Conic cartridge filterdust system at this time. However, it is aware that it will need to perform a VE test within 30 days of startup operation Common Conditions for Emission Unit Nos. 002 & 009 D.1. Visible Emission Limitation: Visible emissions from the Lens Shaping Areas particulate control devices shall not be equal to or exceed 20% opacity. [Rule 62-296.320(4)(b), F.A.C.] per 1030209-027-AF Comments: There were no emissions from either the Airotech reverse pulse fabric filter dust collector, Model 5P-130-8-(10) BA associated with EU 002 and Ultra-Conic cartridge filter, Model UC-412 EU 009 during the time of inspection. D.2. Circumvention Limitation: No person shall circumvent any air pollution control device, or allow the emission of air pollutants without the applicable air pollution control device operating properly. As an indicator the particulate control devices are operating properly, the Department has determined there should be no visible emissions (5% opacity) from each device. Visible emissions in excess of 5% is not considered a violation in and of itself, but an indicator of improper operation and may indicate a violation of Rule 62-210.650, F.A.C. [Rules 62-210.650 and 62-4.070(3), F.A.C.] per 1030209-027-AF Comments: There was no evidence of circumvention during the inspection D.3. Testing Frequency: Each of the 4 dust collectors associated with Emission Unit Nos. 002 and 009 shall be tested for visible emissions at least 105 days prior to and no more than 365 days prior to the expiration date of this permit. Since the Dust-Hog Supra-Blast Dust Collector associated with Emission Unit No. 009 was not operational prior to the effective date of this permit, this dust collector shall be tested within 30 days after being placed back in operation. The test report(s) shall be submitted within 45 days of testing to the Air Compliance Sections of this office and the PCDEM. A copy of the test report(s) shall be submitted with the application to renew this permit. [Rules 62-297.310(7)(a)3. and 62-297.310(8)(b), F.A.C.] per 1030209-027-AF Comments: On 7/6/10 the Airotech reverse pulse fabric filter dust collector, Model 5P-130-8-(10) BA and the Dust-Hog Supra-Blast, Model SBD8-2, cartridge dust collector associated with EU 002 were tested. The Ultra-Conic cartridge filter, Model UC-412 associated with EU 009 was tested. The backup Dust-Hog Supra-Blast Dust Collector, Model SBD8-2 was not tested. The VE test indicated 0% opacity for the 30 minute runs. The test reports were received by Air Quality on 7/7/10. D.4. Test Methods: Compliance with Specific Condition No. D.1. shall be determined using EPA Method 9, contained in 40 CFR 60, Appendix A, and adopted by reference in Chapter 62-204, F.A.C. The visible emission test shall be conducted by a certified observer and be a minimum of 30 minutes in duration. Minimum requirements for stationary point source emission test procedures and reporting shall comply with Chapter 62-297, F.A.C. and 40 CFR 60, Appendix A. [Chapters 62-4, 62-204, and 62-297, F.A.C. and 40 CFR 60, Appendix A] per 1030209-027-AF Comments: The facility was tested by Marvin Scott of AeroMet Engineering who is a certified observer. |  |  |  |  |  |
|  |  |  |  |  | #### Valid Permit [Rule 62-210.300] |  |  |  |  |  |
|  |  |  |  |  | #### Changes to Facility/emission unit [Rule 62-210.300] Does the emission unit description above match what the facility is operating (Number of emission units or points, model number and serial number, etc.) Yes No Comments: |  |  |  |  |  |
|  |  |  | C. Other: |  |  |  |  |  |  |  |
| Pollution Prevention Activities Comments: Chemical Substitution The facility has is beginning to substitute ethanol for methanol in its lens coating production. The initial substitution began in Dec, 10’.[jm] |  |  |  |  |  |  |  |  |  |  |
| Inspector(s): Jeffrey Morris, Pinellas County, Air Quality Division |  |  |  |  |  |  |  |  |  |  |
| Signature(s): Date: |  |  |  |  |  |  |  |  | Date: |  |
| # ACCESS? |  |  |  |  |  | # ARMS? |  |  |  |  |