| 1030218 80595 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FACILITY: M C Graphics, Inc. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | PERMIT ID: 138 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Modern Graphic Arts, A Sandy Alexander Company |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | DISTRICT: Southwest |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ADDRESS: 1527 102nd Ave. North |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CONTACT PHONE: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| St. Petersburg, FL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 727-579-1527 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ARMS NO: 1030218 002 |  |  |  |  |  |  |  | PERMIT NO: |  |  |  |  |  |  |  |  |  |  |  | Expiration Date: 1/3/2012 Renewal Date: 11/4/2011 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | 1030218-012-AF |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Test Due Date: 9/15/2000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EMISSION UNIT DESCRIPTION : Printing Line Nos. 2 & 3: Harris Model M-110 heat-set press and Heidelberg Harris, Model M600 heatset press, controlled by TEC Systems Quantum 4000 catalytic incinerator |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CMS INSPECTION DATE: |  |  |  |  |  |  | ARMS INSPECTION TYPE: |  |  |  |  |  |  |  |  |  |  |  |  |  |  | COMPLIANCE STATUS: |  |  |  |  |  |  |  |  |  |  |  |  |
|  | January 4, 2012 |  |  |  |  |  |  |  |  | INS1 |  |  | INS2 |  |  | INS3 |  |  | FUI |  |  |  |  |  | IN |  |  | MNC |  |  |  |  | SNC |  |
| INSPECTION TYPE: |  |  |  |  | Initial |  |  |  |  |  | Re-inspection |  |  |  |  |  | Complaint |  |  |  |  |  |  | Drive-by |  |  |  |  |  | Quarterly |  |  |  |  |
|  |  |  | A. General Review: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1. |  | Permit File Review |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  | No |
|  |  | Comments: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2. |  | Introduction and Entry |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  | No |
|  |  | Comments: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3. |  | Is the Responsible Official/Authorized Representative still: Jeffery Michael? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  | No |
|  |  | Comments: The Responsible Official/Authorized Representative’s e-mail is: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4. |  | Is the facility contact still: Sharon Pratt? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  | No |
|  |  | Comments: The facility contact’s e-mail is: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| IN | MNC | SNC | B. Specific Conditions: |
| --- | --- | --- | --- |
|                                              |  |  | Permitting note: The effective Date of this permit is 1/3/07 This permit or a copy thereof shall be kept at the work site of the permitted activity. [62-4.160 F.A.C. - General Condition 12.] Comments: The facility did did not have a copy of the permit on-site. 4. Facility VOC & HAP Emission Limitations – Volatile Organic Compound (VOC) emissions and Hazardous Air Pollutants (HAP) emissions which include the stack and fugitive emissions from the printing inks, fountain solutions, wash solutions, natural gas combustion, and miscellaneous VOC containing materials from this facility shall not exceed the following levels. Pollutant; Facility-Wide Emission Limits (tons/consecutive 12 month period). Total Volatile Organic Compounds (VOC); 64.48. Total Hazardous Air Pollutants (HAP); 5.76. [Rule 62-210.200 (203)(Potential to Emit), F.A.C.; 1030218-011-AC - to stay below the Title V major level] Comments: The highest VOC emissions = 0.24 tons/yr (May, 11') and the highest HAP emissions = 0.01 tons/yr (May, 11'). 5. Maximum VOC Loading Rate Limitations – VOC loading rates shall not exceed the following: Emission Unit No.; Maximum VOC Loading Rate Limits (pounds/hour). 001; 47.2. 002; 62.7. 005; 65.9. Comments: During the 9/7/11 Method 25A test, the facility tested at a VOC loading rate of 140.90 lbs/hr. EU 002 is limited to only 62.7 lbs/hr during testing conditions. The facility was in violation. 6. Destruction Efficiency –The VOC destruction efficiency of each catalytic incinerator and thermal oxidizer shall not fall below 90% removal. [Rules 62-210.650, 62-296.320(1)(a), & 62-212.300, F.A.C.; 1030218-011-AC] Comments: Based on the test results, the destruction efficiency for EU 002 (printing line #2 and #3) = 99.18%. 7. Blanket Wash Solution and Meter & Roller Cleaner Operation - During all periods when Blanket Wash Solution and Meter & Roller Cleaner are being utilized, the corresponding catalytic incinerator or thermal oxidizer shall be operating for the solution or cleaner being utilized. [Rule 62-4.070(3), F.A.C.; 1030218-010-AF and 1030218-011-AC] Comments: Based on the records review, the catalytic incinerator was operating at all times for EU 002 when the blanket wash/roller cleaning was in operation. A copy of the maintenance log is attached to the compliance file. 8. General Particulate Emission Limiting Standards: General Visible Emissions Standard - Except for emissions units that are subject to a particulate matter or opacity limit set forth or established by rule and reflected by conditions in this permit, no person shall cause, let, permit, suffer or allow to be discharged into the atmosphere the emissions of air pollutants from any activity, the density of which is equal to or greater than that designated as Number 1 on the Ringelmann Chart (20 percent opacity). EPA Method 9 is the method of compliance pursuant to Chapter 62-297, F.A.C. [Rules 62-296.320(4)(b)1 & 62-296.320(4)(b)4, F.A.C.] Comments: A visible emissions test was performed during the Method 25A test. The opacity was 0% during the 30 minute run and the test results are with the report. 9. General Pollutant Emission Limiting Standards. Objectionable Odor Prohibited. No person shall cause, suffer, allow or permit the discharge of air pollutants which cause or contribute to an objectionable odor. An objectionable odor is any odor present in the outdoor atmosphere which by itself or in combination with other odors, is or may be harmful or injurious to human health or welfare, which unreasonably interferes with the comfortable use and enjoyment of life or property, or which creates a nuisance. [Rules 62-296.320(2) & 62-210.200 (Definitions), F.A.C.; Pinellas County Code, Section 58-178] Comments: There were no objectionable odors detected during the inspection and no odor complaints during the compliance year. 10. Circumvention of Control Equipment - The permittee shall not circumvent any air pollution control device or allow the emissions of air pollutants without the applicable air pollution control device (catalytic incinerator/thermal oxidizer) operating properly and in accordance with the applicable Operation and Maintenance (O&M) Plan as required by Specific Condition No. 13. - Printing Line No. 1 and Printing Line Nos. 2 and 3 shall not operate unless the associated catalytic incinerator used to control emissions is operational and the catalyst bed inlet temperature is greater than 550o F, and is less than 1,100o F. - Printing Line No. 4 shall not operate unless the thermal oxidizer is in service and operating at a combustion zone temperature of 1,400.0 F or greater. (See Specific Condition No. 31 for temperature monitoring requirements.) [Rules 62-4.070(3) & 62-210.650, F.A.C.; 1030218-011-AC] Comments: There was no observed circumvention of control equipment during the inspection. The temperature chart recording was 973 deg F and meets the condition requirements. 11. General Pollutant Emission Limiting Standards – Volatile Organic Compounds Emissions or Organic Solvent Emissions - The permittee shall allow no person to store, pump, handle, process, load, unload or use in any process or installation, volatile organic compounds (VOC) or organic solvents (OS) without applying known and existing vapor emission control devices or systems deemed necessary and ordered by the Department. Work practices to minimize emissions shall include the following: A. Equipment, pipes, hoses, lids, fittings, etc., shall be operated/maintained in such manner as to minimize leaks, fugitive emissions & spills of solvent materials. This includes draining all VOC containing fluids into closed containers during lengthy periods of time when a press is not operating; B. All VOC/OS from washings (equipment clean-up) shall be placed into containers that prevent evaporation into the atmosphere; C. Cover or close all VOC containers when they are not in use; D. Prevent excessive air turbulence across exposed VOC/OS; E. Immediately confine and clean up VOC/OS spills and make sure wastes are placed in closed containers for reuse, recycling or proper disposal. [Rules 62-4.070(3) & 62-296.320(1)(a), F.A.C.] Comments: - No leaks observed from press #2 and #3. - All VOC cleaning solvents, fountain solutions and inks/coatings are drained into small sealed containers. - MC Graphics ensures the prevention of excessive air turbulence by verifying closed solvent containers - Solvent washings from equipment clean-up are stored in sealed containers. 12. Excess Emissions. - Excess Emissions resulting from startup, shutdown or malfunction of any emissions unit shall be permitted providing (1) best operational practices to minimize emissions are adhered to and (2) the duration of excess emissions shall be minimized but in no case exceed two hours in any 24 hour period unless specifically authorized by the Department for longer duration. - Excess Emissions which are caused entirely or in part by poor maintenance, poor operation, or any other equipment or process failure which may reasonably be prevented during startup, shutdown, or malfunction shall be prohibited. [Rule 62-210.700(1) & (4), F.A.C.] Comments: There were no excess emissions observed during the inspection and none reported by the facility during the compliance year. The facility follows best operational practices if there is an upset condition. 13. Operation & Maintenance Plan for Incinerators – The integrated dryer/thermal oxidizer and each catalytic incinerator shall be operated and maintained in accordance with the Operation and Maintenance (O&M) Plan that was submitted January 10, 2005 as part of the renewal/revision application. The O&M Plan shall include but not be limited to the following: A. The operating parameters for each incinerator; B. The timetable for routine incinerator maintenance as specified by the manufacturer; C. The timetable for routine weekly, bi-weekly, or monthly incinerator observations sufficient to ensure proper operation; D. A list of the type and quantity of incinerator spare parts which are stored on the premises; E. A record log which indicates, at a minimum: 1. When incinerator maintenance and observations were performed; 2. What incinerator maintenance and observations were performed; 3. Who performed said maintenance and observations; 4. Acceptable parameter ranges for each operational check. F. Submit the revised O & M Plan per PCDEM Consent Order A06-002, within thirty (30) days of the effective date of this permit. The O&M plan documentation logs shall be maintained at the facility for a minimum of the most recent three-year period and shall be made available to Air Quality Division of the PCDEM, or the Department, upon request. [Rule 62-4.070(3), F.A.C.; Pinellas County Code, Sec. 58-128] Comments: The facility follows the provisions of the condition. A copy of the Quantum 4000 maintenance log is attached to the inspection report. During the compliance year the facility repaired the heat indicator light on 12/21/11. 14. Bypass Stacks - The permittee shall not utilize a bypass stack which vents to the atmosphere at any time while ink is being applied or blanket wash solution is being utilized or meter & roller cleaner is in operation. If ink is not being applied and best operational practices to minimize emissions are adhered to, then it is permitted to utilize a bypass stack only during periods of start up, shutdown, or malfunction. The permittee shall keep records of the date and time of commencement and completion of each time period that a bypass stack is utilized. The records shall include the reason for utilization of the bypass stack. They shall be maintained at the facility for a minimum of the most recent three year period and made available to Air Quality Division of the PCDEM, or the Department, upon request. [Rules 62-210.700 and 62-4.070(3), F.A.C.; 1030218-011-AC] Comments: There is no bypass stack associated with the catalytic incinerator 15. Incinerator and Thermal Oxidizer Flow/Velocity Compliance Testing – Annually, the permittee shall test the incinerator stack outlet on Printing Line No. 1, the common incinerator stack outlet on Printing Line Nos. 2 and 3, and the thermal oxidizer exhaust stack of Printing Line No 4 for stack gas flow rate, stack gas temperature and stack gas velocity, within 60 days prior to the due date of September 15. A copy of the test data shall be submitted to the Air Quality Division of the PCDEM and the Air Compliance Section of the Southwest District Office of the Department within 45 days of testing. [Rules 62-4.070 & 62-297.310(7), F.A.C.] Comments - The facility tested for the stack gas flow rate, stack gas temperature and stack gas velocity during the Method 25A test. Results are with the report. - The test was performed on September 7, 2011. The test was performed 8 days prior to the due date of September 15. - The test report was received by Air Quality on September 28, 2011 and a copy was sent to the Southwest District Office. The test was received 21 after the test date September 7, 2011. 16. Incinerator and Thermal Oxidizer Flow Velocity Compliance Test Method - Compliance with the testing requirements of Specific Condition No. 15 shall be demonstrated using EPA Methods 1 and 2. The sampling time for the EPA Method 2 test shall be at least 60 minutes in duration. [Rules 62-297.401 & 62-297.310(7), F.A.C.] Comments: The facility performed Methods 1 and 2. They were performed on 9/7/11 in conjunction with the Method 25A test. A copy of the results is in the report. The Method 2 sampling time was 60 minutes. 17. Incinerator and Thermal Oxidizer VE Compliance Testing – Annually, the permittee shall test the incinerator exhaust stack on Printing Line No. 1, the common incinerator exhaust stack on Printing Line Nos. 2 and 3, and the thermal oxidizer exhaust stack of Printing Line No. 4 for visible emissions (VE), within 60 days prior to the due date of September 15. A copy of the test data for each exhaust stack shall be submitted with the permit renewal package to the Air Quality Division of the PCDEM and the Air Compliance Section of the Southwest District Office of the Department within 45 days of testing. [Rules 62-4.070 & 62-297.310(7), F.A.C.] Comments: The visible emissions test was performed on 9/7/11 - EU 002. The test was performed 8 days prior to the due date. The test copy was sent as a part of the permit renewal to both the Air Quality Division and FDEP on 9/28/11. 18. Incinerator and Thermal Oxidizer VE Compliance Test Method - Compliance with the visible emission limitations of Specific Condition No. 8 shall be demonstrated using EPA Method 9. The visible emissions test shall be conducted by a certified observer and be a minimum of thirty (30) minutes in duration and include the period during which the highest opacity emissions can reasonably be expected to occur. Testing of emissions shall be conducted while each related printing line or lines are operating under normal conditions. The visible emission test shall be conducted during each incinerator VOC destruction efficiency stack test (see condition below). [Rules 62-4.070(3) & 62-297.310(4)(a)(2), F.A.C.] Comments: - The VE test followed EPA Method 9 requirements and met the General Visible Emissions Standard of 20% under Specific Condition #8. - The test was for 30 minutes and performed by certified observer, Ken Givens of Air Testing and Consulting. - The highest 6 minute avg = 0% with the printing operation under normal conditions. 19. Incinerator and Thermal Oxidizer Destruction Efficiency Compliance Testing – Once prior to permit renewal, the permittee shall test the incinerator stack inlet and outlet on Printing Line No. 1, the common incinerator stack inlet and outlet on Printing Line Nos. 2 & 3, and the thermal oxidizer stack inlet and outlet on Printing Line No. 4 to determine VOC destruction efficiency, within 60 days prior to the due date of September 15. A Method 9 VE test shall be performed simultaneously with one of the three Method 25/25A runs required in order to determine VOC destruction efficiency. A copy of the test data shall be submitted to the Air Quality Division of the PCDEM and the Air Compliance Section of the Southwest District Office of the Department within 45 days of testing. (See Specific Condition No. 13 for operating parameters to be included in the test report.) The results of the destruction efficiency test shall be used to demonstrate compliance of the emission limitations for Printing Lines 1, 2, 3 and 4 as required per Specific Condition No. 4. [Rule 62-297.310, F.A.C.] Comments: - Method 25A test was performed prior to the permit renewal. - Based on the test, the VOC destruction efficiency was 99.18%. A 30 minute VE test was performed simultaneously during run #2. - The Method 25A and VE test was submitted to Air Quality FDEP Southwest District Office 21 days after September 7, 2011. - The results of Method 25A was used to demonstrate compliance with the emission limit. 20. Incinerator and Thermal Oxidizer VOC Destruction Efficiency Test Methods and Procedures - Compliance with the VOC destruction efficiency requirement of Specific Condition No. 6 for Printing Line Nos. 1, 2, and 3 shall be conducted using EPA Methods 1, 2, 3, 4, and 25 or 25A. Compliance with the VOC destruction efficiency requirement of Specific Condition No. 6 for Printing Line No. 4 shall be conducted by the methods and procedures described in Attachment 2 - Alternative Performance Testing Procedure for Printing Line No. 4. During the compliance test, the VOC content of the ink shall either be determined by using data from the most recent MSDS or an EPA Method 25A test. Any alternative method for determining VOC content may only be used after written approval is obtained from the Department or the PCDEM. [Rules 62-4.070 (3), 62-297.310 & 62-297.401, F.A.C.] Comments: - Destruction efficiency = 99.18 %. It did not fall below 90%. - The facility used Methods 1, stack velocity traverses, Method 2, stack gas velocity and volumetric flow rate, Method 3, determination of dry molecular weight and Method 4, determination of moisture content in the stack gas. - The VOC content of the ink is established from the Method 25A test and there was no alternative method used during this test. 21. Compliance Testing Notification - The permittee shall notify the Air Quality Division of the PCDEM at least 15 days prior to the date on which each compliance test is scheduled, of the date, time, and place of each such test, and the test contact person who will be responsible for coordinating each test. [Rule 62-297.310(7)(a)9., F.A.C.] Comments: The Method 25A notification was received by Air Quality 23 days prior to the test on 9/7/11. 22. Press Line Operation During Testing – VOC destruction efficiency testing shall be accomplished while operating 90 - 100% of the maximum VOC loading rate of each Emission Unit as listed in Specific Condition No. 5 (defined as each printing press in operation under normal conditions printing products that could be expected to result in high VOC inlet loading to the associated catalytic incinerator or thermal oxidizer). During testing, the maximum permitted VOC loading rate shall not be exceeded. If it is not practical to test at the maximum VOC loading rate, then a test may be conducted at less than the maximum VOC loading rate. In this case, subsequent operation in a production mode is then limited to 110 percent of the test rate(s) until a new test is conducted. Once the source is so limited, then operation at a higher VOC loading rate is only allowed for no more than 15 consecutive days for the purpose of additional compliance testing in order to regain the maximum VOC loading rate in this permit. Acceptance of a test by the Department of Environmental Protection will automatically amend this permit to the new VOC loading rate, plus 10%, but shall not exceed the maximum VOC loading rate. Failure to submit the actual rates during the test or operating at conditions which do not reflect normal operating conditions, may invalidate the test. [Rules 62-4.070(3), 62-297.310(2) & (8), F.A.C.] Comments: During the Method 25A test, the facility was in violation of exceeding its loading rate. This was noted in the 9/7/11 inspection report. The inspection notes from 9/7/11: Based on a review of the Method 25A test, the maximum VOC loading rate for EU 002 was exceeded during the test. The permitted maximum VOC loading rate is 62.7 lbs/hr for EU 002. The facility tested at 140.90 lbs/hr. The condition states, the VOC loading rate is not to be exceeded during the test and therefore, the facility is in violation of the condition. 23. Compliance Test Report – Test reports shall be submitted to the Air Quality Division of the PCDEM and the Air Compliance Section of the Southwest District Office of the Department within 45 days of testing. The report shall include the information required under Rule 62-297.310(8)(c), F.A.C. including the operating status of each of the presses, the incinerator or thermal oxidizer exhaust gas temperature, the thermal oxidizer combustion zone temperature and/or the change in temperature across the catalyst bed of each catalytic incinerator (include copy of continuous recorder chart/data which includes the test period), the stack gas flow rate, the stack gas velocity, and (for VOC destruction efficiency testing) the inlet and outlet VOC concentrations determined during the testing and used to calculate destruction efficiency. The data on the VOC content in each ink being used during the testing shall be submitted with the compliance test results. The amount of each ink used during the compliance test shall be determined using available meters or other estimating information and shall be submitted with the compliance test report. [Rules 62-4.070(3) & 62-297.310(8)(c), F.A.C.] Comments: The test report included the required information under 62-297.310(8)(c). - Operating status of printing press line #2 and #3 - Stack temperature - Flow rate - Stack gas velocity - Inlet and Outlet VOC concentrations - % VOC Content of the ink - Amount of ink used in lbs 24. Material Usage and VOC/ HAP Emission Records – In order to document compliance with the VOC and HAP limitations of Specific Condition No. 4, monthly VOC/HAP material usage and emission records shall be kept for this facility. The logs shall include the following at a minimum: A. Facility Name, Facility ID No. (1030218), and Emission Unit ID No. (i.e., 001); B. Day, Month and Year; C. Quantity of "As Applied" ink (by name or identification number) used, indicating the amount of each used (gallons or pounds); D. VOC and HAP content (as applied) for each type of ink utilized; (If gallons are recorded for usage, content shall be recorded as lbs/gallon. If pounds are recorded for usage, content shall be recorded as percent by weight.) E. A calculation of VOC and HAP emissions (pounds per month) from each ink based on the information in C. and D. above; F. Quantity of other VOC/HAP containing materials (identify material with description) utilized (gallons and /or pounds per month), less disposal and/or reuse; G. VOC and HAP content (as applied) for each of the above materials utilized; (If gallons are recorded for usage, content shall be recorded as lbs/gallon. If pounds are recorded for usage, content shall be recorded as percent by weight.) H. A calculation of VOC and HAP emissions (pounds per month) from the above materials based upon the information in F. and G. above; I. Monthly total of facility VOC and HAP emissions; J. A calculation of the total facility VOC and HAP emissions for the most recent 12 consecutive month period. [Rule 62-4.070(3), F.A.C.; 1030218-011-AC and Pinellas County Code Section 58-90] Comments: MC Graphics meets the provisions of the condition. A copy of the record is attached to the compliance file. 25. Non-VOC Material Substitution - The substitution of a non-VOC/OS containing material for an existing production material containing VOC/OS shall not require recordkeeping, however, a log entry is required at the time of material substitution stating that a "non-VOC solution/solvent has been substituted for (specify material)". [Rule 62-4.070(3), F.A.C.; Pinellas County Code, Section 58-90] Comments: There were no non-VOC material substitutions during the compliance year. 26. Controlled Emission/Calculations Requirement - The controlled emission recordkeeping and required calculations for Printing Line Nos. 1, 2, 3 and 4 used to demonstrate compliance with the requirements of Specific Condition No. 24 shall be based on the specified incinerator or thermal oxidizer VOC destruction efficiencies from compliance testing, effective no later than January 1 of the year following the compliance test. [Rules 62-4.070(3) & 62-209, F.A.C.; 1030218-011-AC and Pinellas County Code, Section 58-90] Comments: - The new destruction efficiency =99.18% was established during the 9/7/11 Method 25A test. - The facility has begun using the new destruction efficiency as a basis for its monthly emission records effective 1/1/12. 27. VOC/HAP Emission Calculations - Controlled emission calculations for printing inks and fountain solutions (see Notes below) shall be based on the most recent tested VOC destruction efficiency for each catalytic incinerator and thermal oxidizer, effective no later than January 1 of the year following the compliance test, unless otherwise notified in writing by the Air Quality Division of the PCDEM or the Department. (VOC/HAP Emission Calculation Notes - Unless modified by the Department or Pinellas County Department of Environmental Management (PCDEM), the following assumptions are made for the purposes of VOC/HAP emission calculations for this facility: Printing Inks: VOC/HAP emissions from printing inks are based upon the assumption that 20% of the VOCs/HAPs from printing inks are retained on the printed product and are not released to the atmosphere. The remaining 80% of the VOC/HAP emissions from printing inks are captured (100% capture efficiency in the dryer due to the air flow and the composition of the inks) and sent to the catalytic incinerators or thermal oxidizer, each having a90% (minimum) destruction efficiency. Fountain Solutions: Fountain Solution is restricted to a non-VOC solution. A "non-VOC" material is defined as a material which indicates no VOC content per an EPA Method 24 test. Wash Solution (Blanket Wash and Meter & Roller Cleaner Solutions): Blanket Wash and Meter & Roller Cleaner solutions have 100% VOC content. The controlled emission limitations for Emission Unit Nos. 001, 002, and 005 are based upon the assumption that 80% of the combined Blanket Wash and Meter & Roller Cleaner Solutions are captured by the press dryers and incinerators with a resulting VOC destruction determined by testing. The emission limit for Emission Unit No. 003 is uncontrolled and represents the remaining 20% of Blanket Wash and Meter & Roller Cleaner Emissions. Combustion VOC Emissions: VOC emissions from natural gas combustion in the dryers, catalytic incinerators and afterburner can be assumed to be equal to 0.42 tons/year (worst case PTE assuming operation of dryers, incinerators and afterburner for 8,760 hours/year at maximum design heat input rate). [Rule 62-4.070(3), F.A.C.; 1030218-011-AC and Pinellas County Code, Section 58-90] Comments: There are no modifications to the FESOP renewal. The emission calculations are based on the presumptions presented in this condition. 28. VOC/HAP Records Documentation - Supporting documentation ["As Supplied", "As Applied" sheets, MSDS (Material Safety Data Sheet), EPA data sheets, purchase orders, etc.] shall be kept for each ink and VOC/OS (organic solvent)/HAP containing material utilized at the facility, including "non-VOC" solutions in use. This documentation shall include sufficient information to: A. Provide reasonable assurance that any "non-VOC" solution/solvent in use at the facility indicates no VOC content based on an EPA Method 24 or 24A test; - Provide the basis for any electronic recordkeeping used for reporting emissions limitations required in Specific Conditions of this permit. [Rule 62-4.070(3), F.A.C.; Pinellas County Code, Section 58-90] Comments: - The supporting documentation was reviewed by Air Quality including MSDS’s and purchase receipts. - The documentation includes the inks, cleaning solutions and fountain solutions purchased. - There are no non-VOC solutions used during the compliance year. 29. Record Maintenance - All recordkeeping information shall be updated at least monthly, with each entry made no later than 15 days after the end of the month. [Rule 62-4.070(3), F.A.C.; Pinellas County Code, Section 58-90] Comments: The records are updated monthly. MC Graphics performs the update the first week of the next month. 30. Record Retention - All recordkeeping information, supporting documentation, monitoring data, and all compliance test information required by this permit shall be retained at the facility for the most recent three (3) year period and shall be made available to the Florida Department of Environmental Protection (the Department) or the Air Quality Division of the Pinellas County Department of Environmental Management (PCDEM), upon request. [Rule 62-4.160(14)(b),F.A.C.; Pinellas County Code, Section 58-90] Comments: Records have been kept by the facility since 2000 and are made available to FDEP and Air Quality. 31. Temperature Monitoring – In order to document ongoing compliance with Specific Condition No. 10, each catalytic incinerator shall be equipped with devices to continuously measure and record the inlet and outlet temperatures and the thermal oxidizer shall be equipped with devices to continuously measure and record the operating temperature during all periods of printing press operation. The thermocouple shall be calibrated at least annually and shall be accurate to within + 5 F. [Rules 62-4.070(3), 62-297.310(4)(d) & (reference Table 297.310-1), F.A.C.] Comments: Air Quality reviewed the latest thermocouple calibration performed in May, 11’. The thermocouple was found to be accurate within +/- 5 deg F 32. Temperature Monitoring Requirements – In order to document compliance with the temperature monitoring requirements of Specific Condition No. 31, copies of the continuous recorder chart/data for each incinerator and thermal oxidizer and the calibration records for the thermocouple shall be retained at the facility for the most recent three year period and shall be made available to the Florida Department of Environmental Protection (the Department) or the Air Quality Division of the Pinellas County Department of Environmental Management (PCDEM), upon request. [Rules 62-4.070(3) & 62-4.160(14)(b), F.A.C.; Pinellas County Code, Section 58-90] Comments: MC Graphics has kept its continuous temperature chart wheel records since 2000. Additionally, the thermocouple calibration data has been kept since 2000. 33. Excess Emissions/Malfunction - A record shall be made of each period that the catalytic incinerator inlet falls below 550F or the outlet exceeds 1,100F or when the thermal oxidizer combustion zone temperature is below 1,400F (this includes periods of printing press operation when the catalytic incinerator or thermal oxidizer are not in service) and shall include the reason for the incident, the actions taken to bring the catalytic incinerator or thermal oxidizer back into service with the appropriate temperatures, the time the catalytic incinerator or thermal oxidizer was back in service with the appropriate temperatures, and the estimated duration of the incident. If required in accordance with Rule 62-210.700(1), F.A.C., an excess emissions malfunction report shall be submitted in accordance with Rule 62-210.700(6) and General Conditions Item 8. [Rules 62-4.070(3) & 62-210.700, F.A.C.] Comments: The facility did not have an episode of excess emissions. At no time during the compliance year did the catalytic incinerator EU 002 inlet temperature fall below 550 deg F or did the outlet exceed 1,100 deg F. 34. Annual Operating Report – The permittee shall submit each calendar year on or before March 1, a completed DEP Form 62-210.900(5), "Annual Operating Report for Air Pollutant Emitting Facility" (AOR) for the preceding calendar year. The report may be submitted electronically in accordance with the instructions received with the AOR package sent by the Department, or a hardcopy may be sent to the Air Quality Division of the Pinellas County Department of Environmental Management (PCDEM), with a copy to the Air Compliance section of the SWD District of the Department. Total HAP emissions and emissions of individual HAPs that equal or exceed 0.1 tons/year shall be reported in the AOR. [Rule 62-210.370(3), F.A.C.; Pinellas County Code Subsection 58-90(3) & (9)] Comments: EAOR received: 3/31/11 and reviewed on 4/7/11 35. Reasonable Assurance - In order to provide reasonable assurance that the original assumptions about the air management inside this facility do not change, all fans, blowers, dust collection equipment, air conditioning, the catalytic incinerators and the thermal oxidizer in the facility shall be operating under normal operating conditions. Changes to the air management systems of this facility (including air conditioning and other air handling equipment) may render the VOC capture efficiencies ineffective. Notification shall be sent to the Air Quality Division of the PCDEM and the Air Compliance section of the SWD District of the Department under the following circumstances: - The permittee shall submit notification when a change is contemplated. No changes shall be made without the approval of the Air Quality Division of the PCDEM and the Air Compliance section of the SWD District of the Department. - The permittee shall submit notification when any portion of an existing air management system is damaged or becomes ineffective in any manner. This notification shall include any potential impact on VOC emissions. If the problem means the facility may not meet the capture efficiency assumptions, the notice shall be immediate, as in Specific Condition No. 8, otherwise within 7 days. [Rule 62-4.070(3), F.A.C.; 1030218-011-AC] Comments: There were no changes or damages to the catalytic incinerator or its associated equipment during the compliance year 36. Process Changes - The permittee shall report any proposed process changes, including, but not limited to, (a) any proposed significant change of VOC/OS species, or (b) any proposed increase in the production rate, to the Air Programs of the Southwest District Office of the Department and the PCDEM for proper processing. Any physical change in, change in the method of operation of, or addition to a stationary source or facility, which increases the actual emissions of any air pollutant may be considered a "Modification". [Rule 62-210.300, F.A.C.] Comments: There were no process changes reported during the compliance year. 37. FESOP Permit Renewal Application. A completed application for an air operating permit renewal shall be submitted to the Air Program of the Southwest District Office of the Department, with a copy to the Air Quality Division of the PCDEM, no later than 60 days prior to the expiration date of permit 1030218-012-AF. [Rules 62-4.030, 62-4.050, 62-4.070(3), 62-4.090, 62-210.300(2), 62-210.900, & 62-297.310(7)(a)3., F.A.C.] Comments: - The FESOP expired on 1/3/12. - Currently, the facility has applied for an operational renewal effective 10/28/11 for the 3 EU’s (the required Method 25A test was performed on 9/7/11 EU 002 and 9/2/11 EU 005). The AC permit is now pending. (ARMS screen; see below) |
|  |  |  | #### Valid Permit [Rule 62-210.300] |
|  |  |  | #### Changes to Facility/emission unit [Rule 62-210.300] Does the emission unit description above match what the facility is operating (Number of emission units or points, model number and serial number, etc.) Yes No Comments: |

|  |  | C. Other: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Pollution Prevention Activities - P2 Handouts Provided: P2 Brochure; P2 Manual; P2 Checklist - Have any emissions reductions occurred Yes / No less production Chemical Substitution; Equipment Changes; Process Changes Chemical/Material Reuse; On-site Recycling; Other: Comments: no pollution prevention activities during the compliance year 2011 - 0.24 tons/yr is the highest VOC emission total (May, 11’). The highest HAP emission = 0.01 tons/yr (May, 11’) 2010 - Highest VOC emissions =1.61 tons/yr; Highest HAP emissions = 0.02 fugitive |  |  |  |  |  |  |  |
| Closing Conference: |  |  |  |  |  |  |  |
| Inspector(s): Jeffrey Morris, Pinellas County, Air Quality Division |  |  |  |  |  |  |  |
| Signature(s): Date: |  |  |  |  |  |  | Date: |
|  | # ACCESS? |  |  | # ARMS? |  |  |  |

| AIRS ID 1030218 | OWNER M C Graphics, Inc. |  | FACILITY NAME Modern Graphic Arts, A Sandy Alexander Company |
| --- | --- | --- | --- |
| TITLE V |  | SYNTHETIC MINOR | DATE OF THIS FCE January 4, 2012 |
| TITLE V MEGA-SITE* |  | OTHER | DATE OF LAST FCE February 12, 2010 |

*Facility with a large number of complex emissions units.  It is more reasonable to evaluate a Title V Mega-Site once every 3 years instead of once every 2 years.

Review of All Required Reports

| PERIODIC REPORTS |  | COMMENTS |
| --- | --- | --- |
| DONE N/A | Annual operating report |  |
| DONE N/A | Statement of compliance |  |
| DONE N/A | Annual |  |
| DONE N/A | Semi-annual |  |
| DONE N/A | Quarterly |  |
| DONE N/A | Other : |  |
| DONE N/A | Other : |  |

| CONTINUOUS EMISSION MONITOR REPORTS |  | COMMENTS |
| --- | --- | --- |
| DONE N/A | Quarterly excess emissions |  |
| DONE N/A | Semi-annual |  |
| DONE N/A | RATA |  |
| DONE N/A | CGA |  |
| DONE N/A | Other : |  |
| DONE N/A | Other : |  |

Assessment of Control Device and Process Operating Conditions

| OFF-SITE ASSESSMENT |  | (Describe the off-site assessment in comments) |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ON-SITE ASSESSMENT |  | (Document the on-site inspection below) |  |  |  |
| DATE OF INSPECTION | DATE OF INSPECTION REPORT |  | My office maintains the inspection report... |  |  |
|  |  |  | ...in the ARMS database through EASIIR. | ...with the paper or electronic compliance files | ...in another location (specify). |
| 1/4/12 | 1/10/12 |  |  |  |  |
| COMMENTS |  |  |  |  |  |

Review of Tests and Records

| TESTS, OBSERVATIONS AND RECORDS |  | COMMENTS |
| --- | --- | --- |
| DONE N/A | Visible emission observation(s) | Performed during Method 25A test on 9/7/11 |
| DONE N/A | Review of facility records and logs | Copies of material usage and EU 002 HAP and VOC emis. |
| DONE N/A | Assessment of process parameters (feed rates, process rates, raw material compositions, etc.) | Copy of material usage |
| DONE N/A | Assessment of control equipment performance parameters (water flow rates, pressure drops, temperatures, ESP power levels, etc.) | Catalytic Incinerator temperature checked |
| DONE N/A | Stack test(s) | Method 25A test perform as a part of FESOP renewal |
| DONE N/A | Other : |  |
| DONE N/A | Other : |  |

Compliance Monitoring (CM) Information

| CM ELEMENT | My office maintains this information... |  |  |  |
| --- | --- | --- | --- | --- |
|  | ...electronically, in the ARMS database. | ...in the permit. | ...in the inspection report. | ...in another location (specify). |
| Facility information |  |  |  |  |
| Applicable requirements |  |  |  |  |
| Inventory of emission units |  |  |  |  |
| Enforcement history |  |  |  |  |
| Compliance activities |  |  |  |  |
| Findings and recommendations |  |  |  |  |
| COMMENTS |  |  |  |  |

Other Comments

Prepared by:	   Jeffrey Morris		Date:

Reviewed by:			Date:

1.  Complete the form as much as possible during for inspection; 2.  The remainder of checklist, for activities in the FY, are filled in no later than 9/15;  3.  Between 9/15 and 9/30, complete the FCE by closing out the FCS project in ARMS.  The INSP > FCS record only accepts a completion date and a comment; it does not accept information on the lower portion of the form.  If an FCS project already exists for the fiscal year in question, enter a completion date and comment in the existing record, do not create a duplicate FCS record.