| 1030480 88667 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FACILITY: Duckworth Steel Boats, Inc. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | PERMIT ID: 742 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | DISTRICT: Southwest |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ADDRESS: 1051 Island Avenue |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CONTACT PHONE: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Tarpon Springs, FL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 727-934-2550 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ARMS NO: 1030480 001 |  |  |  |  |  |  |  | PERMIT NO: |  |  |  |  |  |  |  |  |  |  |  | Expiration Date: 3/15/17 Renewal Date: 1/14/17 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | 1030480-006-AO |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Test Due Date: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EMISSION UNIT DESCRIPTION : Ship Building and Ship Repair Painting operations. Emissions controlled by work practices and tarps. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| INSPECTION DATE: |  |  |  |  |  |  | ARMS INSPECTION TYPE: |  |  |  |  |  |  |  |  |  |  |  |  |  |  | COMPLIANCE STATUS: |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | June 26, 2014 |  |  |  |  |  |  |  |  | INS1 |  |  | INS2 |  |  | INS3 |  |  | FUI |  |  |  |  |  | IN |  |  | MNC |  |  |  |  |  | SNC |  |
| INSPECTION TYPE: |  |  |  |  | Initial |  |  |  |  |  | Re-inspection |  |  |  |  |  | Complaint |  |  |  |  |  |  | Drive-by |  |  |  |  |  |  | Quarterly |  |  |  |  |
|  |  |  | A. General Review: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1. |  | Permit File Review |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  | No |
|  |  | Comments: yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2. |  | Introduction and Entry |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  | No |
|  |  | Comments: I met with the facility authorized representative/ facility contact, and owner |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3. |  | Is the Responsible Official/Authorized Representative still: Joel Duckworth? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  | No |
|  |  | Comments: same no change The Responsible Official/Authorized Representative’s e-mail is: joeduckworth@verizon.net |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4. |  | Is the facility contact still: Joel Duckworth? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  | No |
|  |  | Comments: same no change The facility contact’s e-mail is: joeduckworth@verizon.net |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| IN | MNC | SNC |  |  | B. Specific Conditions: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|             |  |  |  |  | Inspection Note: This permit became effective on 3/14/12 General Conditions [62-4.160, F.A.C.] 8. If, for any reason, the permittee does not comply with or will be unable to comply with any condition or limitation specified in this permit, the permittee shall immediately provide the Department with the following information: a. A description of and cause of noncompliance; and b. The period of noncompliance, including dates and times; or, if not corrected, the anticipated time the noncompliance is expected to continue, and steps being taken to reduce, eliminate, and prevent recurrence of the noncompliance. The permittee shall be responsible for any and all damages which may result and may be subject to enforcement action by the Department for penalties or for revocation of this permit. Comments: There have been no non compliance issues reported and none were observed during inspection of the facility and records review. 12. This permit or a copy thereof shall be kept at the work site of the permitted activity. Comments: A copy of the permit was was not kept at the worksite, in Mr. Duckworth’s office Common Conditions [Rule 62-296.320(4)(c), F.A.C.] Unless otherwise specified in the permit, the following conditions apply to all emissions units and activities at the facility. Emissions and Controls 2. Circumvention: The permittee shall not circumvent the air pollution control equipment or allow the emission of air pollutants without this equipment operating properly. [Rule 62-210.650, F.A.C.] Comments: There is no evidence of circumventing the air pollution control equipment, which is a filter system used only during the pneumatic loading of the sand storage silo, and the facility utilizes tarps when sandblasting the ships or pieces are blasted in a partial enclosure on the west side of the yard. It remains unchanged at this time. 6. VOC or OS Emissions: No person shall store, pump, handle, process, load, unload or use in any process or installation, volatile organic compounds (VOC) or organic solvents (OS) without applying known and existing vapor emission control devices or systems deemed necessary and ordered by the Department. [Rule 62-296.320(1), F.A.C.] Comments: The facility is not using, any VOC/OS products in the shipping yard at this time. The paints and thinning products are stored in paint shed in a storage shed on the north east side of the property in closed containers. There is a solvent recycling machine used for spent solvent products, and maintenance by Safety Kleen. 7. Objectionable Odor Prohibited: No person shall cause, suffer, allow or permit the discharge of air pollutants, which cause or contribute to an objectionable odor. An “objectionable odor” means any odor present in the outdoor atmosphere which by itself or in combination with other odors, is or may be harmful or injurious to human health or welfare, which unreasonably interferes with the comfortable use and enjoyment of life or property, or which creates a nuisance. [Rules 62-296.320(2) and 62-210.200(Definitions), F.A.C.] Comments: An upwind/downwind survey of the facility was conducted. The observed parameters were: Downwind odor level detected- ; Wind direction - WNW Upwind odor level detected-none There were no odors detected off the property at the time of inspection. 9. Unconfined Particulate Emissions: a. No person shall cause, let, permit, suffer or allow the emissions of unconfined particulate matter from any activity, including vehicular movement; transportation of materials; construction, alteration, demolition or wrecking; or industrially related activities such as loading, unloading, storing or handling; without taking reasonable precautions to prevent such emissions. b. Any permit issued to a facility with emissions of unconfined particulate matter shall specify the reasonable precautions to be taken by that facility to control the emissions of unconfined particulate matter. c. Reasonable precautions include the following: (1) Paving and maintenance of roads, parking areas and yards. (2) Application of water or chemicals to control emissions from such activities as demolition of buildings, grading roads, construction, and land clearing. (3) Application of asphalt, water, oil, chemicals or other dust suppressants to unpaved roads, yards, open stock piles and similar activities. (4) Removal of particulate matter from roads and other paved areas under the control of the owner or operator of the facility to prevent reentrainment, and from buildings or work areas to prevent particulate from becoming airborne. (5) Landscaping or planting of vegetation. (6) Use of hoods, fans, filters, and similar equipment to contain, capture and/or vent particulate matter. (7) Confining abrasive blasting where possible. (8) Enclosure or covering of conveyor systems. Comments: The facility yard does not have paved areas where ships are dry docked is all sand. There is no application of water on yard as there traffic is mainly foot traffic of employees and crane operations. There is no addition of oil or suppressants on the yard because sand yard which is in close proximity to open body of Anclote water body. There is no entrainment of sand from rear ship yard to the roadways. The landscaping is only the natural growth that surrounds the property. To confine particulate matter the facility uses a three sided metal enclosure during the sandblasting of parts. There was no sandblasting activities at the time of inspection (See photo) Permit Administrative Requirements 9 Annual Operating Report: On or before April 1 of each year, the permittee shall submit a completed DEP Form 62-210.900(5), "Annual Operating Report for Air Pollutant Emitting Facility" (AOR) for the preceding calendar year. The report may be submitted electronically in accordance with the instructions received with the AOR package sent by the Department, or a hardcopy may be sent to the Compliance Authority. [Rule 62-210.370(3), F.A.C.] Comments: The annual operating report for calendar year 201_was submitted on 3/27/14. 10 Operation Permit Renewal Application: A completed application for renewal of the operation permit shall be submitted to the Permitting Authority with a copy to PCDEM (Compliance Authority) no later than 60 days prior to the expiration date of the operation permit. [Rules 62-4.030, 62-4.050, 62-4.070(3), 62-4.090, 62-210.300(2), and 62-210.900, F.A.C.] Comments: The permit expires on 3/15/2017. An application is required to be submitted no later than 1/14/2017. The renewal is not due at this time. A.1. Reasonably Available Control Technology for Volatile Organic Compounds: In order to not subject the facility to the requirements of VOC RACT [Rule 62-296.513, F.A.C.], the permittee shall not paint the following: - The interior of any marine vessel (repair, new, or any other operation); and - Newly fabricated portions of marine vessels. This includes any portion of a ship or prefabricated portion of a ship. The facility may paint the exterior of an entire marine vessel. Comments: The facility uses pre-coated steel panels for the construction of ships. The paints used are primer, and colored enamel ship coatings for the exterior of the ships. The painting typically is, per Mr. Duckworth rolled on and brush work on the smaller parts for the vessels in yard. There was no spray painting activities at this time. (See Photo) A.3. HAP and VOC Emissions: Emissions from the painting operation shall not exceed 4.93 tons per any consecutive 12-month period for VOCs. [Construction Permit 1030480-004-AC] Permitting Note: All HAP emissions from this facility are expected to be VOCs; therefore, the VOC emissions limit will limit the HAP emissions below the major source thresholds. Comments: The records are maintained by Joel Duckworth. The 12 month totals for 2013 and 2014 were below the 4.93 tons for the 12 month consecutive totals. The highest monthly total was 0.4533 tons for April 2014. The April 2014, 12 month total was 2.423 tons. The 2014, 12 month total as end of June was 2.393 ton. There were 2 months no paint usage January and February 2014. (See attached 12 month consecutive total logs). The facility is complying with the permit limitation. A.4. Monthly Records: In order to document compliance with VOC emissions limitation in Specific Condition No. 12, the permittee shall calculate the painting operation’s total monthly VOC emissions and consecutive 12-month total VOC emissions, in tons. A log shall be maintained to document the monthly and consecutive 12-month total emissions. The log shall be completed by the tenth day of the following month. The log shall include, at a minimum, the following information: - Facility ID number and emission unit ID and description (e.g., 1030480, EU 001-Painting operation); - Month and Year; - Monthly, the amount of each coating (e.g. by identification number, name, etc.) used, in gallons and the corresponding VOC lbs/gallon; - Monthly, the amount of each clean-up solvent (e.g. by identification number, name, etc.) used, in gallons and the corresponding VOC lbs/gallon; - The monthly VOC emissions, in tons; and - The consecutive 12-month total VOC emissions, in tons. These records shall be maintained at the facility for a minimum of the most recent three (3) year period and made available to the Department and the PCAQD upon request. [Rule 62-4.070(3), F.A.C.] Comments: The facility is maintaining monthly log sheets for the paint coatings and solvents used on site as required in subsections (a-f). (See attached painting operations log) A.5. Supporting Documentation: Supporting documentation (Material Safety Data Sheets, "As Supplied" sheets, purchase orders, inventory records, production records, etc.) shall be kept for each VOC-containing material used in order to provide sufficient information to document how VOC emissions were calculated. Documentation for VOC/organic solvents reclaimed, recycled or disposed. All of the above records shall be retained for at least three (3) years, and kept on site and made available to the Department and the PCDEM upon request. [Rules 62-296.320(1)(a) and 62-4.070(3), F.A.C.] (Permitting Note: The permittee will use a mass balance method to determine net usage of VOC-containing materials and organic solvents.) Comments: Joel Duckworth maintains copies of the MSD sheets for the products in use at the facility. The current copies are obtained from his vendor on his personal computer. He also maintains the hard copies in file. |  |  |  |  |  |
|  |  |  |  |  | #### Valid Permit [Rule 62-210.300] |  |  |  |  |  |
|  |  |  |  |  | #### Changes to Facility/emission unit [Rule 62-210.300] Does the emission unit description above match what the facility is operating (Number of emission units or points, model number and serial number, etc.) Yes No Comments: No changes observed |  |  |  |  |  |
|  |  |  | C. Other: |  |  |  |  |  |  |  |
| Pollution Prevention Activities - P2 Handouts Provided: P2 Brochure; P2 Manual; P2 Checklist - Have any emissions reductions occurred Yes / No no reductions noted Chemical Substitution; Equipment Changes; Process Changes Chemical/Material Reuse; On-site Recycling; Other: Comments: solvents, used oil and scrap metals salvaged |  |  |  |  |  |  |  |  |  |  |
| Closing Conference: I informed Joel Duckworth the facility appears to be complying with the permit. |  |  |  |  |  |  |  |  |  |  |
| Inspector(s): Shea Jackson, Pinellas County, Air Quality Division |  |  |  |  |  |  |  |  |  |  |
| Signature(s): Date: |  |  |  |  |  |  |  |  | Date: July 1, 2014 |  |
| # ACCESS? |  |  |  |  |  | # ARMS? |  |  |  |  |

Duckworth Steel Boats, Inc.

1051 Island Avenue, Tarpon Springs

Project Id:	88667	Permit No: 1030480-006-AO	Arms Number: 0480 001

Inspector:	Shea Jackson	Inspection Date / Time: 6/26/14  / _______

Source (EU):	Ship Building and Ship Repair Painting operations.  Emissions controlled by work practices and tarps.

# Description:	[There was one scallop ship under construction at time of inspection.  There were no painting or sand blasting activities at this time.  There was mainly welding in process. ]