| 1030480 88668 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FACILITY: Duckworth Steel Boats, Inc. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | PERMIT ID: 2292 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | DISTRICT: Southwest |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ADDRESS: 1051 Island Avenue |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CONTACT PHONE: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Tarpon Springs, FL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 727-934-2550 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ARMS NO: 1030480 002 |  |  |  |  |  |  |  | PERMIT NO: |  |  |  |  |  |  |  |  |  |  |  | Expiration Date: 3/15/17 Renewal Date: 1/14/17 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | 1030480-006-AO |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Test Due Date: 12/27/00 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EMISSION UNIT DESCRIPTION : Abrasive blasting operations: Consists of blasting pots and sand storage silo. Sand blasting controlled by work practices and tarps. Silo controlled by 2’ by 3’ box with a generic polypropylene filter media and a water hose to generate a water spray |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| INSPECTION DATE: |  |  |  |  |  |  | ARMS INSPECTION TYPE: |  |  |  |  |  |  |  |  |  |  |  |  |  |  | COMPLIANCE STATUS: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | June 26, 2014 |  |  |  |  |  |  |  |  | INS1 |  |  | INS2 |  |  | INS3 |  |  | FUI |  |  |  |  |  | IN |  |  | MNC |  |  |  |  |  | SNC |  |  |
| INSPECTION TYPE: |  |  |  |  | Initial |  |  |  |  |  | Re-inspection |  |  |  |  |  | Complaint |  |  |  |  |  |  | Drive-by |  |  |  |  |  |  | Quarterly |  |  |  |  |  |
|  |  |  | A. General Review: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1. |  | Permit File Review |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  | No |  |
|  |  | Comments: yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2. |  | Introduction and Entry |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  | No |  |
|  |  | Comments: I met with the facility authorized representative/ facility contact, and owner |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3. |  | Is the Responsible Official/Authorized Representative still: Joel Duckworth? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  | No |  |
|  |  | Comments: same no change The Responsible Official/Authorized Representative’s e-mail is: joeduckworth@verizon.net |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4. |  | Is the facility contact still: Joel Duckworth? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  | No |  |
|  |  | Comments: same no change The facility contact’s e-mail is: joeduckworth@verizon.net |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| IN | MNC | SNC |  |  | B. Specific Conditions: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|           |  |  |  |  | Inspection Note: This permit became effective on 3/14/12 B.1. Sand Usage Limit: The sandblasting operation is limited to a maximum sand usage of 97.6 tons per any consecutive 12-month period. [Rule 62-210.200 (PET), F.A.C.; Construction Permit 1030480-004-AC] Comments: The facility record for the sand storage silo from June 2013 – May 2014 showed there to be three loads received. and pneumatic loading occurred. This facility typically loads the silo with 25 tons one to times per calendar year. There were no exceedances of this permit limitation. The 12 month consecutive total was 71.33 tons and below the limitation. B.3. Particulate Matter (PM) Emissions Control: A particulate filter shall control emissions from Sand Storage Silo during loading. [Construction Permit 1030480-004-AC] Comments: The facility sand storage silo has a filtering box system for capturing the emissions during the pneumatic loading process. The control system has a PVC tube which conveys any sand emitted from the top of the silo baghouse filter compartment down into a 55 gallon drum. The drum is sprayed inside with a water hose hooked up for the suppression of particulate emissions. Another PVC pipe connects to the drum and conveys emissions up through an additional filter compartment to any escaping sand. This was approved by permit to prevent emissions during the pneumatic loading process. B.4. Visible Emissions: Visible Emissions (VE) from loading the Sand Storage Silo shall not exceed five percent (5%) opacity. [Construction Permit 1030480-004-AC] Comments: The last test results were 0.58 % opacity performed on 1/16/2014. The test was reviewed and opacity compiled with limitation below 5% . B.5. Compliance Tests : Test the Sand Storage Silo the first time it is loaded during every federal fiscal year (October 1st to September 30th). [Rules 62-4.070(3) and 62-297.310, F.A.C.] Comments: The sand storage silo is typically pneumatically loaded on infrequent bases, during the year. The silo was loaded only three times in past 12 months. The facility has had difficulty scheduling the VE performance testing date to be within the current fiscal year. The 1/16/14 test was for this fiscal year period of October 2013 – September 30, 2014. (See records) B.6. Test Requirements: Tests shall be conducted in accordance with the applicable requirements specified in Appendix D (Common Testing Requirements) of this permit. [Rule 62-297.310, F.A.C.] Comments: The Facility has been submitting annual VE test reports and the test report has been reviewed for This test met with the applicable requirements specified in Appendix D. IE. The test was performed within 90 – 100% of the maximum process rate of 25 TPH. The test length was 30 minutes, and was performed annually. The test as received by AQ division was within 45 days of test date on 2/25/14 for review 44 days after test performance on 1/16/14. The source complied with this condition. B.7. Test Method(s): Required tests shall be performed in accordance with the following reference method(s). Method(s); Description of Method and Comments. 9; Visual Determination of the Opacity of Emissions from Stationary Sources. The above method(s) are described in Appendix A of 40 CFR 60 and are adopted by reference in Rule 62-204.800, F.A.C. No other method(s) may be used unless prior written approval is received from the Department. [Rules 62-204.800 and 62-297.100, F.A.C.; and Appendix A of 40 CFR 60] Comments: The Environmental Engineering Consultant, Marvin Scott performed the test on 1/16/14, for the facility as required by the method , and submit additional process information per Appendix A The test met the minimum requirements for identifying the emission unit and the specification for the operations and demonstrated the test results to be valid. The facility has demonstrated compliance of the opacity limitations. B.8. Test Notification: The permittee shall notify the Compliance Authority in writing at least 15 days prior to any required tests. The notification must include the following information: the date, time, and location of each test; the name and telephone number of the facility’s contact person who will be responsible for coordinating the test; and the name, company, and the telephone number of the person conducting the test. {Permitting Note: The notification should also include the relevant emission unit ID No(s)., test method(s) to be used, and pollutants to be tested.} [Rule 62-297.310(7)(a)9., F.A.C.] Comments: The facilities consultant Environmental Engineer Marvin Scott notified the AQ dept 1/9/14 7 days prior to the requested 1/16/14 test performance date. The 15 day advance notice requirement was relinquished by PCAQ due to the difficulty the facility has when trying to determine the sand storage silo empty level, and the facilities infrequent annual loading of the silo, which could result in going past test requirement for testing every 12 months. B.9. Test Reports: The permittee shall prepare and submit reports for all required tests in accordance with the requirements specified in Appendix D (Common Testing Requirements) of this permit. The following information shall be included in the test report: - The loading rate of the silo (tons/hour); and - A copy of the monthly log required in Specific Condition No. B.10. for the month when the VE test is conducted. Failure to submit any of the information listed above or operating at conditions during testing which are not representative of normal operating conditions may invalidate the tests. [Rules 62-4.070(3) and 62-297.310(8), F.A.C.] Comments: Environmental Engineering Consultant Marvin Scott submitted the required information. The information met the minimum requirements for identifying the emission unit, and the specification for the operations described; demonstrated the test was performed as required and the test results were valid. The additional documents needed for valid test performance and review were received for the Method 9 test performed on 1/16/14, the report did contain: (a) the statement of the loading rate. (b) A copy of the monthly sand log was submitted with the test report. These documents are with the test report review in the permit compliance file. B.10. Monthly Log: In order to document compliance with Specific Condition No. B.1. the permittee shall maintain a monthly log of sand received. The monthly log shall be completed by the tenth day of the following month. The log shall include, at a minimum, the following information: - Facility ID number and emission unit ID and description (e.g., 1030480, EU 002-Sandblasting operation); - Month and Year; - On a monthly basis, the tons of sand received; and - The consecutive 12-month total of sand received, in tons. These records shall be maintained at the facility for a minimum of the most recent three (3) year period and made available to the Department and the PCAQD upon request. [Rule 62-4.070(3), F.A.C.] Comments: Joel Duckworth maintains the records for the sand storage silo usage. The records meet the above required detail information (a-d). The facility log consists of zero entries for months silo not loaded. There was 1 loads received in 2013. During 2014 to the date of this inspection, two loadings have occurred. (See log record copies attached) B.11. Operation and Maintenance (O & M) Plan for Particulate Matter Control: For the filter at the Sand Storage Silo, the permittee shall maintain and implement an O & M Plan, which was submitted with the application dated February 21, 2012 (see Appendix E). The control device shall be operated and maintained in accordance with the plan and as revised with the prior approval of the PCAQD. Such parameters and records shall include, at a minimum, that shown below. a. Performance Parameters: - Operating parameters of the pollution control device; - Schedule for the routine maintenance of the pollution control device as specified by the manufacturer; - Schedule for routine observations of the pollution control device sufficient to ensure proper operation; and - A list of the type and quantity of the required spare parts for the pollution control device which are stored on the premises of the permittee. The O&M Plan shall be retained onsite at all times. b. Operation and Maintenance Log: The O&M Plan Log shall include, at a minimum, the following information: - When maintenance and observations were performed (Date and Time); - What maintenance and observations were performed; - Who performed the maintenance and observations (Name of Person(s)); and - Acceptable parameter ranges for each operational check. These records shall be maintained at the facility for a minimum of the most recent three (3) year period and made available to the Department and the PCAQD upon request. [Rule 62-4.070(3), F.A.C.] Comments: Mr. Joel Duckworth performs maintenance and records for the silo. The maintenance is performed prior to pneumatic loadings. Due to infrequent pneumatic loading, this is acceptable practice. (See maintenance record attached). |
|  |  |  |  |  | #### Valid Permit [Rule 62-210.300] |  |  |  |  |  |
|  |  |  |  |  | #### Changes to Facility/emission unit [Rule 62-210.300] Does the emission unit description above match what the facility is operating (Number of emission units or points, model number and serial number, etc.) Yes No Comments: no changes |  |  |  |  |  |
|  |  |  | C. Other: |  |  |  |  |  |  |  |
| Pollution Prevention Activities - P2 Handouts Provided: P2 Brochure; P2 Manual; P2 Checklist - Have any emissions reductions occurred Yes / No no reductions noted Chemical Substitution; Equipment Changes; Process Changes Chemical/Material Reuse; On-site Recycling; Other: Comments: solvents, used oil and scrap metals salvaged |  |  |  |  |  |  |  |  |  |  |
| Closing Conference: I informed Joel Duckworth the facility appears to be complying with the permit. |  |  |  |  |  |  |  |  |  |  |
| Inspector(s): Shea Jackson, Pinellas County, Air Quality Division |  |  |  |  |  |  |  |  |  |  |
| Signature(s): Date: |  |  |  |  |  |  |  |  | Date: July 1, 2014 |  |
| # ACCESS? |  |  |  |  |  | # ARMS? |  |  |  |  |

Duckworth Steel Boats, Inc.

1051 Island Avenue, Tarpon Springs

Project Id:	84608	Permit No: 1030480-006-AO	Arms Number: 0480 002

Inspector:	Shea Jackson	Inspection Date / Time: 6/26/2014 / _______

Source (EU):	Abrasive blasting operations: Consists of blasting pots and sand storage silo.  Sand blasting controlled by work practices and tarps.  Silo controlled by  2’ by 3’ box with a generic polypropylene filter media and a water hose to generate a water spray

# Description:	[There was no blasting activity at this time.  The sand storage silo to the west of the ship contains the sand for blasting, no pneumatic loading at this time.]