| 1030482 70975 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FACILITY: Gause Built Boats, Inc. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | PERMIT ID: 1739 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | DISTRICT: Southwest |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ADDRESS: 720 Wesley Avenue, Unit 2, 3, 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CONTACT PHONE: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Tarpon Springs, FL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 727-278-4999 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ARMS NO: 1030482 001 |  |  |  |  |  |  |  | PERMIT NO: |  |  |  |  |  |  |  |  |  |  |  | Expiration Date: 9/19/2012 Renewal Date: 7/21/2012 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | 1030482-005-AF |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Test Due Date: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EMISSION UNIT DESCRIPTION : Polyester Resin Boat Manufacturer at two locations (720 Wesley Avenue, Units 2, 3, and 4 & 1450 Rainville Road) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| INSPECTION DATE: |  |  |  |  |  |  | ARMS INSPECTION TYPE: |  |  |  |  |  |  |  |  |  |  |  |  |  |  | COMPLIANCE STATUS: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | April 29, 2010 |  |  |  |  |  |  |  |  | INS1 |  | ü | INS2 |  |  | INS3 |  |  | FUI |  |  |  | ü |  | IN |  |  | MNC |  |  |  |  |  | SNC |  |  |
| INSPECTION TYPE: |  |  |  | ü | Initial |  |  |  |  |  | Re-inspection |  |  |  |  |  | Complaint |  |  |  |  |  |  | Drive-by |  |  |  |  |  |  | Quarterly |  |  |  |  |  |
| ü |  |  | A. General Review: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1. |  | Permit File Review |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  | No |  |
|  |  | Comments: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2. |  | Introduction and Entry |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  | No |  |
|  |  | Comments: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3. |  | Is the Responsible Official/Authorized Representative still: Robert Gause? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  | No |  |
|  |  | Comments: attn: Robert Gause The Responsible Official/Authorized Representative’s e-mail is: gausebuiltboats.com |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4. |  | Is the facility contact still: Robert Gause? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  | No |  |
|  |  | Comments: attn: Robert Gause The facility contact’s e-mail is: gausebuiltboats.com |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| IN | MNC | SNC |  |  | B. Specific Conditions: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ü ü ü ü ü ü ü ü ü ü ü ü ü ü ü |  |  |  |  | This permit or a copy thereof shall be kept at the work site of the permitted activity. [General Condition 12.] Comments: reviewed on-site permit 4. Volatile Organic Compound (VOC) and Hazardous Air Pollutant (HAP) Emission Limitations - Facility-wide emissions of VOCs and HAPs, as defined in Rule 62-210.200, F.A.C., shall not exceed the following: a. Styrene emissions shall not exceed 9.4 tons per any 12 consecutive month period; and b. VOC emissions, including styrene, shall not exceed 24.0 tons per any 12 consecutive month period. [Rule 62-210.200, F.A.C. (“Potential to Emit”); construction permit 1030482-003-AC] Comments: The highest reported emissions were 0.10 tons Styrene/12-month period and 0.14 tons VOC/12-month period for the month of February, 10’. Reviewed records for the months of March, 09’ – January, 10’______ 5. General Pollutant Emission Limiting Standards. Objectionable Odor Prohibited - The permittee shall not cause, suffer, allow or permit the discharge of air pollutants, which cause or contribute to an objectionable odor. An "Objectionable Odor" is defined as any odor present in the outdoor atmosphere, which by itself or in combination with other odors, is or may be harmful or injurious to human health or welfare, which unreasonably interferes with the comfortable use and enjoyment of life or property, or which creates a nuisance. If the Department or the Pinellas County Department of Environmental Management (PCDEM) receive a valid odor complaint from either location, the Department reserves the right to require additional odor control measures/work practices be implemented. [Rules 62-210.200 and 62-296.320(2), F.A.C.; Pinellas County Code, Section 58-178] Comments: An upwind/downwind survey of the facility was conducted. The observed parameters were: Downwind odor level detected- None; Wind direction – S/10 mph Upwind odor level detected-None 6. General Standards: Volatile Organic Compound Emissions and/or Organic Solvent Emissions - The permittee shall allow no person to store, pump, handle, process, load, unload or use in any process or installation, volatile organic compounds (VOC) or organic solvents (OS) without applying known and existing vapor emission control devices or systems deemed necessary and ordered by the Department. The following procedures shall be utilized to minimize pollutant emissions: a. all resin activities, at the 720 Wesley Avenue location, shall be conducted inside the lamination building; b. the dust collector, located on the north wall of the main lamination area at the 720 Wesley Avenue location, shall be run for at least 2 hours after major resin operations have ceased; c. doors and windows at the 720 Wesley Avenue location shall be closed whenever feasible during production to ensure that materials which emit VOC's are captured and discharged through the collector; d. all equipment, pipes, hoses, lids, fittings, etc., shall be operated/maintained in such a manner as to minimize leaks, fugitive emissions, and spills of solvent materials. e. all solvent from solvent washings (equipment clean-up) shall be directed into containers that prevent evaporation in the atmosphere; f. tightly cover or close all VOC and/or solvent containing vessels, drums, totes, etc., when they are not in use; g. immediately confine and clean up VOC and/or solvent spills, and make sure wastes are placed in closed containers for reuse, recycling or proper disposal; and h. used cleanup solvents shall be captured and stored in closed containers and recycled or disposed of as required by the appropriate waste disposal regulations. [Rules 62-4.070(3) and 62-296.320(1), F.A.C.; construction permit 1030482-003-AC] Comments: Facility complies with conditions A-E. 7. General Particulate Emission Limiting Standards: General Visible Emissions Standard - Except for emissions units that are subject to a particulate matter or opacity limit set forth or established by rule and reflected by conditions in this permit, no person shall cause, let, permit, suffer or allow to be discharged into the atmosphere the emissions of air pollutants from any activity, the density of which is equal to or greater than that designated as Number 1 on the Ringelmann Chart (20 percent opacity). [Rule 62-296.320(4)(b)1., F.A.C.] Comments: No visible emissions observed during the inspection 8. General Standards: Unconfined Particulate Matter (PM) - All reasonable precautions shall be taken to prevent and control generation of unconfined emissions of particulate matter in accordance with the provisions in Rule 62-296.320, F.A.C. These provisions are applicable to any source, including but not limited to, vehicular movement, transportation of materials, construction, alteration, demolition or wrecking, or industrial related activities such as loading, unloading, storing and handling. Unconfined particulate emissions generated from activities such as cutting, sanding, trimming, shaping, and grinding shall be controlled by portable “shop-vac” vacuum collectors, frequent sweeping and cleaning of equipment, floors and other surfaces and other good housekeeping procedures. As an indicator that the above precautions to control unconfined particulate matter emission are adequate, visible emissions from the facility should not exceed 5% opacity. If this value is exceeded it will not be considered a violation in and of itself, but an indicator that additional controls and/or measures may be required. [Rules 62-4.070(3) and 62-296.320(4)(c), F.A.C.] Comments: No unconfined PM observed during the inspection 9. Operating Limitation: The percent (%) of available monomer (styrene) shall not exceed the following: Resin Layup - 50% Gelcoats ------ 45% [Rule 62-210.200, F.A.C. (“Potential to Emit”); construction permit 1030482-003-AC] Comments: The facility never exceeded 50% or 45% styrene content for resin or gel coat 10. Emission Calculation Requirements: Emissions of styrene, methyl methacrylate (MMA), and other volatile organic compounds (VOCs) shall be determined as follows until notified by the Department or the Pinellas County Department of Environmental Management. Note, styrene and MMA are each considered a VOC and a HAP. a. Styrene emissions shall be determined using the following equation: Emissions (tons)=Ms×EF2000 lb/ton where: Ms = amount of styrene containing material used (in tons) EF = emission factor (lb/ton) for styrene monomer content (from table below) The applicable styrene emission factor shall be obtained (interpolated/extrapolated, if applicable) from the following table, in conjunction with the percent of available non-vapor suppressed (NVS) styrene monomer in the resin/gelcoat: % Monomer; Resin** Hand (Non-Spray) Layup; Resin** Mechanical Atomized Spray Layup; Resin Mechanical Nonatomized* Spray Layup; Gel Coat** Mechanical Atomized Spray Layup; Gelcoat Mechanical Nonatomized* Spray Layup. 18; 39; 51; 39; 138; 67. 19; 41; 54; 41; 148; 70. 20; 43; 58; 43; 158; 79. 21; 45; 61; 45; 168; 88. 22; 48; 64; 47; 178; 97. 23; 50; 68; 49; 189; 106. 24; 52; 71; 51; 200; 115. 25; 55; 75; 54; 211; 124. 26; 57; 78; 56; 221; 133. 27; 59; 82; 58; 233; 142. 28; 62; 86; 60; 244; 151. 29; 64; 90; 62; 256; 160. 30; 67; 94; 64; 269; 169. 31; 69; 97; 66; 281; 178. 32; 71; 102; 68; 295; 187. 33; 74; 105; 71; 308; 196. 34; 77; 112; 74; 322; 205. 35; 80; 118; 77; 336; 214. 36; 83; 127; 80; 354; 223. 37; 86; 136; 83; 371; 232. 38; 90; 146; 86; 390; 241. 39; 92; 156; 89; 408; 250. 40; 95; 166; 93; 427; 259. 41; 98; 176; 96; 446; 268. 42; 100; 187; 99; 466; 278. 43; 112; 192; 102; 477; 287. 44; 117; 202; 105; 495; 296. 45; 122; 212; 108; 513; 305. 46; 127; 221; 111; 532; 314. 47; 133; 232; 115; 551; 323. 48; 138; 242; 118; 571; 332. 49; 143; 252; 121; 590; 341. 50; 148; 263; 124; 609; 350. * Nonatomized means any application technology in which the resin is not broken into droplets or an aerosol as it travels from the application equipment to the surface of the part. Nonatomized resin application technology includes, but is not limited to, flowcoaters, chopper flowcoaters, and pressure fed resin rollers. In addition, the device must be operated according to the manufacturer’s directions, including instructions to prevent the operation of the device at excessive spray pressures. ** The emission factors in these columns are based on a weighted average using the following assumptions: - Using the Unified Emission Factors - 20% by weight of the facility's usage of resins and gelcoats are for reinforced plastic composites production for items such as hatches and doors, but not for hulls and decks. - Using the National Marine Manufacturer Association's Factors – 80% by weight of the facility's usage of resins and gelcoats are for items such as decks and hulls. b. MMA emissions shall be determined by the following equations: Emissions (tons)=Mmma×Cmma×0.75 where: Mmma = amount of MMA containing material used (in tons) Cmma = MMA content (percent/100) c. Other VOC emissions shall be determined by the following equation for each material. These values shall be used in conjunction with the above styrene and MMA emissions to determine total VOC: Emissions (tons)=Mvoc×1.00×Cvoc where: Mvoc = amount of VOC containing material used (in tons) Cvoc = VOC content (percent/100) [Rule 62-210.200, F.A.C. (“Potential to Emit”)] Comments: The facility follows the requirements of Subpart VVVV and this condition. 12. VOC and HAP Recordkeeping - A monthly log based on the tracking of all styrene and/or VOC containing material used in association with manufacturing activities shall be kept for this facility to document compliance with the emission limitations of Specific Condition No. 4. At a minimum, the logs shall contain the following: a. facility name, facility ID No., emission unit ID No., and description (i.e., Gause Built Boats, Inc., 1030482, E.U. 001, fiberglass boat manufacturing); b. identity and quantity of each resin, coating, solvent, chemical, etc. used at the facility that contains styrene and/or VOC, per application method; c. appropriate emission factor(s), per application method(s), used to determine styrene and/or VOC emissions; d. total styrene emissions for the month (tons); e. total VOC (including styrene) emissions for the month (tons/month); f. total styrene emissions for the most recent 12 consecutive month period (tons/12 consecutive month period); and g. total VOC (including styrene) emissions for the most recent 12 consecutive month period (tons/12 consecutive month period). The logs may be based on the beginning and ending inventories, deliveries, shipments, etc. At the option of the permittee, “purchases” may be used instead of “usage” in determining VOC/HAP emissions, provided that no materials are used which are not purchased. Documentation of each chemical reclaimed will use a mass balance method to determine usage/emissions (amount used minus amount collected for disposal or recycle). Supporting documentation such as styrene/VOC content, styrene/VOC emission factors used, MSDS sheets, purchase orders, "As Supplied" data sheets, etc. shall be kept for each chemical and associated products, which includes sufficient information to determine emissions. The monthly logs shall be completed by the end of the following month. These records shall be maintained at the facility for at least 3 years in a form suitable for inspection and be made available to the Department and PCDEM upon request. [Rule 62-4.070(3), F.A.C.] Comments: The facility meets the conditions. A copy of the spreadsheet is attached to the report 13. Annual Operating Report - Each calendar year on or before March 1, the permittee shall submit a completed DEP Form 62-210.900(5), "Annual Operating Report for Air Pollutant Emitting Facility" (AOR) for the preceding calendar year. The report may be submitted electronically in accordance with the instructions received with the AOR package sent by the Department, or a hardcopy may be sent to the to the Air Compliance section of the Pinellas County Department of Environmental Management. [Rule 62-210.370(3), F.A.C.] Comments: 09’ AOR reviewed 4/5/10 and forwarded to oversight same day 14. Operation Permit Renewal Application - A completed application to renew this operation permit shall be submitted to the Air Program of the Southwest District Office of the Department no later than 60 days prior to the expiration date of this permit… [Rules 62-4.050, 62-4.070(3), 62-4.090, 62-210.300(2), and 62-210.900, F.A.C.] Comments: The permit application is due to be submitted on, or before 7/21/12 |
| ü |  |  |  |  | #### Valid Permit [Rule 62-210.300] |  |  |  |  |  |
|  |  |  |  |  | #### Changes to Facility/emission unit [Rule 62-210.300] Does the emission unit description above match what the facility is operating (Number of emission units or points, model number and serial number, etc.) Yes No Comments: |  |  |  |  |  |
| ü |  |  | C. Other: |  |  |  |  |  |  |  |
| Pollution Prevention Activities - P2 Handouts Provided: P2 Brochure; P2 Manual; P2 Checklist - Have any emissions reductions occurred Yes / No lower production Highest 12-mo total (VOC) = 2.72 tons/yr, 2.68 tons/yr HAP, 2.49 tons/yr Styrene (January, 08'). : The highest reported emissions were 0.67 tons Styrene/12-month period and 0.85 tons VOC/12-month period for the month of February, 10’. Reviewed records for the months of March, 09’ – January, 10’______ Chemical Substitution; Equipment Changes; Process Changes Chemical/Material Reuse; On-site Recycling; Other: Comments: |  |  |  |  |  |  |  |  |  |  |
| Closing Conference: |  |  |  |  |  |  |  |  |  |  |
| Inspector(s): Jeffrey Morris, Pinellas County, Air Quality Division |  |  |  |  |  |  |  |  |  |  |
| Signature(s): Date: |  |  |  |  |  |  |  |  | Date: |  |
| # ACCESS? |  |  |  | ü |  | # ARMS? | ü |  |  |  |

| AIRS ID 1030482 | OWNER Gause Built Boats, Inc. |  | FACILITY NAME |
| --- | --- | --- | --- |
| TITLE V |  | SYNTHETIC MINOR | DATE OF THIS FCE September 12, 2010 |
| TITLE V MEGA-SITE* |  | OTHER | DATE OF LAST FCE |

*Facility with a large number of complex emissions units.  It is more reasonable to evaluate a Title V Mega-Site once every 3 years instead of once every 2 years.

Review of All Required Reports

| PERIODIC REPORTS |  | COMMENTS |
| --- | --- | --- |
| DONE N/A | Annual operating report | 09’ AOR review 4/5/10 submitted to FDEP |
| DONE N/A | Statement of compliance |  |
| DONE N/A | Annual |  |
| DONE N/A | Semi-annual |  |
| DONE N/A | Quarterly |  |
| DONE N/A | Other : |  |
| DONE N/A | Other : |  |

| CONTINUOUS EMISSION MONITOR REPORTS |  | COMMENTS |
| --- | --- | --- |
| DONE N/A | Quarterly excess emissions |  |
| DONE N/A | Semi-annual |  |
| DONE N/A | RATA |  |
| DONE N/A | CGA |  |
| DONE N/A | Other : |  |
| DONE N/A | Other : |  |

Assessment of Control Device and Process Operating Conditions

| OFF-SITE ASSESSMENT |  | (Describe the off-site assessment in comments) |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ON-SITE ASSESSMENT |  | (Document the on-site inspection below) |  |  |  |
| DATE OF INSPECTION | DATE OF INSPECTION REPORT |  | My office maintains the inspection report... |  |  |
|  |  |  | ...in the ARMS database through EASIIR. | ...with the paper or electronic compliance files | ...in another location (specify). |
| 4/29/10 | 5/3/10 |  |  |  |  |
| COMMENTS |  |  |  |  |  |

Review of Tests and Records

| TESTS, OBSERVATIONS AND RECORDS |  | COMMENTS |
| --- | --- | --- |
| DONE N/A | Visible emission observation(s) |  |
| DONE N/A | Review of facility records and logs | Copy of spreadsheet attached w/inspection |
| DONE N/A | Assessment of process parameters (feed rates, process rates, raw material compositions, etc.) |  |
| DONE N/A | Assessment of control equipment performance parameters (water flow rates, pressure drops, temperatures, ESP power levels, etc.) |  |
| DONE N/A | Stack test(s) |  |
| DONE N/A | Other : |  |
| DONE N/A | Other : |  |

Compliance Monitoring (CM) Information

| CM ELEMENT | My office maintains this information... |  |  |  |
| --- | --- | --- | --- | --- |
|  | ...electronically, in the ARMS database. | ...in the permit. | ...in the inspection report. | ...in another location (specify). |
| Facility information |  |  |  |  |
| Applicable requirements |  |  |  |  |
| Inventory of emission units |  |  |  |  |
| Enforcement history |  |  |  |  |
| Compliance activities |  |  |  |  |
| Findings and recommendations |  |  |  |  |
| COMMENTS |  |  |  |  |

Other Comments

Prepared by:	   Jeffrey Morris		Date:

Reviewed by:			Date:

1.  Complete the form as much as possible during for inspection; 2.  The remainder of checklist, for activities in the FY, are filled in no later than 9/15;  3.  Between 9/15 and 9/30, complete the FCE by closing out the FCS project in ARMS.  The INSP > FCS record only accepts a completion date and a comment; it does not accept information on the lower portion of the form.  If an FCS project already exists for the fiscal year in question, enter a completion date and comment in the existing record, do not create a duplicate FCS record.